3GPP TSG-RAN WG4 Meeting #115                                        R4-2505586

Malta, May 19 - 23, 2025

Source: 	Huawei, HiSilicon

Title: 	On leftover issues for 6Rx

Agenda Item:	7.1.2.3.1

Document for:	Approval

Introduction

During RAN4#114bis, following issues have been discussed. FFS points can be found in [1].

- General aspects of 6Rx requirements Issue 2: FWA terminology Agreement: Further discuss possible clarifications and improvements on the use of FWA terminology in TS 38.101-1

In this contribution, we would like to share our views regarding this topic.

Discussion

- FWA terminology

In NR specification, following highlighted notes are used for 4Rx and 8Rx, respectively.

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R Operating band; ΔRIB,4R (dB). n5, n8, n13, n26, n28, n71, n85, n105; -2.71. n5, n8, n13, n28, n71, n20, n26; -2.42. n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70; -2.7. n48, n77, n78, n79, n104; -2.2. NOTE 1: When 4 Rx operation is supported by FWA form factor NOTE 2: When 4Rx operation is supported by handheld UE.. For UE(s) equipped with 8 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,8R in Table 7.3.2-2a for the applicable operating bands. Table 7.3.2-2a: Eight antenna port reference sensitivity allowance ΔRIB,8R Operating band; ΔRIB,8R (dB). n1, n3, n7; -4.5. n34, n39, n41; -4.3. n77, n78, n79; -4.0. NOTE 1: 8 Rx operation is targeted for FWA/CPE/Vehicle/Industrial devices form factor..

Unlike LTE spec, there is only FWA defined in TS 38.101-1 but no CPE. So in last meeting proponent proposed to align the wording for disambiguation. From our understanding, such clarification might be harmless due to following reasons:

- The intention for having those notes is to distinguish the REFSENS requirements between handheld and non-handheld UE and there seems no reason to have another requirement just for CPE.

- In FWA, traffic is maintained between the base station and the antenna of the Customer Premises Equipment (CPE, as defined in TS 36.101) device mounted either inside or outside the building. Hence, CPE is more like a specific component of the whole FWA system.

In conclusion, we have the following proposals.

Proposal 1: Suggest RAN4 to clarify whether “FWA form factor” and “FWA/CPE/Vehicle/Industrial devices form factor” have identical interpretation from RF requirement perspective.

Proposal 2: RAN4 considers following two alternatives for better specification clarity.

- Alt 1. Unify the wording as “FWA form factor” if no disagreement.

- Alt 2. No need to unify the spec language but reproduce the definition of CPE from LTE spec.

- CPE, Customer Premises Equipment.

Conclusion

In this contribution we discussed on the remaining issues of 6Rx. We have the following proposals:

Proposal 1: Suggest RAN4 to clarify whether “FWA form factor” and “FWA/CPE/Vehicle/Industrial devices form factor” have identical interpretation from RF requirement perspective.

Proposal 2: RAN4 considers following two alternatives for better specification clarity.

- Alt 1. Unify the wording as “FWA form factor” if no disagreement.

- Alt 2. No need to unify the spec language but reproduce the definition of CPE from LTE spec.

- CPE, Customer Premises Equipment.

- References

- R4-2505101, “WF on 6Rx RF requirements and SRS IL imbalance”, Intel, 3GPP TSG-RAN4 Meeting #114bis, Wuhan, China, April 7-11, 2025.