3GPP TSG-RAN WG4 Meeting #115                                           R4-2505589

Malta, May 19 - 23, 2025

Source: 	Huawei, HiSilicon

Title: 	On general aspects for SBFD

Agenda Item:	7.25.2

Document for:	Approval

Introduction

During RAN4#114bis, general aspects (including RAN4 aspects for SBFD system parameters) for SBFD have been discussed. Following discussion points are reproduced from [1].

##### Declaration and flexibility for SBFD operation Agreement: - For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains. ##### Restriction on the number of UL subband sizes supported Agreement: - For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared - Maximum 3 subband configurations allowed for FR1 WA BS - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS. - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains. ##### Restriction on the number of DL subband sizes supported Agreement: - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains. ##### Restriction on DL subband size Agreement: - If the SSB symbol is not overlapped with SBFD symbol, - There is no restriction on DL subband size. - If the SSB symbol is overlapped with SBFD symbol, - FFS the impact from RAN4 perspective. ##### Update for HPUE duty cycle evaluation Way forward: - RAN4 has discussed the two following points: - For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission; - For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission. - FFS RAN4 specification impact by taking into account the above understanding for SBFD-aware UE. ##### RF requirement impact for PUSCH with resource muting Way forward: - RAN4 has discussed this issue and will try to draft a reply LS to RAN1 based on existing results in next meeting. - Evaluation on remaining scope can be continued, and the issues can be raised if found in future.

In this contribution, we would like to share our views regarding this topic.

Discussion

The power limitation for FR1 MR SBFD-capable BS

In last meeting, whether and how to restrict the declaration and flexibility for SBFD operation for FR1 MR BS has been discussed. The proposed justification on the power level threshold is that it is closely related to whether specific implementation e.g. the usage of analog filter can be expected. Regarding the proposed value of such threshold, we take the 33dBm with a grain of salt. Reviewing the feasibility study captured in TR 38.858 clause 9.3, we can identify that the feasibility of FR1 MR for SBFD was mainly concluded with 38dBm Tx power assumption. Therefore we have the following proposal.

Proposal 1: Use 38dBm conducted output power as the power level threshold for FR1 MR SBFD capable BS.

Restriction on DL subband size

Setting a DL subband size that is not enough to transmit SSB may not be a wise move from network perspective. Thus considering the flexibility in terms of SSB time/frequency domain configuration in NR, the network can make the right decision even with the RAN1 clarification on that the SSB block is assumed to be within DL subband and we think there is no need to capture any further restriction in RAN4 spec.

Proposal 2: No RAN4 spec impact regarding DL subband size configuration is expected when the SSB symbol is overlapped with SBFD symbol.

Spec update for HPUE duty cycle

Proponent raised one question that whether the current specification “uplink symbols transmitted in a certain evaluation period” is clear enough for SBFD-aware UE. Note that for legacy UE the “uplink symbols” actually means the symbols with uplink transmission, which include e.g. flexible symbols configured with periodic uplink transmission. Thus it may be no harm to further update the spec as e.g. “uplink symbols and SBFD symbols transmitted in a certain evaluation period” in order to better accommodate SBFD-aware UE.

Observation 1: Might be no harm to further update the spec as e.g. “uplink symbols and SBFD symbols transmitted in a certain evaluation period” with the intention of better accommodating SBFD-aware UE max duty cycle evaluation.

On the potential RAN4 impact for UL resource muting

According to the WID [2], the potential transmit signal quality/MPR requirement impact due to UL resource muting will be checked. Details on this CLI handling solution are reproduced from WID as follows.

The LS in [3] also provides RAN1 study progress so far, we understand that the most important information to help RAN4 evaluation is a comb-2 pattern will be applied when UL resource muting is enabled on a symbol for PUSCH.

To our understanding, this technique would have no impact to the existing transmit quality/MPR requirement at least for lower modulation order. The reason is comb pattern is nothing new from Rel-15. For instance, DMRS multiplexing is specified but there is no dedicated minimum RF requirement for it so that doesn’t mean a UE can meet the aforementioned RF requirements may fail the very same requirements when transmission in comb pattern is scheduled. Further, if the total transmission power remain the same, which is already supported by 3dB power boosting as specified in the spec, there is no performance loss can be expected, either.

If MPR performance degradation can be identified for e.g. 256QAM, RAN4 shall discuss how to reflect it without minimum requirements impact.

Observation 2: For UL resource muting, there is no transmit quality or MPR requirement impact can be expected for PUSCH at least for lower modulation order as comb-2 pattern is already supported since LTE.

Proposal 3: If MPR performance degradation can be identified, scheduling restriction can be considered rather than introducing RAN4 minimum requirements impact.

Conclusion

In this contribution we discussed on the general aspects for SBFD. We have the following observations and proposals:

Observation 1: Might be no harm to further update the spec as e.g. “uplink symbols and SBFD symbols transmitted in a certain evaluation period” with the intention of better accommodating SBFD-aware UE max duty cycle evaluation.

Observation 2: For UL resource muting, there is no transmit quality or MPR requirement impact can be expected for PUSCH at least for lower modulation order as comb-2 pattern is already supported since LTE.

Proposal 1: Use 38dBm conducted output power as the power level threshold for FR1 MR SBFD capable BS.

Proposal 2: No RAN4 spec impact regarding DL subband size configuration is expected when the SSB symbol is overlapped with SBFD symbol.

Proposal 3: If MPR performance degradation can be identified, scheduling restriction can be considered rather than introducing RAN4 minimum requirements impact.

- References

- R4-2504751, “Way Forward for [114bis][306] NR_duplex_evo_General”, Samsung, 3GPP TSG-RAN4 Meeting #114bis, Wuhan, China, April 7-11, 2025.

- RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, 3GPP TSG RAN Meeting #104, Shanghai, China, June 17-20, 2024.

- R4-2500006, “LS on impact on transmit signal quality/MPR requirement”, RAN1, 3GPP TSG-RAN4 Meeting #113, Orlando, USA, November 18-22, 2024.