3GPP TSG-RAN WG4 Meeting #115	R4-2505594

Saint Julian’s, Malta, 19th May 2025 – 23rd May 2025

Agenda item:	7.16.3

Source:	MediaTek Inc.

Title:	Simulation results of Spatial Channel Modelling

Document for:	Discussion

# Introduction

In RAN4#114-bis, RAN4 agreed set of alignment simulations for Spatial Channel Modelling [1]. In this contribution, we provide our simulation results for Spatial Channel Modelling.

# Simulation results

## 2.1 Simulation parameters

In this section, we provide our simulation setup for Spatial Channel Modelling. In RAN4#113, RAN4 agreed that general simulation configuration can follow existing 38.101-4 8RX TDD requirement configuration specified in Chapter 5.2.4.2.1 [2]. Common parameters are listed in Table 1. We have assumed 0% TxEVM and LMMSE MIMO demodulation.

Table 1: Common test parameters of 4RX and 8RX tests (Table 5.2.4.2.1-2 in [2])

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 for rank <= 4 2 for rank > 4 |
| Codebook configuration | CodebookType |  | typeI-SinglePanel |
|  | CodebookMode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) for 4Tx (4,1) for 8Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Test specific MCS with common system bandwidth, subcarrier spacing, and TDD UL-DL pattern are listed in Table 2. Note that demodulation tests use only Ranks 4 and 8, whereas PMI tests use only Ranks 2 and 4. Also note that PDSCH data is allocated only in full slots, i.e. not in special slots in our simulations.

Table 2: Test parameters of 4RX and 8RX demodulation and PMI tests

| Test | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Test type |
| --- | --- | --- | --- | --- |
| Rank 2 | 40 / 30 | 16QAM, 0.48 (MCS13) | FR1.30-1 | PMI |
| Rank 4 | 40 / 30 | 16QAM, 0.48 (MCS13) | FR1.30-1 | Demodulation, PMI |
| Rank 8 | 40 / 30 | 16QAM, 0.48 (MCS13) | FR1.30-1 | Demodulation |

We have simulated CDL model options A2 (untruncated) and A4 (truncated) agreed after RAN#114bis. For the CDL model tables we used, see Appendix A. Our CDL channel model implementation follows TR38.827 randomness reduction, and employs ray-splitting for the untruncated A2 model where sub-clusters exist.

The CDL channel parameters are listed in Table 3. The concept of antenna array virtualization (AAV) via TX beamforming is illustrated in Figure 1. For CDL channel we have simulated AAV options 1 and 3.

Table 3: CDL channel parameters

| Carrier frequency | 3.5 GHz |
| --- | --- |
| UE speed | 3 km/h |
| UE travelling direction (fv, qv) | (65°, 90o) |
| Channel profile | 827 UMa CDL-C (A1) |
| Antenna array virtualization for 4RX | - AAV Option 3 (1, 2, 2, 1, 1) (Direct passthrough) - AAV Option 1 (8, 2, 2, 8, 1) (Fixed subarray size) |
| Antenna array virtualization for 8RX | - AAV Option 3 (1, 4, 2, 1, 1) (Direct passthrough) - AAV Option 1 (8, 4, 2, 8, 1) (Fixed subarray size) |
| Subarray virtualizer for Fixed subarray size | - Broadside (all ones) |
| BS (α, β, γ) | (0°, 10°, 0°) |
| BS polarization slant | (45°, -45o) |
| UE (α, β, γ) | (180°, 0°, 0°) |
| UE polarization slant | (0°, 90o) |
| BS antenna radiation pattern | As defined in TR38.901 Table 7.3-1 |
| UE antenna radiation pattern | Omnidirectional |

Figure 1: Antenna array virtualization (AAV) concept

## 2.2 PDSCH demod testing in CDL-C

All PDSCH demodulation simulations were run with MCS 13 and 3 km/h UE speed.

In Figure 2 we show PDSCH demodulation performance of 4TX-4RX Rank 4 using random precoding. In Figure 3 we show PDSCH demodulation performance of 8TX-8RX Rank 8 with random precoding, and in Figure 4 with fixed PMI (4, 0, 0) precoding.

Figure 2: 4TX-4RX CDL-C Rank 4 with random precoding

Figure 3: 8TX-8RX CDL-C Rank 8 with random precoding (left: total tput, right: per-CW tput)

Figure 4: 8TX-8RX CDL-C Rank 8 with fixed PMI (4, 0, 0) precoding (left: total tput, right: per-CW tput)

In Table 4 we list the required SNR in each test to achieve 70% relative throughput in 4TX-4RX Rank 4 using random precoding.

In Table 5 we list the required SNR in each test to achieve 70% relative throughput in 8TX-8RX Rank 8 using both random and fixed precoder.

Table 4: SNR [dB] at 70% tput for 4TX-4RX Rank 4 in CDL-C

| CDL-C | A2 | A4 |
| --- | --- | --- |
| AAV 1 / random precoding | 16.5 | 16.8 |
| AAV 3 / random precoding | 16.5 | 15.9 |

Table 5: SNR [dB] at 70% tput for 8TX-8RX Rank 8 in CDL-C

| CDL-C | AAV | precoding | CW0 | CW1 | Total |
| --- | --- | --- | --- | --- | --- |
| A2 | AAV 1 | random | 17.5 | 26.3 | 19.5 |
|  | AAV 3 |  | 17.0 | 24.3 | 18.8 |
| A4 | AAV 1 |  | 17.5 | 27.5 | 19.6 |
|  | AAV 3 |  | 16.7 | 24.0 | 18.4 |
| A2 | AAV 1 | fixed | 17.6 | 22.6 | 19.0 |
|  | AAV 3 |  | 17.0 | 20.7 | 18.5 |
| A4 | AAV 1 |  | 17.4 | 22.8 | 18.9 |
|  | AAV 3 |  | 17.0 | 19.7 | 18.1 |

## 2.3 Follow-PMI testing in CDL-C

In Figures 5-6 we show the performance of different PMI reporting options (Type1 WB-PMI, eType2 SB-PMI, and random Type1 with WB-PMI) with MCS13 and 3 km/h UE speed. Figure 5 depicts results for AAV option 1 and Figure 6 for AAV option 3. PMI feedback delay is 6.5 ms. For eType2, subband size 8 PRBs and paramCombination-r16 6 are used.

Figure 5: 8TX-4RX AAV Option 1 PMI reporting in CDL-C (left: rank 2, right: rank 4)

Figure 6: 8TX-4RX AAV Option 3 PMI reporting in CDL-C (left: rank 2, right: rank 4)

In Table 6 we list the required SNR to achieve 90% relative throughput in PMI tests with different PMI reporting options, ranks, and AAV options.

In Table 7 we list gammas (throughput ratio between follow and random PMI) for both Type1 WB-PMI and eType2 SB-PMI.

Table 6: SNR [dB] at 90% relative tput of 8TX-4RX PMI reporting in CDL-C

| CDL-C |  | Type1 follow WB-PMI | eType2 follow SB-PMI | Type1 random WB-PMI |
| --- | --- | --- | --- | --- |
| Rank 2 | A2 / AAV 1 | 2.9 | 2.7 | 14.2 |
|  | A2 / AAV 3 | 3.4 | 3.0 | 13.7 |
|  | A4 / AAV 1 | 3.9 | 3.5 | 14.4 |
|  | A4 / AAV 3 | 3.7 | 3.3 | 13.8 |
| Rank 4 | A2 / AAV 1 | 12.4 | 13.5 | 25.9 |
|  | A2 / AAV 3 | 12.8 | 13.6 | 24.2 |
|  | A4 / AAV 1 | 13.4 | 13.5 | 26.7 |
|  | A4 / AAV 3 | 12.6 | 12.6 | 23.2 |

Table 7: Gammas of 8TX-4RX PMI reporting in CDL-C

| CDL-C |  | Type1 WB-PMI | eType2 SB-PMI |
| --- | --- | --- | --- |
| Rank 2 | A2 / AAV 1 | 2.52 | 2.63 |
|  | A2 / AAV 3 | 2.34 | 2.49 |
|  | A4 / AAV 1 | 2.32 | 2.46 |
|  | A4 / AAV 3 | 2.30 | 2.44 |
| Rank 4 | A2 / AAV 1 | 1.85 | 1.72 |
|  | A2 / AAV 3 | 1.78 | 1.68 |
|  | A4 / AAV 1 | 1.76 | 1.75 |
|  | A4 / AAV 3 | 1.77 | 1.78 |

## 2.4 PDSCH demod testing in multi-cluster TDL channels

In this section, we show PDSCH demodulation simulation results of the multi-cluster TDL models 1, 2, and 3 as specified in [3]. The cluster model tables can also be found in Appendix B. All simulations employ LMMSE MIMO demodulation.

In Figure 7, we show the performance of 4TX-4RX-4L using random precoding for cluster models 1 and 2. In Figure 8, we show 8TX-8RX-8L with fixed PMI (0, 0 ,0) precoding for cluster models 1, 2, and 3.

Figure 7: 4TX-4RX Rank 4 with random precoding in multi-cluster TDL channels

Figure 8: 8TX-8RX Rank 8 with fixed PMI (0, 0 ,0) precoding in multi-cluster TDL channels

In Table 8 we list the required SNR in each test to achieve 70% relative throughput for 4TX-4RX Rank 4 using random precoding. In Table 9 we list the required SNR in each test to achieve 70% relative throughput for 8TX-8RX Rank 8 using fixed PMI (0, 0, 0) precoding.

Table 8: SNR [dB] at 70% tput for 4TX-4RX Rank 4 in multi-cluster TDL-C

| TDL-C cluster model 1 / random precoding | 15.8 |
| --- | --- |
| TDL-C cluster model 2 / random precoding | 17.2 |

Table 9: SNR [dB] at 70% tput for 8TX-8RX Rank 8 in multi-cluster TDL-C

|  | CW0 | CW1 | Total |
| --- | --- | --- | --- |
| TDL-C cluster model 1 / fixed precoding | 11.0 | 19.7 | 13.6 |
| TDL-C cluster model 2 / fixed precoding | 9.5 | 23.9 | 19.1 |
| TDL-C cluster model 3 / fixed precoding | 11.1 | 19.7 | 13.8 |

## 2.5 Follow-PMI testing in multi-cluster TDL channels

In this section, we show the follow-PMI simulation results of the multi-cluster TDL models 1 and 2 as specified in [3]. The cluster model tables can also be found in Appendix B. All simulations employ LMMSE MIMO demodulation.

In Figures 9-10 we show the performance of different PMI reporting options (Type1 WB-PMI, eType2 SB-PMI, and random Type1 with WB-PMI) with MCS13 and 10Hz Doppler. Figure 9 depicts the results in TDL cluster model 1, and Figure 10 in cluster model 2. PMI feedback delay is 6.5 ms. For eType2, subband size 8 PRBs and paramCombination-r16 6 are used.

Figure 9: 8TX-4RX PMI reporting in TDL cluster model 1 channel (left: rank 2, right: rank 4)

Figure 10: 8TX-4RX PMI reporting in TDL cluster model 2 channel (left: rank 2, right: rank 4)

In Table 9 we list the required SNR to achieve 90% relative throughput in PMI tests with different PMI reporting options, ranks, and TDL cluster models.

In Table 10 we list the gamma values (throughput ratio between follow and random PMI) for Type1 WB-PMI and eType2 SB-PMI.

Table 9: SNR [dB] at 90% relative tput of 8TX-4RX PMI reporting in multi-cluster TDL-C

|  | Cluster model | Type1 follow WB-PMI | eType2 follow SB-PMI | Type1 random WB-PMI |
| --- | --- | --- | --- | --- |
| Rank 2 | TDL-C cluster model 1 | 5.9 | 4.7 | 11.4 |
|  | TDL-C cluster model 2 | 5.4 | 4.5 | 13.8 |
| Rank 4 | TDL-C cluster model 1 | 14.4 | 16.6 | 19.3 |
|  | TDL-C cluster model 2 | 14.0 | 15.7 | 22.9 |

Table 10: Gammas of 8TX-4RX PMI reporting in multi-cluster TDL-C

|  | Cluster model | Type1 follow WB-PMI | eType2 follow SB-PMI |
| --- | --- | --- | --- |
| Rank 2 | TDL-C cluster model 1 | 1.69 | 1.89 |
|  | TDL-C cluster model 2 | 1.86 | 2.05 |
| Rank 4 | TDL-C cluster model 1 | 1.63 | 1.30 |
|  | TDL-C cluster model 2 | 1.69 | 1.53 |

# Conclusion

In this paper we provided our simulation setup and results for Spatial Channel Modelling.

# Reference

- R4-2504756, “Way Forward for [114bis][320] NR_SCM”, Nokia

- 3GPP TS 38.101-4 V18.6.0 (2024-12), User Equipment (UE) radio transmission and reception, Part 4: Performance requirements

- R4-2504762, “Simulation assumptions for SCM”, Huawei, HiSilicon, MediaTek

- R4-2505595, “TP to TR38.753: TDL approaches and related Annex”, MediaTek

# Appendix A: CDL-C A2 and A4 tables

Untruncated CDL-C A2 model used in our simulations:

Truncated CDL-C A4 models used in our simulations for AAV Options 3 and 1:

# Appendix B: Multi-cluster TDL-C tables

Table 11: Cluster model 1 with TDL-C and 4 clusters

| Delay profile | Cluster index | Power (dB) | θtx1 | θrx1 |
| --- | --- | --- | --- | --- |
| TDLC 300 | 1 | 0 | 0 | 0 |
| TDLC 300 | 2 | 0 | -π/2 | -π/2 |
| TDLC 300 | 3 | -9 | -π | -π |
| TDLC 300 | 4 | -9 | -3π/2 | -3π/2 |

Table 12: Cluster model 2 with TDL-C and 4 clusters

| TDLC 300 delay profile |  |  | TX-RX beam steering |  |  |
| --- | --- | --- | --- | --- | --- |
| Tap | Delay (ns) | Tap power (dB) | Cluster index | θtx1 | θrx1 |
| 1 | 0 | -6.9 | 1 | 0 | 0 |
| 2 | 65 | 0 |  |  |  |
| 3 | 70 | -7.7 | 2 | -π/2 | -π/2 |
| 4 | 190 | -2.5 |  |  |  |
| 5 | 195 | -2.4 |  |  |  |
| 6 | 200 | -9.9 |  |  |  |
| 7 | 240 | -8.0 |  |  |  |
| 8 | 325 | -6.6 |  |  |  |
| 9 | 520 | -7.1 | 3 | -π | -π |
| 10 | 1045 | -13.0 |  |  |  |
| 11 | 1510 | -14.2 | 4 | -3π/2 | -3π/2 |
| 12 | 2595 | -16.0 |  |  |  |

Table 13: Cluster model 3 with 2 TDL-C channels and 4 clusters

| TDLC 300 delay profile |  |  | TX-RX beam steering |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tap | Delay (ns) | Tap power (dB) | TDL channel 1 Power: 0 dB |  |  | TDL channel 2 Power: -9 dB |  |  |
|  |  |  | Cluster index | θtx1 | θrx1 | Cluster index | θtx1 | θrx1 |
| 1 | 0 | -6.9 | 1 | 0 | 0 | 3 | -π | -π |
| 2 | 65 | 0 |  |  |  |  |  |  |
| 3 | 70 | -7.7 |  |  |  |  |  |  |
| 4 | 190 | -2.5 |  |  |  |  |  |  |
| 5 | 195 | -2.4 | 2 | -π/2 | -π/2 | 4 | -3π/2 | -3π/2 |
| 6 | 200 | -9.9 |  |  |  |  |  |  |
| 7 | 240 | -8.0 |  |  |  |  |  |  |
| 8 | 325 | -6.6 |  |  |  |  |  |  |
| 9 | 520 | -7.1 |  |  |  |  |  |  |
| 10 | 1045 | -13.0 |  |  |  |  |  |  |
| 11 | 1510 | -14.2 |  |  |  |  |  |  |
| 12 | 2595 | -16.0 |  |  |  |  |  |  |