# 3GPP TSG RAN WG4 Meeting#115	R4-2505615

# St. Julians, Malta, May 19 – May 23, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements for 6Rx devices

Agenda item:	7.1.4

Document for:	Discussion

# 1. Introduction

During the last RAN4 meeting, the scope of the performance requirements for 6Rx WI including various simulation assumptions were discussed [1]. In this paper, we present our perspectives on the remaining open issue for defining demodulation and CSI reporting requirements.

2. General

2.1 Test applicability rule for 6Rx test cases

Test applicability rule for 6Rx test cases

- Way forward

- Option 1: RAN4 can introduce UE declaration on the 6Rx UE type (e.g., handheld or FWA) without capability signaling and design proper test applicability in the specification.

- Option 2:

  - 6 MIMO layers performance requirements will be only applicable for FWA devices

  - 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices.

During the previous meeting, several proposals were discussed regarding the includion of 6Rx demodulation requirements in the specification. The details of these proposals are outlined below.

We note that, under a separate issue, we are still deliberating whether to apply the same 4-layer requirements to both handheld and FWA devices. In our view, the discussion on including demodulation requirements in the specifications should commence only after the requirements discussions have concluded.

Proposal 1: Address the specification impact only after the requirements discussions have settled.

3. PDSCH Requirements for 6Rx

3.1 Test scenarios for 6 MIMO layers

Test scenarios

- Way forward

- Option 1: Test scenarios without interference: PDSCH Mapping Type A

- Option 2: Test scenarios with interference

  - PDSCH demodulation requirements with inter-cell interference

  - PDSCH demodulation requirements with intra-cell inter-user interference

During the last RAN4 meeting, the type of test scenario to be considered for PDSCH test cases for 6Rx demodulation requirements was discussed. The way forward is outlined below.

Since this is the first release where 6Rx requirements are being considered, we believe that test scenarios without interference, similar to those used for 8Rx, can be applied. Therefore, we support option 1.

Proposal 2: Support option 1 (test scenarios without interference: PDSCH Mapping Type A)

3.2 Test scenarios for 4 MIMO layers

Test cases for 4 MIMO layers for FWA and Handheld UE

- Way forward

- Option 1: One single test case for 4 MIMO layers that is applicable for both FWA and Handheld UE

- Option 2: Two separate test cases for 4 MIMO layers for FWA and Handheld UE

  - Option 2a: One test case with ULA Low for FWA; Another test case with ULA Medium with β = 0.00718 for Handheld UE

  - Other options

RAN4 has agreed to introduce 4-layer requirements for both FWA and handheld UE devices. Several proposals were discussed during the last meeting. The way forward is outlined below.

To this end, our preference is to consider a single test case for the 4 MIMO layers that applies to both FWA and handheld UE devices. This approach would minimize the number of test cases required for the 4-layer PDSCH demodulation requirements.

Proposal 3: Support option 1 (One single test case for 4 MIMO layers that is applicable for both FWA and Handheld UEs)

3.3 Propagation channel and MIMO correlation for 6 MIMO layers

RAN4

The propagation channel and MIMO correlation for 6 MIMO layers were discussed during the last meeting. The way forward is outlined below.

Propagation channel and MIMO correlation for 6 MIMO layers

- Way forward

- Option 1: TDLA 30-10 ULA Low

- Option 2: TDLD 30-5 ULA Low

We note that TDLD is a LOS channel model and there are very few requirements involving the TDLD channel in the current specification. Additionally, for the 8Rx FWA requirements, RAN4 defined demodulation requirements using the TDLA30-10 channel model only.

Proposal 4: Support option 1 (TDLA 30-10 ULA Low)

3.4 Propagation channel for 4 MIMO layers

Propagation channel for 4 MIMO layers

- Way forward

  - Option 1: TDLA30-10

  - Option 2: TDLC300-100

  - Option 3: TDLA30-10 and TDLC300-100

The propagation channel for 4 MIMO layers was discussed during the last meeting. The way forward is outlined below.

Similar to the 6L case, we propose considering TDLA 30-10 only for demodulation requirements applicable to 4 MIMO layers.

Proposal 5: Support option 1 (TDLA30-10)

3.5 MIMO correlation for 4 MIMO layers

The MIMO correlation for 4 MIMO layers was also discussed during the last meeting. The way forward is outlined below.

MIMO correlation for 4 MIMO layers

- Way forward

  - Option 1: ULA Medium

RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β425*β925*β425*β125*1

    - Option 1a: with β = 0.00718

    - Option 1b: with β = 0.005154

Option 2: ULA Low

Similar to the 6L case, we propose to consider ULA Low only for 4-layer demodulation requirements.

Proposal 6: Support option 2 (ULA Low)

3.6 DMRS configuration for 6 MIMO layers case

DMRS configuration for 6 MIMO layers case

- Way forward

- Option 1: Rel-15 Type 1 DMRS 2+2

- Option 2: Rel-15 Type 1 DMRS 2+0

- Option 3: Rel-18 eType1 DMRS 1+1

The DMRS configuration for 6 MIMO layers case was briefly discussed during the last meeting. The way forward is outlined below.

We note that Rel-18 eType1 DMRS is likely to become more prevalent in advanced 5G use cases due to its reduced overhead and better support for higher-order MIMO. Additionally, RAN4 has already introduced requirements for eType1 DMRS during the Rel-18 timeframe [2]. Additionally, based on the simulation results provided in our contribution for RAN4#114 [3], it is evident that Rel-18 eType1 DMRS offers comparable performance with just one frontloaded DMRS symbol compared to the Rel-15 2+0 case. It also provides robustness in high Doppler scenarios due to the additional DMRS symbol. While the Rel-15 2+2 DMRS configuration could be considered for 6Rx requirements, it is important to note that this configuration significantly increases the DMRS overhead, resulting in lower peak throughput. After evaluating all these options and analyzing their pros and cons, we believe that eType1 DMRS should be considered for 6-layer performance requirements for 6Rx devices.

Proposal 7: Support option 3 (Rel-18 eType1 DMRS 1+1)

3.7 Antenna configuration for 4 MIMO layers case

Antenna configuration for 4 MIMO layers case

- Way forward

- Option 1: 4Tx6Rx

- Option 2: 8Tx6Rx

The way forward from the previous meeting is outlined below.

We note that 4L requirements were defined with a 4Tx configuration for 8Rx WI. Following this precedent, we propose considering a 4Tx configuration for 4L requirements in 6Rx WI.

Proposal 8: Support option 1 (4Tx6Rx)

3.8 MCS for 6 MIMO layers case

MCS for 6 MIMO layer case

- Way forward

- Option 1: MCS13 (Table 2) : 64QAM with CR = 567/1024

- Option 2: MCS 13 as starting point (Table 1) : 16QAM with CR = 490/1024

- Option 3: 256QAM

- Option 4: 64QAM (MCS Table 1)

- Option 5: MCS17 as starting point (Table 1): 64QAM with CR = 438/1024

- Option 6: MCS13 and MCS 17 as starting point

- Way forward

- Option 1: MCS13 (Table 2)

- Option 2: MCS 13 as starting point (Table 1)

- Option 3: 256QAM

- Option 4: 64QAM (MCS Table 1) with the same MCS for both CW

- Option 5: MCS17 as starting point (Table 1)

The way forward from the previous meeting is outlined below.

There are multiple proposals currently under consideration for MCS choices in the context of 6 MIMO layers. We note that both 8-layer and 4-layer requirements in the 8Rx WI were defined with MCS Table 1 (MCS17, 64QAM) [2]. However, our evaluation suggests that higher MCS levels can be achieved with 6Rx, even for the agreed TxEVM, using MCS Table 2 (256QAM). Therefore, we propose that RAN4 investigate the feasibility of defining requirements with MCS20 from MCS Table 2 for the 6 MIMO layers scenario.

Proposal 9: MCS Table 2 with MCS 20 as a starting point for 6 MIMO layers case.

3.9 MCS for 4 MIMO layers case

The way forward from the previous meeting is outlined below.

MCS for 4 MIMO layer case

- Way forward

- Option 1: MCS13 (Table 2) : 64QAM with CR = 567/1024

- Option 2: MCS 13 as starting point (Table 1) : 16QAM with CR = 490/1024

- Option 3: 256QAM

- Option 4: 64QAM (MCS Table 1)

- Option 5: MCS17 as starting point (Table 1): 64QAM with CR = 438/1024

- Option 6: MCS13 and MCS 17 as starting point

Similar to the 6-layer case, there are multiple proposals currently under consideration for MCS choices in the context of 4 MIMO layers. We propose that RAN4 also investigate the feasibility of defining requirements with MCS20 from MCS Table 2 for the 4 MIMO layers scenario.

Proposal 10: MCS Table 2 with MCS 20 as a starting point for 4 MIMO layers case.

4. SDR Requirements for 6Rx

4.1 SDR requirements for 1024QAM

SDR requirements for 1024QAM

- Agreement

- Define SDR requirements for 1024QAM with 2 layers

- FFS Define SDR requirements for 1024QAM with 4 layers

During the last meeting, RAN4 agreed to introduce SDR requirements for 1024QAM with 2 layers. However, the decision on whether to introduce SDR requirements for 1024QAM with 4 layers was deferred for future study. The outlined way forward is as follows:

We believe that the SNR cap is too low due to EVM limitations for meaningful 1024QAM tests with 4 layers. Therefore, we propose not to introduce SDR requirements for 1024QAM with 4 layers.

Proposal 11: Do not introduce SDR requirements for 1024QAM with 4 layers.

5. CSI Requirements for 6Rx

5.1 CQI test for 6Rx

CQI test for 6Rx

- Way forward

- Option 1: Not define CQI test

- Option 2: Define CQI test

  - Option 2a: Only define CQI requirements with rank 4 under static channel

  - Option 2b: Define the following CQI test

    - Periodic CQI reporting requirements under AWGN

    - Wideband CQI reporting requirements with inter-cell interference under fading conditions

During the last meeting, RAN4 discussed the introduction of CQI reporting requirements for 6Rx. Several proposals are currently under consideration. The way forward is as follows.

We believe that higher CQI/MCS levels are achievable with 6Rx, even at rank 4, compared to 4Rx alone. Therefore, it makes sense to introduce CQI reporting requirements. Additionally, we think it is not essential to consider interference scenarios or fading channels, as the primary goal is to verify CQI reporting functionality and performance under 6Rx.

Proposal 12: Support option 2a (only define CQI requirements with rank 4 under static channel).

5.2 RI test for 6Rx

During the last meeting, RAN4 briefly discussed whether to introduce RI reporting requirements for 6Rx. However, no agreements were reached. The way forward is as follows.

RI test

- Way forward

- Option 1: Not define RI test for 6 layers

- Option 2: Define RI test for 6 layers

We note that there were no RI tests with 6 layers for 8Rx WI during Rel-18. Furthermore, the feasibility study for 6-layer handheld requirements found that rank 6 is feasible for 6Rx, even for handheld UE devices. Therefore, it would be justified to consider introducing an RI test with 6 layers for 6Rx FWA UE requirements.

Proposal 13: Support option 2 (Define RI test for 6 layers)

6. Conclusions

In this paper, we share our perspectives on the remaining open issues regarding simulation assumptions for the 6Rx demodulation and CSI reporting requirements. The following proposals have been made:

Proposal 1: Address the specification impact only after the requirements discussions have settled.

Proposal 2: Support option 1 (test scenarios without interference: PDSCH Mapping Type A)

Proposal 3: Support option 1 (One single test case for 4 MIMO layers that is applicable for both FWA and Handheld UEs)

Proposal 4: Support option 1 (TDLA 30-10 ULA Low)

Proposal 5: Support option 1 (TDLA30-10)

Proposal 6: Support option 2 (ULA Low)

Proposal 7: Support option 3 (Rel-18 eType1 DMRS 1+1)

Proposal 8: Support option 1 (4Tx6Rx)

Proposal 9: MCS Table 2 with MCS 20 as a starting point for 6 MIMO layers case.

Proposal 10: MCS Table 2 with MCS 20 as a starting point for 4 MIMO layers case.

Proposal 11: Do not introduce SDR requirements for 1024QAM with 4 layers.

Proposal 12: Support option 2a (only define CQI requirements with rank 4 under static channel).

Proposal 13: Support option 2 (Define RI test for 6 layers)

7. References

[1] R4-2503014, “WF on 6Rx UE performance requirements,” 3GPP TSG RAN WG4 Meeting #114-bis, Wuhan, China, Apr 7 – 11, 2025.

[2] TS 38.101-4, ‘User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)’, v18.6.0, Jan 2025.