# 3GPP TSG-RAN WG4 Meeting #115	R4-2505618

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.26.3.2

Source: 	CMCC

Title: 	Discussion on A-IoT device RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #106 meeting, a new WI[1] of A-IoT is approved with following RAN4 lead RF part objectives:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In this contribution, we discuss the RF requirements A-IOT devices.

- Discussion

2.1 Tx requirements

- Backscattering power

Last meeting agreements are listed as below:

Issue 3-1-1: Transmit output power Agreement: - Performance metric of transmit output power: - Backscattering loss - Backscattering output power is defined as the minimum backscattered power of Device 1 at certain input CW Tx power - Minimum Backscattering power at device antenna= Input CW Tx power at device antenna−Backscattering loss - For the Input CW Tx power to define requirements: - FFS on the whether to use one value or a range of CW Tx power - FFS on the value(s) of CW Tx power - Consider following aspects to determine backscattering loss to define requirements: - modulation loss - antenna efficiency - backscattering circuitry loss - impairment margin - minimum distance to the reader - differential radar cross section in two different modulation states - differential reflection coefficient in two different modulation states - the spectral components of the backscatter signal to be included in the power measurement, e.g. only the 1st Lower SB and 1st Upper SB, excluding the reflected carrier itself - Other factors not excluded

One issue is the gain orientation for Tx requirements. Define requirements like the spherical coverage will give more flexibility for final device deployment, e.g. no limiting the device orientation. It’s suggested to define spherical coverage related requirements. Besides, when defining Rx requirements, we also consider orientation related requirements. Both Tx and Rx are used to define orientation related requirements, to reduce testing time, we can only define one orientation requirements.

Proposal 1: it’s suggested to only define either Tx backscatter power or Rx REFSENSE with orientation requirements.

- SFO requirements

Issue 3-2-1: SFO requirement Agreement: - 10% SFO is assumed as the minimum performance for devices. - FFS on whether to define SFO requirement and how to test the SFO requirement if defined.

- As discussed in D2R channel bandwidth, SFO already been reflected into CBW definition which will be considerred during all other RF requirements testing/definition. At reader side, reader could know device’s SFO performance by using midable/preamble. So even device has worst SFO performance, it doesn’t impact reader’s receiver performance. So no need for further SFO requirements.

- Proposal 2: no needs for further SFO requirements.

- Modulation quality requirements

Issue 3-2-2: Modulation quality requirements Agreement: - Define modulation quality requirements for both BPSK and OOK. - Consider to define same modulation quality metric for both BPSK and OOK if possible. - Reference measurement process needs to be discussed

Device support OOK and BPSK modulation scheme. For NR BPSK modulation, 17.5% EVM is defined which equals to 15dB SNR. In theory this is the max SNR at receiver side. For A-IoT, from D2R coverage point of view, reader may receive such high SNR signal, so this EVM requirement is still significant. But since OOK signal has no EVM related concept, EVM can’t be used as one uniform requirements for both BPSK and OOK. It’s suggested to use RF plusewidth as signal quality for both BPSK and OOK. Now device doesn’t support PIE code, so the basic time unit should be updated as chip duration, chip duration is equalt to 1/2 Tari. So legacy RF pluswidth should also be updated accordingly as min = 0.53 and max = 1.05 chip duration

- Occupied bandwidth

- Following list the background of ITU, UTRA, E-UTRA and NR occupied bandwidth requirements.

- Definition in ITU background

- the occupied bandwidth enables operating agencies, national and international organizations, to carry out measurements to quantify the bandwidth actually occupied by a given emission and thus to ascertain, by comparison with the necessary bandwidth, that such an emission does not occupy an excessive bandwidth for the service to be provided and is, therefore, not likely to create interference beyond the limits laid down for this class of emission;

UTRA Background

Occupied bandwidth = The width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage β/2 of the total mean power of a given emission. Unless otherwise specified by the Radiocommunication Assembly for the appropriate class of emission, the value of β/2 should be taken as 0.5%.

Occupied bandwidth is used in some regions as a regulatory requirement. It is not defined as a regulatory requirement by the FCC or CEPT/ECC and it is not used as input to any compatibility studies. It is today a mandatory requirement for the UE in RAN4 specifications.

E-UTRA/NR requirements

Occupied bandwidth is a measure of the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel.

Observation 2: occupied bandwidth concept dating from ITU recommendation. The main purpose of it is used as comparision with the necessary bandwidth to identify whether the excessive bandwidth creating extra interference beyond the spurious limits.

- For OOK-4 signal, the harmoics will only occupies part of power, e.g. 99% power will include 5th harmocis. So the occupied bandwidth will be larger than the necessary bandwidth. There is no ITU guidline of how to identify actual spurious emission when occupied bandwidth larger than necessary bandwidth. When occupied bandwidth larger than NB, the excessive bandwidth will create interference beyond the limits which means if we still use current requirements, this is much hard to be achieved for device since it’s occupied bandwidth is larger than NB. So reusing current spurious limit will meet ITU requirements even the occupied bandwidth is larger than NB.

- Observation 3: reusing current spurious limit will meet ITU requirements and is hard for device to meet since its occupied bandwidth is larger than NB.

- One reason in RAN4 to define occupied bandwidth is to show the spectrum efficiency rather than from co-existence/interference point of view. For device, the modulation scheme decides it will not have very high spectrum efficiency. so no need for the occupied bandwidth requirements as long as we can ensure no extra interference between devices or between device and other system.

- Proposal 3: no need for occupied bandwidth.

- Emission requirements

Issue 3-3-1: Emission requirements Agreement: - Discuss whether to define EIPR or TRP as performance metric for emission requirements - Spurious emission: reuse legacy NR UE spurious emission requirement and further discuss the OOB boundary. - FFS on CW maximum input power level for testing

- Emission requirements is used to evaluate interference to other system. From victim system point of view, TRP basis emission requirements is needed to help analize received interference. Besides, in actual deployment scenario, victim system is everywhere, there is no requriements for certain direction since EIRP is the requirements only for for direction. The only advantages of EIRP performance is reduing testing time. But if we define the EIRP requirements as max emission requirements among all directions. then during the testing, we still have to 3D-scan all directions to find the max EIRP direction which will cost the similar time as TRP requirements. So, TRP performance metric is preferred.

- Proposal 4: use TRP as performance metric for emisison requirements.

- Different from NR system, the device RF part may have no impact on SEM performance so we have two options for final SEM requirements

- No SEM requirements

- SEM requirements only based on signal characteristics

- Observation 4: device RF part have no impact on SEM performance so we have two options for final SEM requirements.

- No SEM requirements

- SEM requirements only based on signal characteristics

From regulatory point of view, we prefer to still define SEM requirements only considering the signal characteristics.

During the SI evaluation, 7.5kHz chip rate is used to derive leakage performance with assumed flat SEM model.  at that SI phase, RAN1 have no conclusion of final Tb/R/Tchip. So at WI stage, it’s suggested to re-evaluate SEM requirements with flat SEM assumption, one solution is to use max chip rate based on RAN1 agreements. During SI, we propose 16dBc in R4-2411765 with no RF impact. Certain relaxation is also allowed based on digital signal performance to consider the RF impacts. Besides, the SEM requirements applies from the edge of D2R CBW.

Detailed analysis of SEM parameters are listed as below:

Following list our initial simulation for the SEM performance with following parameters parameter; value. Data rate; We consider 7kbps. Larger data rate is the worst case.. coding; The FEC impact for final spectrum performance is tiny. But this will impact finally chip rate, we use 1/3code rate convolution code. Compared with miller code, Manchester code has wider spectrum characteristics, it’s suggested to use Manchester code.. Modulation scheme; The spectrum characteristics of OOK modulation will much concentrate on center but BPSK will have wider spectrum bandwidth, we can use BPSK.. Following fig show the spectrum characteristics based on above parameters. Observation 7: OOK spectrum will much concentrate in the center frequency compared with BPSK. Besides, larger chip rate, the larger spectrum bandwidth and worse leakage performance. Compared with miller code, manchester code have worst leakage performance. When assuming D2R transmit at the frequency center, The Tx leakage is 16dBc at the 90kHz offset from center.

Proposal 5: SEM applies from the edge of CBW with flat model assumption. 16dBc is suggested as baseline with following assumptions.

| parameter | value |
| --- | --- |
| Data rate | We consider 7kbps. Larger data rate is the worst case. |
| coding | The FEC impact for final spectrum performance is tiny. But this will impact finally chip rate, we use 1/3code rate convolution code. Compared with miller code, Manchester code has wider spectrum characteristics, it’s suggested to use Manchester code. |
| Modulation scheme | The spectrum characteristics of OOK modulation will much concentrate on center but BPSK will have wider spectrum bandwidth, we can use BPSK. |

For spurious emission, legacy OOB of UE requriements is mainly used to leave transient bandwidth for RF band filter. For device, there is no transmit RF band filter. As discussed for occupied bandwidth, device has larger occupied bandwidth than NB which will be more easily to create interference. In theory, it would be much stricter for device to meet ITU spurious emission requirements. But spurious emission requirments come from ITu, device should still meet such requirements. So the OOB boundary should be larger than 500kHz or 10*NB, here the NB is suggested to equal to D2R CBW.

Proposal 6: the OOB is suggested to be defined as Fb as in ITU, i.e. max of 500kHz and 10 times NB where NB equals to D2R CBW.

2.1 Rx requirements

- REFSENSE

Issue 3-4-1: Reference sensitivity Agreement: - Define minimum requirements of REFSENS based on link budget (target is [15m]) and measurement results reported by companies - Consider to use inventory procedure for REFSENS testing, success rate ([90%]) or miss detection rate ([10%]) as the metric for inventory - At least to define maximum gain direction sensitivity requirements - FFS on how to different sensitivity requirements on different orientations

- The REFSENSE analysis is listed as below

| reader power | 33 |
| --- | --- |
| Coupling loss | 63 |
| REFSENSE | -30 |

- Proposal 7: REFSENSE is suggested as -30dBm

In actual deployment environment, the tag will rotae in all directions, so spherical coverage like receiver requriements could be defined and we can define smaller CDF point to ensure larger tag rotate ranges. Usually device will have omni-directional or using dipole antenna (omi-directional in ɵ plane).

Proposal 8: define spherical coverage like Rx requirements.

- Max input power

Issue 3-5-1: Maximum input power Agreement: - Define maximum input level, consider following options: - Option 1: using [1m] distance assumption from reader to device - Option 2: using square law appliance range - Option 3: using dynamic range of device receiver

If we assume 1m distance, the max input level is 38dBm+3dB(2Tx assumption) - 31.59=9.41dBm, if assume -30dBm REFSENSE which means 39.41dB dynamic range. Typical dynamic range for tag is in the range from 30dB to 40dB, if we choose 30dB dynamic range, max input level is -30+30=0dBm.

Proposal 9: [0]dBm max input power with 30dB dynamic range assumption.

# 3. Conclusions

In this contribution, RF requirements for device 1 are discussed with following observations and proposals.

Observation 1: testing show backscatter loss for devices is almost 6dB when testing by connecting antenna connector for OOK modulation.

Proposal 1: it’s suggested to only define either Tx backscatter power or Rx REFSENSE with orientation requirements.

- Proposal 2: no needs for further SFO requirements.

Observation 2: occupied bandwidth concept dating from ITU recommendation. The main purpose of it is used as comparision with the necessary bandwidth to identify whether the excessive bandwidth creating extra interference beyond the spurious limits.

- Observation 3: reusing current spurious limit will meet ITU requirements and is hard for device to meet since its occupied bandwidth is larger than NB.

- Proposal 3: no need for occupied bandwidth.

- Proposal 4: use TRP as performance metric for emisison requirements.

- Observation 4: device RF part have no impact on SEM performance so we have two options for final SEM requirements.

- No SEM requirements

- SEM requirements only based on signal characteristics

Proposal 5: SEM applies from the edge of CBW with flat model assumption. 16dBc is suggested as baseline with following assumptions.

| parameter | value |
| --- | --- |
| Data rate | We consider 7kbps. Larger data rate is the worst case. |
| coding | The FEC impact for final spectrum performance is tiny. But this will impact finally chip rate, we use 1/3code rate convolution code. Compared with miller code, Manchester code has wider spectrum characteristics, it’s suggested to use Manchester code. |
| Modulation scheme | The spectrum characteristics of OOK modulation will much concentrate on center but BPSK will have wider spectrum bandwidth, we can use BPSK. |

Proposal 6: the OOB is suggested to be defined as Fb as in ITU, i.e. max of 500kHz and 10 times NB where NB equals to D2R CBW.

- Proposal 7: REFSENSE is suggested as -30dBm

Proposal 8: define spherical coverage like Rx requirements.

Proposal 9: [0]dBm max input power with 30dB dynamic range assumption.

# 4. Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- R4-2420470, WF on RF requirements for A-IOT, Moderator (Huawei).