# 3GPP TSG-RAN WG4 Meeting #115	R4-2505619

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.26.2

Source: 	CMCC

Title: 	Discussion on A-IoT System parameters

Document for:	Discussion

# 1. Introduction

In RAN plenary #106 meeting, a new WI [1] of A-IoT is approved with following RAN4 lead RF part objectives:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In this contribution, we discuss the system parameters for A-IOT.

- Discussion

The conclusion on system parameters in study item are captured in following table.

| System parameter for A-IoT |  | Potentially needed or not |  |  |
| --- | --- | --- | --- | --- |
|  |  | A-IoT BS | A-IoT device | Intermediate UE |
| Operating band |  | YES |  |  |
| Channel bandwidth | Transmission bandwidth | YES the value needs be discussed in WI phase depending on other WG conclusion. |  |  |
|  | channel bandwidth | YES |  |  |
| Channel arrangement | Channel spacing | TBD in WI phase for standalone NA for in-band/Guard-band |  |  |
|  | Channel raster | IB/SA:YES GB:TBD | GB:NO IB/SA:TBD | IB/SA:YES GB:TBD |
|  | Synchronization raster | NO for device 1/2a |  |  |

- D2R channel bandwidth

Last meeting agreement is listed as below:

- Define D2R channel bandwidth for A-IoT device: - -Based on 2SB transmission, - -Consider SFO impact - -Consider small frequency shift defined in RAN1 - -D2R transmission bandwidth (2 times of 1SB), consider bit rate defined in RAN1

RAN1 agreements for SFS factor R are listed as below and RAN1 will further down-selection of bit duration and SFS factor.

According to RAN1 agreements, the frequency shift equals to +/- R/Tb Hz and the transmission bandwidth(2SB) when R=1 equals to 2/Tb. Following table lists the total transmission bandwidth which start from the lower edge of lower SB to the higher edge of higer SB considering SFS and SFO: 1/Tb*1.1+2*1.1*R/Tb.

Table 1 The total transmisison bandwidth based on current RAN1 agreements are listed as below for information:

There are so many transmit bandwidth and it’s hard to use all these for CBW requirements in RAN4. before discussing the CBW definition. At first we lists the main motivations of CBW definition as below:

- Outdoor NR will blank corresonding resrouces and it’s noted NR BS will reserve in PRB granularity. so NR outdoor BS need to know the reserved PRB.

- Use such channel bandwidth edge to define RF requirements to avoid the interference to other system, e.g NR system

- Use channel bandwidth to scale RF requirements, e.g. unwanted emission

Following list some options for D2R channel bandwidth definition.

- Option 1: use equation rather than explicit values for D2R CBW definition which will make the spec much clear. Fixed or a set of spectrum efficiency is required to transfer transmission to CBW, e.g. CBW equals to (1/Tb*1.1+2*1.1*R/Tb)/spectrum efficiency

- Option 2: define D2R CBW in RB/SCS granularity which will reduce number of CBW values

- For option 1, following equaltion is the transmission bandwidth: 1/Tb*1.1+2*1.1*R/Tb. For NR, CBW euqals to transmission divide spectrum efficiency. Here spectrum efficiency is decided by BB shaping filter. It’s noted for NR, final spectrum efficiency is compromise among all companies and there is no clear rule for different CBW. For device 1, no BB shaping filter and the main motivation to reuse fixed spectrum efficiency e.g. 90% is to leave some space to achieve target unwanted emission requirements due to device’s imperfection.

For option 2, the main motivation is to reduce the number of transmission bandwidth. Besides, current NR system will reserve resource for in-band A-IoT in per PRB/SCS basis.

Following list the candidate RB numbers which Round up in RB(180kHz) units on top of table 1. Besides, for the bandwidth less than 1RB, the candidate bandwidth value is round up in SCS units (15kHz). noted, here the round up is something like the spectrum efficiency used in option 1 and both is to leave space to achieve target unwanted emission requirements due to device’s imperfection.

15kHz, 30kHz, 45kHz, 60kHz, 75kHz, 90kHz, 105kHz, 120kHz, 150kHz, 1PRB, 2PRB(360kHz), 3PRB(540kHz), 4PRB(720kHz), 5PRB (900kHz), 6PRB(1080kHz),7PRB(1260kHz), 8PRB (1440kHz), 9PRB(1620kHz), 10PRB(1800kHz), 12PRB (2160kHz), 13PRB (2340kHz), 14PRB(2520kHz), 15PRB(2700kHz), 18PRB(3240kHz)

- there are two options for D2R CBW defintion

- Option 1: use equation rather than explicit values for D2R CBW definition which will make the spec much clear. Fixed or a set of spectrum efficiency is required to transfer transmission to CBW, e.g. CBW equals to (1/Tb*1.1+2*1.1*R/Tb)/spectrum efficiency

- Option 2: define D2R CBW in RB/SCS granularity which will reduce number of CBW values, following list the candidate value which is round up in RB/SCS units based on calucalated transmission bandwidth.

The main drawback of large unmber of CBW is it will lead to longer testing time if we testing each CBW for all RF requirements. But now RAN5 define test case with testing configuration which will mainly consider lower, middle and higher CBW of partof them for each RF requirements. So it seems both above option 1 and option 2 will not enlarge testing time.

- Proposal 1: it’s suggested to define D2R CBW for all R, bit duration and chip duration values based on RAN1 conclusion.

- Considering there are so many CBW values, defining RF requirements for all CBW will make the spec much complex. Besides, testing will cost much time. One solution is define RF requirements only for typical CBW and detailed CBW value could be choosen case by case.

- Proposal 2: it’s suggested to define RF/testing requirements only for typical CBW. Details CBW can be decided case by case.

Another issue is whether to define in-band RF requirements which applies within the CBW. The main motivation of such requirements is to avoid the interfernece due to FDMA operaiton among dvices. In RAN1, using different SFS, the mainlobe will not be overlapping among different devices. But it’s very hard to ensure not overlapping between mainlobe and odd harmonics. In theory, RAN4 may needs to define the in-band requirements something like the IBE to make sure the FDMA operation among devices, especially to avoide the interference due to odd harmonics.

Observation 1: after careful SFS design, mainlobes will not overlap with each other among devices, but one device’s mainlobe may overlap with another device’s odd harmonics due to SFO. Interference occurs between devices using FDMA operation.

So in theory, RAN4 may needs the requirements for device Tx within the CBW but outside the transmission bandwidth, which is something like the IBE requirements. Before discussing such requirements, following list summary of IBE requirements.

The general part of IBE include three parts, following list the reason of each part.

- First part: , here the -25dBc is the impat of phase noise.The LO leakage (IQ origin offset) is already excluded as part of the measurement definition. It was pointed out at RAN4 #46b that this doesn’t eliminate all impacts of the phase noise. To solve this, -25dBc is proposed around the DC frequency.  part is emission floor PSD relative to the total transmitted power in all allocated RBs

- Second prat: ,  The IFFT quantization noise is the dominant factor creating in-band emissions outside of the IM3 area around the allocated RBs.

- Third part:  , this is used to avoid too strict requirements when considering transmission at minimum power level. For example, with a single RB allocated BW and 20MHz system BW, the mission floor would be -85dBm/180kHz at the minimum Tx power of -40dBm.  This would be difficult to meet. Therefore it would be useful to have an absolute limit and the requirement would be the looser of the relative and absolute limits.  The WCDMA off power limit is -56dBm/3.84MHz, which can be scaled to about -69dBm/180kHz. -57dBm/180kHz is proposed as LTE requirement. NR update based on SCS value.

Observation 2: the UE IBE general requirements includes three parts which are used to reflect phase noise, IFFT quantization noise, avoiding too strict requirements at minimum power level.

For device, there is no IFFT quantization noise and no phase noise. So it seems at RAN4 parts, no need for new in-bands requirements which means RF part impairment for in-band emission is negigible for device. It’s noted there may be certain backscattered loss but this loss applies for the whole CBW.

Proposal 3: no needs for RF requirements within D2R CBW.

- Channel raster

Last meeting agreement is listed as below

- Issue 2-3-1: R2D channel raster - Agreement: - Consider following options for R2D channel raster - Option 1: define 10KHz as R2D channel raster (i.e. same as enhanced channel raster) - Option 2: Define channel raster considering the RB orthogonality between macro BS and A-IoT BS - Other options are not precluded.

Above option 1 is from enhanced channel raster which is mainly used for the case when gNB’s RB number is odd/even but UE’s RB number is even/odd for the bands using 100kHz channel raster. The main reason is that the carrier center for odd and even RB number is different, the former is at the center of the center RB but the later is at the edge of certer two RB. For A-IoT D2R there is no channel raster concept. So no needs to reuse enhanced channel raster. We can use the same channel raster as macr BS to ensure the orthogonality.

Proposal 4: Define the same channel raster as macro reader.

# 3. Conclusions

In this contribution, system parameters are discussed with following proposals.

- Proposal 1: it’s suggested to define D2R CBW for all R, bit duration and chip duration values based on RAN1 conclusion.

- Proposal 2: it’s suggested to define RF/testing requirements only for typical CBW. Details CBW can be decided case by case.

Observation 1: after careful SFS design, mainlobes will not overlap with each other among devices, but one device’s mainlobe may overlap with another device’s odd harmonics due to SFO. Interference occurs between devices using FDMA operation.

Observation 2: the UE IBE general requirements includes three parts which are used to reflect phase noise, IFFT quantization noise, avoiding too strict requirements at minimum power level.

Proposal 3: no needs for RF requirements within D2R CBW.

Proposal 4: Define the same channel raster as macro reader.

# 4. Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- R4-2420470, WF on RF requirements for A-IOT, Moderator (Huawei).

- R4-2417088, WF on RF requirement impact of A-IoT, Huawei.

- R4-2414305, WF on RF requirements for A-IoT, Huawei

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID, Specification for RFID Air Interface Protocol for Communications at 860 MHz - 960 MHz, Version 2.0.0 Ratified