# 3GPP TSG-RAN WG4 Meeting #115	R4-2505620

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.26.5

Source:	CMCC

Title:	Discussion on OTA test method for A-IoT device

Document for:	Approval

1	Introduction

In RAN#106 meeting, a new WI Solutions for Ambient IoT (Internet of Things) in NR was approved [1]. following list the objective of this OTA test part.

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

- In this paper, we will focus on the following issues for OTA testing.

2	Discussion

Last meeting agreement for general part is listed as below:

Issue 4-1-1: testing frequencies Agreement: - Only test low, middle and high frequencies within a band and average the performance metrics from the three frequencies - Check whether three frequencies are applicable for all requirements.

- It’s noted the frequency range listed in 38.870 include all low, middle and high frequency ranges. But RAN5 define test configurations case by case for each RF requirements and the detailed test frequencies for different RF requirements are different. Some requirements are tested for low, middle and high but some requirements are tested only for middle range. Details are listed as below table.

| - RF requirements | - testing frequency range |
| --- | --- |
| - CBW | - - |
| - Channel raster | - - |
| - Transmit output power | - Low, middle and high |
| - Modulation quality | - Low, middle and high |
| - Occupied bandwidth | - Mid range by default, exceptions listed in Table 6.5.1.4.1-2 in TS 38.521-1 |
| - Emission requirements | - Low range, High range |
| - Reference sensitivity | - Low, middle and high |
| - Max input power | - middle |

- Observation 1: testing frequency range is defined uniform as general part in TR 38.870. but in TS 38.521-1, testing frequency range is defined case by case.

- For transmit output power and modulation quality, testing frequency range includes low, middle and high range.

- For occupied bandwidth and max input power, testing frequency range include middle range.

- For emission and REFSENSE requirements, testing frequency range include low and high ranges.

- From out understanding, 38870 only define the general principles for frequency range and it’s RAN5’s responsibility to define detailed test frequency range for each RF requirements case by case. For A-IoT we can only list all low, middle and high frequency ranges as general in the TR but leave RAN5 to define detailed test condifurations case by case.

- Proposal 1: list all low, middle and high frequency ranges as general part in TR 38.870 and leave RAN5 to determine detailed testing frequency range case by case for each RF requirements.

- Last meeting agreements for test setup are listed as below, main remaining issues focus on how to ensure the isolation between CW node and measurement antenna.

- Topic 4-2: Test setup Issue 4-2-1: testing antenna and CW antenna relationship Agreement: Whole sphere scan is supported The position of CW antenna is a set of fixed position(s) as baseline (depending on core requirements discussion). - FFS on relative positions between reader antenna and CW Issue 4-2-2: how to ensure the isolation between CW node and measurement antenna Agreement: - Further discuss power difference between backscatter signal and the CW signal leakage, e.g. 6dB, 12dB, etc. - Encourage TE vendors provide the feasibility of TE extracting D2R power when CW source is transmitting.

- the motivation of ensuring isolation between CW and measurement antenna is to avoid impact from CW signal to device’s transmit requirements testing. Device Tx requirements include output power, unwanted emission and spurious requirements. For the output power it is within the D2R bandwidth where the CW wanted signal and phase noise works. For the uwnanted emission and spurious requirements it is outside the D2R bandwidth, basicly the CW leakage works.

- When testing device backscattered power, CW transmit power is much higher, e.g. 53dB difference assuming 33dBm CW output power, -20dBm backscatterred power and the same distance to testing antenna without isolation. It’s very hard to only use isolation method to avoid CW signal impact because accoding to the RFID testing only 56dB isolation is required. Besides, the CW wanted signal will only occipies narrow bandwidth and will be easily to be extracted from received signal. Even if TE can’t extracted the CW power, we can at first testing the received CW wanted power at testing antenna. It’s noted we can only test the received power from CW once for all testing devices which will not cost much time.

- CW phase noise will impact device backscattered power but according to the initial analysis from companies, the phase noise is in the range from -110~-160dBc/Hz which is much lower than backscattered power from device, so no need to ensure the isolaiton from CW phase noise to measurement antenna.

- the main motivation of isolation between CW node and testing antenne is to avoid impact from CW leakage to device unwanted and spurious emission requirements.

- Usually during BS testing, we assume the TE NF should be 6dB less than target received power which means the TE NF will lead to amlost 1dB increased MU compared with pure testing data. So 6dB less is acceptable from out point of view. 12dB less will contribute to less testing MU, i.e. 0.266 dB increase but this will lead to larger isolation requirements. Now there is no conclusion of CW unwanted emisison and device unwanted emission requirements, so it’s hard to evaluate the detailed isolaiton between CW and testing antenna. But at least 50dB isolaiton is required referred from RFID spec, i.e. the lower bound of CW and testing antenna isolation is 50dB. Another reason for this 50dB is that it equals to 8.328m distance from CW to testing antenna which is much like the actual deployment environment. If less isolation is required than actual deployment environment then residual CW interference would be large and leading to worse desense.

- Observation 2: 6dB power difference is used between TE and target signal in BS testing.

- Proposal 1: use 6dB as the baseline for power difference between backscatter signal and CW leakage. In addition to, if final isolaiton is less than 50dB, we can use 50dB in stead as used in RFID spec, i.e. 50dB and lower bound for isolation.

- As for the relative positions between testing antenna and CW antenna, the max isolation achieved by spatial isolation is 36.7dB assuming max 1.8m distance which is still less than 50dB isolaiton. So isolation material is necesary to achieve the isolation rather than only using distance isolation. it's noted there is certain position constrain when using isolation material. It’s better not rotate the CW and testing antenna to make sure the isolation doesn’t variate during whole testing. The figure 7.2-1 could be used as the baseline with fixed testing and CW antenna location.

- Proposal 2: it’s suggested to use figure 7.2-1 as the baseline with fixed located testing antenna and CW antenna and 3D-rotated devices. There is no specified relative positions between testing and CW antenna as long as target isolation is achieved by isolation materials.

Figure 7.2-1: Example of a FR1 TRP TRS OTA test system with combined axis

- Another issue that is not discussed before is whether to test the isolation between CW and testing antenna before formal testing. This is necessary to avoid CW impact this could be done after calibration procedure. Therefore we updae the testing procedure as below.

Test procedure for Tx RF requirements

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: set the [rated] transmit power for CW signal. Measure and recode the received power from CW node and calculate the isolation at testing antenna. If the isolation meet target value, then goto step 3, otherwise update isolation material to ensure target isolation.

- Step 3: Place DUT based on UE positioning guidelines.

- Step 4: Device is assumed charging during the measurement according to device declaration on the required energy conditions.

- Step 5: Set the target test frequency and transmit power for signal generator and CW signal. The received power at the antenna of DUT is Pin. Pin is set as the power level of minimum reference sensitivity of device equal to or larger than “sensitivity+[X]dB”

- Step 6: Measure and record the backscattering signal level from DUT and calculate the backscattering power level transmitting from DUT antenna, i.e., Pout, based on the calibration data from step 1

- Step 7: Move to next measurement point and repeat Step 4-6 until complete all the testing point in the measurement grid.

- Step 8: Calculate the ratio of CW power level to D2R signal power (i.e., before and after backscattering) according to the equation: backscattering loss (dB) = Pout (TRP) – Pin (TRP)

- Step 9: FFS on performance metric. FFS on how to specify the performance metric.

Test procedure for receiver RF requirements

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: set the [rated] transmit power for CW signal. Measure and recode the received power from CW node and calculate the isolation at testing antenna. If the isolation meet target value, then goto step 3, otherwise update isolation material to ensure target isolation.

- Step 3: Place DUT based on UE positioning guidelines.

- Step 4: Device is assumed charging during the measurement according to device declaration on the required energy conditions.

- Step 5: Set the target test frequency and transmit power for signal generator and CW signal. The transmit power of signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device. FSS on the CW signal level.

- Step 6: Determine whether DUT can send the correct response in D2R channel within timing window, e.g., TR2D_min

- Step 7: Move to next measurement point and repeat Step 4-6 until complete all the testing point in the measurement grid.

- Step 8: FFS on performance metric. FFS on how to specify the performance metric.

- Proposal 3: add the testing step to ensure target isolation from CW node to testing antenna as below:

- Step 2: set the [rated] transmit power for CW signal. Measure and recode the received power from CW node and calculate the isolation at testing antenna. If the isolation meet target value, then goto step 3, otherwise update isolation material to ensure target isolation.

- In last meeting candidate agreements for MU is list in the table listing which MU could be reused and which requirements needs to be updated. But last meeting agreements is only for TRS not for TRP MU. Following list the candidate requirements for TRP. The main difference between TRP and TRS table is that certain source is for receiver chain for TRP but for transmitter chain for TRS. Besides, there is sensitivity measurement: output level step resolution for TRS which is not needed for TRP.

- Observation 3: last meeting agreement for MU is for TRS not for TRP. Certain sources of MU are different for TRP and TRS.

- Proposal 4: following list the MU requirements for TRP.

| UID | Description of uncertainty contribution | Reuse or not |
| --- | --- | --- |
| Stage 2: DUT measurement (Figure 7.2-1, Figure 7.2-2) |  |  |
| 1 | Mismatch of receiver chain | reused |
| 2 | Insertion loss of receiver chain | reused |
| 3 | Influence of the measurement antenna cable | reused |
| 4 | Measurement Receiver: uncertainty of the absolute level | FFS |
| 5 | Measurement distance | reused |
| 6 | Quality of quiet zone | reused |
| 7 | DUT Tx-power drift | FFS |
| 8 | Uncertainty related to the use of phantoms | No needs |
| 9 | Coarse sampling grid | - For TRP and EIRP, use 30 degree as starting point - For TRS and EIS, use 45 degree as starting point |
| 10 | Random uncertainty | reused |
| 11 | Frequency Response | reused |
| Stage 1: Calibration measurement, network analyzer method (Figure 7.3-1) |  |  |
| 12 | Uncertainty of network analyzer | reused |
| 13 | Mismatch of receiver chain | reused |
| 14 | Insertion loss of receiver chain | reused |
| 15 | Mismatch in the connection of calibration antenna | reused |
| 16 | Influence of the calibration antenna feed cable | reused |
| 17 | Influence of the measurement antenna cable | reused |
| 18 | Uncertainty of the absolute gain/ radiation efficiency of the calibration antenna | reused |
| 19 | Measurement distance | reused |
| 20 | Quality of the Quiet Zone | reused |
| Systematic Errors |  |  |
| 21 | Systematic Error related to TRP grids | reused |

- Another issue is in 38.870, MU of measurement distance source is different for three modes, i.e. hand only (browsing mode), Beside Head and Hand (Talk mode) and Wrist-Worn device measurement. For device, we should make it clear to refer to which modes.

| - Uncertainty value | - hand only (browsing mode) | - Beside Head and Hand (Talk mode) | - Wrist-Worn device |
| --- | --- | --- | --- |
| - Measurement distance | - 0,0 | - 0.27,0.27 | - 0,0 |

- From out view, the measurement distance uncertainty budget should be zero, i.e. the same as hand only could be reused.

- Proposal 5: the MU of measurement distance source is the same as hand only mode.

3	Conclusion

This contribution makes the following observations and proposals.

- Observation 1: testing frequency range is defined uniform as general part in TR 38.870. but in TS 38.521-1, testing frequency range is defined case by case.

- For transmit output power and modulation quality, testing frequency range includes low, middle and high range.

- For occupied bandwidth and max input power, testing frequency range include middle range.

- For emission and REFSENSE requirements, testing frequency range include low and high ranges.

- Proposal 1: list all low, middle and high frequency ranges as general part in TR 38.870 and leave RAN5 to determine detailed testing frequency range case by case for each RF requirements.

- Observation 2: 6dB power difference is used between TE and target signal in BS testing.

- Proposal 2: use 6dB as the baseline for power difference between backscatter signal and CW leakage. In addition to, if final isolaiton is less than 50dB, we can use 50dB in stead as used in RFID spec, i.e. 50dB and lower bound for isolation.

- Proposal 3: it’s suggested to use figure 7.2-1 as the baseline with fixed located testing antenna and CW antenna and 3D-rotated devices. There is no specified relative positions between testing and CW antenna as long as target isolation is achieved by isolation materials.

- Proposal 4: add follow testing step to ensure target isolation from CW node to testing antenna as below:

- Step 2: set the [rated] transmit power for CW signal. Measure and recode the received power from CW node and calculate the isolation at testing antenna. If the isolation meet target value, then goto step 3, otherwise update isolation material to ensure target isolation.

- Observation 3: last meeting agreement for MU is for TRS not for TRP. Certain sources of MU are different for TRP and TRS.

- Proposal 4: following list the MU requirements for TRP.

| UID | Description of uncertainty contribution | Reuse or not |
| --- | --- | --- |
| Stage 2: DUT measurement (Figure 7.2-1, Figure 7.2-2) |  |  |
| 1 | Mismatch of receiver chain | reused |
| 2 | Insertion loss of receiver chain | reused |
| 3 | Influence of the measurement antenna cable | reused |
| 4 | Measurement Receiver: uncertainty of the absolute level | FFS |
| 5 | Measurement distance | reused |
| 6 | Quality of quiet zone | reused |
| 7 | DUT Tx-power drift | FFS |
| 8 | Uncertainty related to the use of phantoms | No needs |
| 9 | Coarse sampling grid | - For TRP and EIRP, use 30 degree as starting point - For TRS and EIS, use 45 degree as starting point |
| 10 | Random uncertainty | reused |
| 11 | Frequency Response | reused |
| Stage 1: Calibration measurement, network analyzer method (Figure 7.3-1) |  |  |
| 12 | Uncertainty of network analyzer | reused |
| 13 | Mismatch of receiver chain | reused |
| 14 | Insertion loss of receiver chain | reused |
| 15 | Mismatch in the connection of calibration antenna | reused |
| 16 | Influence of the calibration antenna feed cable | reused |
| 17 | Influence of the measurement antenna cable | reused |
| 18 | Uncertainty of the absolute gain/ radiation efficiency of the calibration antenna | reused |
| 19 | Measurement distance | reused |
| 20 | Quality of the Quiet Zone | reused |
| Systematic Errors |  |  |
| 21 | Systematic Error related to TRP grids | reused |

- Proposal 5: the MU of measurement distance source is the same as hand only mode.

References

- RP-243312, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR