3GPP TSG-RAN WG4 Meeting #115							         		R4-2505621

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.1.2.1.2 Intra-band contiguous and non-contiguous UL CA with PC1.5 [NR_ENDC_RF_Ph4-Core]

Title: 	PC1.5 non-contiguous ULCA MPR, NS_04 A-MPR and PCmax requirements

Source: 	Skyworks Solutions, Inc.

Document for:	Approval

# 1	Background

In R4#114b the way forward [1] had no conclusions on PC1.5 non-contiguous ULCA requirements. Since we already agreed that only the DualPA architecture will be supported, in this contribution, we make proposals for MPR, NS_04 A-MPR and PCmax.

# Discussion

## 2.1 DualPA PC1.5 UL NCCA architecture back-off vs PC2

In the current specification, MPR is already defined for DualPA PC2 ULNCCA in sections 6.2A.2.2.1.2 and 6.2A.2.2.2.2 for -30dBm/MHz and -13dBm/MHz limits respectively.

Since there is no limitation on reaching PCmax in each CC for PC2 DualPA, it can be implemented in two ways:

- Using two PC2 PAs (one per CC)

- Using one PC2 PA + one PC3 PA and LO switching to enable reaching PCmax on the CC with highest allocated RB bandwidth.

The first architecture is also the agreed one for PC1.5 DualPA case with the restriction that only 26dBm can be reached on each CC (PCmax,C=26dBm).

Observation: From a back-off perspective, the only difference between a PC1.5 and a PC2 DualPA case is the PCmax reference that is 29dBm instead of 26dBm. Thus, if emissions can be met for a PC2 case for a given back-off, it can be met for PC1.5 with the same back-off +3dB (same power per CC in both cases). It is thus sufficient to increase the MPR and A-MPR of PC2 by 3dB to derive the PC1.5 MPR and AMPR.

Proposal: The PC1.5 DualPA MPR and NS_04 A-MPR are specified by adding 3dB to the DualPA PC2 MPR and A-MPR requirements as implemented in companion draft CR [2].

## 2.2 PCmax requirement for DualPA PC1.5 UL NCCA

As already discussed in [3], PC1.5 non-contiguous ULCA using two PC2 PAs in DualPA architecture cannot deliver more than 26dBm part CC. Moreover, when there is a large difference in allocated RB bandwidth in each CC, this limitation amplifies the potential for large PSD differences in each CC.

Observation: The current agreement and consensus in RAN4 on this aspect is the following:

- It is agreed that PCMAX,c  is limited to 26dBm but PCMAX can reach 29dBm when both CCs are at 26dBm.

- There is no consensus that PCMAX equations should be modified to account for equal PSD or a maximum PSD difference of 6dB as was proposed in [3].

- However, there is consensus that the UE is allowed to drop the SCC when the PSD difference is too large and may result in the UE not meeting the emission requirement with the agreed MPR and A-MPR.

- There was a consensus that 6dB PSD difference may be considered in MPR evaluation.

Based on the above observation and to finalize the PC1.5 non-contiguous ULCA feature in time for Rel19, we suggest the following way forward.

Proposal:

- PCmax equation are not modified to account for a PSD difference threshold of 0dB or 6dB.

- The limitation of PCMAX,c  to 26dBm is captured in the specification as implemented in companion draftCR [2].

- Send an LS to RAN5 clarifying that UEs are allowed drop the SCC if the PSD difference between the two CCs exceeds 6dB.

- FFS if above behavior needs to be captured in TS 38.101-1.

Due to the 26dBm per CC limitation for PC1.5, an additional aspect needs to be considered in the PCmax section that has not been thoroughly discussed in RAN4: The fall-back behavior when only one UL CC is with RBs allocated. There are 3 possibilities:

- The UE is RRC reconfigured for single UL: the UE shall meet PC1.5 single CC requirement by using the two PAs with UL MIMO and/or TxD but LO switching interruption is needed.

- The UE is still configured for NC ULCA but has RBs only in one CC (LCRB1 or LCRB0 = 0 in the PCmax section)

  - If no LO switching interruption is allowed, then PCmax will be limited to 26dBm PCmax,c

  - If LO switching interruption is allowed, then the UE can meet PC1.5 single CC requirement by using the two PAs with UL MIMO and/or TxD.

For all these cases, the behavior of the PC1.5 NCCA DualPA architecture behaves differently from the PC3 and PC2 cases where PAs are capable of delivering PCmax per CC and this needs to be reflected in the specification.

Proposal:

- RAN4 agrees on LO switching interruption needed between ULCA and single CC RRC reconfiguration.

- RAN4 decides if LO switching interruption is allowed or not when in ULCA mode and LCRB1 or LCRB2 =0.

  - If not allowed, when LCRB1 or LCRB2 =0: PCmax=PCmax,c=26dBm.

  - If allowed, when LCRB1 or LCRB2 =0: PCmax= 29dBm.

- The current version of the draftCR [2] is based on interruptions not allowed but we are open to revise it according to RAN4 agreements.

# Conclusion

In this contribution, we have discussed the differences between DualPA PC2 and PC1.5 architectures and behavior and made the following proposals.

Proposal: The PC1.5 DualPA MPR and NS_04 A-MPR are specified by adding 3dB to the DualPA PC2 MPR and A-MPR requirements as implemented in companion draft CR [2].

Proposal:

- PCmax equation are not modified to account for a PSD difference threshold of 0dB or 6dB.

- The limitation of PCMAX,c  to 26dBm is captured in the specification as implemented in companion draftCR [2].

- Send an LS to RAN5 clarifying that UEs are allowed drop the SCC if the PSD difference between the two CCs exceeds 6dB.

- FFS if above behavior needs to be captured in TS 38.101-1.

Proposal:

- RAN4 agrees on LO switching interruption needed between ULCA and single CC RRC reconfiguration.

- RAN4 decides if LO switching interruption is allowed or not when in ULCA mode and LCRB1 or LCRB2 =0.

  - If not allowed, when LCRB1 or LCRB2 =0: PCmax=PCmax,c=26dBm.

  - If allowed, when LCRB1 or LCRB2 =0: PCmax= 29dBm.

- The current version of the draftCR [2] is based on interruptions not allowed but we are open to revise it according to RAN4 agreements.

# References

[1] R4-2505100 WF on HPUE RF requirements, Qualcomm, MediaTek, Skyworks, LGE, R4#114b

[2] R4-2505614 DraftCR to 38101-1-j10 on PC1.5 NC ULCA MPR, A-MPR and PCmax, Skyworks Solutions Inc., R4#115

[3] R4-2419739 Inputs to PCmax for PC1.5 intra-band ULCA, Skyworks Solutions Inc., R4#113