3GPP TSG-RAN WG4 Meeting #115							         		R4-2505622

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.1.2.1.2 Intra-band contiguous and non-contiguous UL CA with PC1.5 [NR_ENDC_RF_Ph4-Core]

Title: 	PC1.5 contiguous ULCA MPR for NC allocations and NS_04 A-MPR

Source: 	Skyworks Solutions, Inc.

Document for:	Approval

# 1	Background

In R4#114b the way forward [1] captured the tentative MPR values for PC1.5 contiguous ULCA non-contiguous allocations based on the input of multiple companies including [2]. In this contribution, we confirm the proposal for MPR and provide corrections for the NS04 part.

# Discussion

## 2.1 Way forward input

The way forward [1] captured the tentative agreements on PC1.5 contiguous ULCA MPR in italics below.

## MPR values for intra-band contiguous ULCA with non-contiguous RB allocation (with TxD)

  - The MPR values for PC3(1Tx) and PC2(2Tx) listed in the below table is just for reference

| Modulation |  | PC3 (1Tx) |  |  | PC2 (2Tx) |  |  | PC1.5 (2Tx) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  |  | Handheld |  |  | Handheld |  |  |
|  |  | inner | Outer12 | Outer23 | inner | Outer12 | Outer23 | inner | Outer12 | Outer23 |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 6 | 11.5 | 41 | 8.5 | 14.5 | [4.5-5] | [9-9.5] | [15.5-17] |
|  | QPSK | 2.5 | 6 | 11.5 | 41 | 8.5 | 14.5 | [4.5-5] | [9-9.5] | [15.5-17] |
|  | 16QAM | 3 | 6 | 11.5 | 41 | 8.5 | 14.5 | [4.5-5] | [9-9.5] | [15.5-17] |
|  | 64QAM | 5 | 6 | 11.5 | 6 | 8.5 | 14.5 | [6.5-7] | [9-9.5] | [15.5-17] |
|  | 256QAM | 6.5 | 6.5 | 11.5 | 7.5 | 8.5 | 14.5 | [8-8.5] | [9-9.5] | [15.5-17] |
| CP-OFDM | QPSK | 3.5 | 7 | 12 | 4.51 | 9 | 15.5 | [5-5.5] | [9.5-10] | [16.5-17] |
|  | 16QAM | 3.5 | 7 | 12 | 4.51 | 9 | 15.5 | [5-5.5] | [9.5-10] | [16.5-17] |
|  | 64QAM | 5 | 7 | 12 | 6 | 9 | 15.5 | [6.5-7] | [9.5-10] | [16.5-17] |
|  | 256QAM | 7.5 | 7.5 | 12 | 8.5 | 9 | 15.5 | [9-9.5] | [9.5-10] | [16.5-17] |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |  |  |  |  |  |  |

Agreement: Further discussion in upcoming meetings based on the tentative values in the above table.

The way forward [1] also captured the online agreement on sending an LS to RAN5 on PSD conditions in italics below.

LS to RAN5 on PSD condition RAN4 assumed to derive MPR requirements: Online Agreement:

  - Send LS to RAN5 on the PSD condition (i.e., up to 6dB) RAN4 assumed to derive MPR/A-MPR requirements for PC1.5 intra band contiguous and non-contiguous ULCA, after all MPR/A-MPR values are agreed

    - RAN4 suggests that the test for PC2, PC3 and PC1.5 can be based on equal PSD assumption.

## 2.2 MPR for PC1.5 contiguous ULCA non-contiguous allocations

In [2], we proposed a similar table as in the way forward but used the wrong PC2 reference. This would have offset our proposed values to higher values as our proposal is based on a measured back-off difference between PC2 2Tx and PC1.5 2Tx. With the proper PC2 2Tx reference, our corrected values can be found in Table 1 below which takes into account the ranges from the tentative agreement.

Table 1: Corrected proposal based on [2]

| Modulation |  | PC3 (1Tx) |  |  | PC2 (2Tx) |  |  | PC1.5 (2Tx) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  |  | Handheld |  |  | Handheld |  |  |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 6 | 11.5 | 41 | 8.5 | 14.5 | [5] | [9] | [15.5] |
|  | QPSK | 2.5 | 6 | 11.5 | 41 | 8.5 | 14.5 | [5] | [9] | [15.5] |
|  | 16QAM | 3 | 6 | 11.5 | 41 | 8.5 | 14.5 | [5] | [9] | [15.5] |
|  | 64QAM | 5 | 6 | 11.5 | 6 | 8.5 | 14.5 | [7] | [9] | [15.5] |
|  | 256QAM | 6.5 | 6.5 | 11.5 | 7.5 | 8.5 | 14.5 | [8.5] | [9] | [15.5] |
| CP-OFDM | QPSK | 3.5 | 7 | 12 | 4.51 | 9 | 15.5 | [5.5] | [9.5] | [16.5] |
|  | 16QAM | 3.5 | 7 | 12 | 4.51 | 9 | 15.5 | [5.5] | [9.5] | [16.5] |
|  | 64QAM | 6 | 9 | 15.5 | 6 | 9 | 15.5 | [7] | [9.5] | [16.5] |
|  | 256QAM | 8.5 | 9 | 15.5 | 8.5 | 9 | 15.5 | [9.5] | [9.5] | [16.5] |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. |  |  |  |  |  |  |  |  |  |  |

Observations:

- For QPSK, our proposed and corrected values for inner would have been higher than the tentative agreement range but we are fine as our absolute back-off is still within the tentative agreement range.

- For QPSK, our proposed values for outer1 and outer2 are lower than the tentative agreement values range. Given that we aligned to the lower value of the tentative agreement range and its large value, we considered that measurements at low VCC to account for APT/ET architecture are not needed and that up to 6dB PSD difference is covered.

- For contiguous allocations, the FWA MPR is 0.5dB lower than the handheld one.

Based on the above observations, and to finalize the PC1.5 contiguous ULCA feature in time for Rel19, we suggest the following way forward.

Proposal: The table below is used to specify the PC1.5 handheld and FWA MPR for contiguous ULCA and non-contiguous allocations.

| Modulation |  | Handheld |  |  | FWA3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | PI/2 BPSK | 5 | 9 | 15.5 | 4.5 | 8.5 | 15 |
|  | QPSK | 5 | 9 | 15.5 | 4.5 | 8.5 | 15 |
|  | 16QAM | 5 | 9 | 15.5 | 4.5 | 8.5 | 15 |
|  | 64QAM | 7 | 9 | 15.5 | 6.5 | 8.5 | 15 |
|  | 256QAM | 8.5 | 9 | 15.5 | 8 | 8.5 | 15 |
| CP-OFDM | QPSK | 5.5 | 9.5 | 16.5 | 5 | 9 | 16 |
|  | 16QAM | 5.5 | 9.5 | 16.5 | 5 | 9 | 16 |
|  | 64QAM | 7 | 9.5 | 16.5 | 6.5 | 9 | 16 |
|  | 256QAM | 9.5 | 9.5 | 16.5 | 9 | 9 | 16 |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |  |  |  |

## 2.2 NS04 A-MPR for PC1.5 contiguous ULCA non-contiguous allocations

We aimed to provide proposals for NS04 A-MPR for PC1.5 contiguous ULCA for both contiguous and non-contiguous. However, our current data set taken for the MPR evaluation was too limited in aggregated BW cases, as was the set of allocations to explore the limitations for the NS04 OOB -25dBm/MHz emissions requirement below 2490.5MHz, especially for large contiguous allocations.

While studying the current PC3 and PC2 NS04 requirements, we found that:

- For contiguous allocations, the NS04 A-MPR is related to the contiguous ULCA MPR and single CC NS04 A-MPR, thus for PC1.5 we just need to check that the logic still holds for PC1.5

- For non-contiguous allocations, except for the specific case where IMD3 low falls into the -25dBm/MHz OOB region within the filter BW where A-MPR in Clause 6.2A.3.1.1.1.3 applies, the A-MPR is related to the contiguous ULCA MPR.

  - However, when examining Clause 6.2A.3.1.1.1.3, we found that it is defined for PC3 but not for PC2.

Observation:

- Other than A-MPR values in Clause 6.2A.3.1.1.1.3, the NS04 A-MPR for contiguous ULCA with contiguous or non-contiguous allocations is derived from the related power class and allocation type of contiguous ULCA MPR or the single CC A-MPR.

- A-MPR values in Clause 6.2A.3.1.1.1.3 are only valid for PC3, PC2 values are missing.

Based on the above observation, before we provide inputs for PC1.5 in Clause 6.2A.3.1.1.1.3 we should first include the PC2 values.

The values for PC3 -25dBm/MHz NS04 in Clause 6.2A.3.1.1.1.3 (contiguous CA with non-contiguous allocations) can be compared with the PC3 -25dBm/MHz NS04 in Clause 6.2A.3.1.2.1.1 (non-contiguous ULCA) as the worst case IMDs for contiguous CA non-contiguous allocations is the same as for non-contiguous allocations ULCA. The only difference is that PC3 n41B/C uses one PC3 PA while n41(2A) uses two PC3 PAs. Thus, IMD products are lower for n41(2A) since it is reverse IMD with two PAs with 3dB intrinsic back-off compared to forward IMD of one PA with no intrinsic backoff.

The two PC3 A-MPR can be compared in Table 2.

Table 2: PC3 AMPRIM3 to meet -25dBm/MHz

| Clause 6.2A.3.1.1.1.3 NS04 Contiguous ULCA with NC allocations |  | Clause 6.2A.3.1.2.1.1 NS04 NCULCA |  | NC to C Delta [dB] |
| --- | --- | --- | --- | --- |
| MA[dB] | Allocation BW[MHz] | MA[dB] | Allocation BW[MHz] |  |
| 13 | 0 ≤ B < 2.16 | 12 | 0 ≤ B < 2.16 | 1 |
| 11.5 | 2.16 ≤ B < 3.24 | 11 | 2.16 ≤ B < 3.24 | 0.5 |
| 10.5 | 3.24 ≤ B < 5.04 | 10 | 3.24 ≤ B < 5.04 | 0.5 |
| 9.5 | 5.04 ≤ B < 10.08 | 9 | 5.04 ≤ B < 10.08 | 0.5 |
| 8 | 10.08 ≤ B < 16.56 | 8 | 10.08 ≤ B < 16.38 | 0 |
| 7 | 16.56 ≤ B < 21.96 | 7 | 16.38 ≤ B < 21.78 | 0 |
| 6 | 21.96 ≤ B | 6 | 21.78 ≤ B | 0 |

Observations:

- The different allocation bandwidth cases are very similar for both clauses and the same up to 10.08MHz

- The difference is 1dB for the smallest allocation bandwidth then 0.5dB up to 10.08MHz allocation bandwidth.

Since Clause 6.2A.3.1.2.1.1 also has values for PC2 based on two PC2 PAs, one could derive the PC2 1Tx NS04 A-MPR by applying the same deltas and the same allocation bandwidth ranges. Furthermore, since n41C PC2 is also feasible using two PC3 PAs and UL MIMO and/or TxD, one can further apply the difference between the PC2 1Tx and PC2 2Tx contiguous ULCA MPR for non-contiguous outer 2 allocations which is 1dB. However, we keep the principle that PC2 A-MPR cannot exceed PC3 A-MPR+3dB which would be the case for small allocations.

These calculations are reflected step-by-step in Table 3. An asterisk to the 1dB delta in the fifth column denotes a case where the delta is clamped to avoid PC2 A-MPR to exceed PC3 A-MPR+3dB.

Table 3: PC2 AMPRIM3 to meet -25dBm/MHz

| Clause 6.2A.3.1.2.1.1 NS04 NCULCA |  | 2PA NC to 1PA C Delta [dB] | 1Tx Clause 6.2A.3.1.1.1.3 NS04 Contiguous ULCA with NC allocations |  | 1Tx to 2Tx Delta [dB] | 2Tx Clause 6.2A.3.1.1.1.3 NS04 Contiguous ULCA with NC allocations |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MA[dB] | Allocation BW[MHz] |  | MA[dB] | Allocation BW[MHz] |  | MA[dB] | Allocation BW[MHz] |
| 14.5 | 0 ≤ B < 1.44 | 1 | 15.5 | 0 ≤ B < 1.44 | 1* | 16 | 0 ≤ B < 1.44 |
| 14 | 1.44 ≤ B < 2.88 | 0.5 | 14.5 | 1.44 ≤ B < 2.88 | 1* | 15 | 1.44 ≤ B < 2.88 |
| 13 | 2.88 ≤ B < 5.76 | 0.5 | 13.5 | 2.88 ≤ B < 5.76 | 1* | 14 | 2.88 ≤ B < 5.76 |
| 11 | 5.76 ≤ B < 10.8 | 0.5 | 11.5 | 5.76 ≤ B < 10.8 | 1 | 12.5 | 5.76 ≤ B < 10.8 |
| 9.5 | 10.8 ≤ B < 23.04 | 0 | 9.5 | 10.8 ≤ B < 23.04 | 1 | 10.5 | 10.8 ≤ B < 23.04 |
| 9 | 23.04 ≤ B | 0 | 9 | 23.04 ≤ B | 1* | 9 | 23.04 ≤ B |

For contiguous ULCA NS04, we do not see the benefit of specifying both 1Tx and 2Tx PC2 A-MPR for non-contiguous allocation as this applies to a small number of corner cases. Thus, we suggest applying the 2Tx PC2 A-MPR for 1Tx PC2.

Proposal: Clause 6.2A.3.1.1.1	A-MPR for CA_NS_04 is updated as follows:

- For Clause 6.2A.3.1.1.1.3 the red underline text below is added:

6.2A.3.1.1.1.3	AMPRIM3 to meet -25dBm/MHz for PC3

AMPR in this clause is for PC3 intra-band contiguous CA_n41B and CA_n41C. The allowed maximum output power reduction is defined as:

AMPRIM3=MA, Where MA is defined as follows

MA = 	13; 	0 ≤ B < 2.16

11.5; 	2.16 ≤ B < 3.24

10.5;       3.24 ≤ B < 5.04

9.5; 	5.04 ≤ B < 10.08

8; 	10.08 ≤ B < 16.56

7;    16.56 ≤ B < 21.96

6; 	      21.96 ≤ B

Where:

B=(LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000 (MHz), where SCS1 and SCS2 are expressed in kHz.

and LCRB1, SCS1 are for CC1, LCRB2, SCS2 are for CC2, CC1 is the component carrier with lower frequency.

- Clause 6.2A.3.1.1.1.4 is added as follows:

6.2A.3.1.1.1.4	AMPRIM3 to meet -25dBm/MHz for PC2 1Tx and 2Tx

AMPR in this clause is for PC2 1Tx and 2Tx intra-band contiguous CA_n41B and CA_n41C. The allowed maximum output power reduction is defined as:

AMPRIM3=MA, Where MA is defined as follows

MA = [16]; 	0 ≤ B < 1.44

[15]; 	1.44 ≤ B < 2.88

[14];       2.88 ≤ B < 5.76

[12.5]; 	5.76 ≤ B < 10.8

[10.5]; 	10.8 ≤ B < 23.04

[9];    23.04 ≤ B

Where:

B=(LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000 (MHz), where SCS1 and SCS2 are expressed in kHz.

and LCRB1, SCS1 are for CC1, LCRB2, SCS2 are for CC2, CC1 is the component carrier with lower frequency.

- If agreeable, a CRs with the additional Clause 6.2A.3.1.1.1.4 will be presented for Rel17/18/19 in the August meeting

- NS04 A-MPR values in the new Clause 6.2A.3.1.1.1.4 are further checked during PC1.5 evaluation and are used as the baseline for the difference between PC2 2Tx and PC1.5 2Tx cases.

- PC1.5 NS04 A-MPR is specified without distinction between handheld and FWA UE and uses handheld values.

# Conclusion

In this contribution, we have discussed the MPR and A-MPR for PC1.5 contiguous ULCA and made the following proposals.

Proposal: The table below is used to specify the PC1.5 handheld and FWA MPR for contiguous ULCA and non-contiguous allocations.

| Modulation |  | Handheld |  |  | FWA3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | PI/2 BPSK | 5 | 9 | 15.5 | 4.5 | 8.5 | 15 |
|  | QPSK | 5 | 9 | 15.5 | 4.5 | 8.5 | 15 |
|  | 16QAM | 5 | 9 | 15.5 | 4.5 | 8.5 | 15 |
|  | 64QAM | 7 | 9 | 15.5 | 6.5 | 8.5 | 15 |
|  | 256QAM | 8.5 | 9 | 15.5 | 8 | 8.5 | 15 |
| CP-OFDM | QPSK | 5.5 | 9.5 | 16.5 | 5 | 9 | 16 |
|  | 16QAM | 5.5 | 9.5 | 16.5 | 5 | 9 | 16 |
|  | 64QAM | 7 | 9.5 | 16.5 | 6.5 | 9 | 16 |
|  | 256QAM | 9.5 | 9.5 | 16.5 | 9 | 9 | 16 |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |  |  |  |

Additionally, we propose to correct 38.101-1 by adding the missing PC2 NS04 A-MPR for contiguous ULCA with non-contiguous allocations.

Proposal: Clause 6.2A.3.1.1.1	A-MPR for CA_NS_04 is updated as follows:

- For Clause 6.2A.3.1.1.1.3 the red underline text below is added:

6.2A.3.1.1.1.3	AMPRIM3 to meet -25dBm/MHz for PC3

AMPR in this clause is for PC3 intra-band contiguous CA_n41B and CA_n41C. The allowed maximum output power reduction is defined as:

AMPRIM3=MA, Where MA is defined as follows

MA = 	13; 	0 ≤ B < 2.16

11.5; 	2.16 ≤ B < 3.24

10.5;       3.24 ≤ B < 5.04

9.5; 	5.04 ≤ B < 10.08

8; 	10.08 ≤ B < 16.56

7;    16.56 ≤ B < 21.96

6; 	      21.96 ≤ B

Where:

B=(LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000 (MHz), where SCS1 and SCS2 are expressed in kHz.

and LCRB1, SCS1 are for CC1, LCRB2, SCS2 are for CC2, CC1 is the component carrier with lower frequency.

- Clause 6.2A.3.1.1.1.4 is added as follows:

6.2A.3.1.1.1.4	AMPRIM3 to meet -25dBm/MHz for 1Tx and 2Tx PC2

AMPR in this clause is for 1Tx and 2Tx PC2 intra-band contiguous CA_n41B and CA_n41C. The allowed maximum output power reduction is defined as:

AMPRIM3=MA, Where MA is defined as follows

MA = [16]; 	0 ≤ B < 1.44

[15]; 	1.44 ≤ B < 2.88

[14];       2.88 ≤ B < 5.76

[12.5]; 	5.76 ≤ B < 10.8

[10.5]; 	10.8 ≤ B < 23.04

[9];    23.04 ≤ B

Where:

B=(LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000 (MHz), where SCS1 and SCS2 are expressed in kHz.

and LCRB1, SCS1 are for CC1, LCRB2, SCS2 are for CC2, CC1 is the component carrier with lower frequency.

- If agreeable, a CRs with the additional Clause 6.2A.3.1.1.1.4 will be presented for Rel17/18/19 in the August meeting

- NS04 A-MPR values in the new Clause 6.2A.3.1.1.1.4 are further checked during PC1.5 evaluation and are used as the baseline for the difference between PC2 2Tx and PC1.5 2Tx cases.

- PC1.5 NS04 A-MPR is specified without distinction between handheld and FWA UE and uses handheld values.

# References

[1] R4-2505100 WF on HPUE RF requirements, Qualcomm, MediaTek, Skyworks, LGE, R4#114b

[2] R4-2504386 Results for PC1.5 contiguous ULCA with comparison to PC2 and 6dB PSD imbalance, Skyworks Solutions Inc., R4#114b