3GPP TSG-RAN4 #115	R4-2505650

Malta Island, Malta, 19th May 2025 – 23rd May 2025

Title:	Discussion on RB allocation in test cases for reference sensitivity for CA

Source:	Anritsu Limited

Agenda Item:	4.2.2

Document for:	Discussion

# Introduction

#### 4.5.2.2	DFT-s-OFDM waveform

The following RB allocation is the closest number lower or equal to CP-OFDM maximum RB allocation satisfying the following equation, partial RB allocations shall also conform to this equation:

number of RB=2^X*3^Y*5^Z

Table 4.5.2.2-1: Range 1 NR UE maximum RB allocation for DFT-s-OFDM

| SCS [kHz] | Channel bandwidths [MHz] |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 |
| 15 | 25 | 50 | 75 | 100 | 128 | [160] | 216 | 270 | N.A | N.A | N.A |
| 30 | 10 | 24 | 36 | 50 | 64 | [75] | 100 | 128 | 162 | 216 | 270 |
| 60 | N.A | 10 | 18 | 24 | 30 | [36] | 50 | 64 | 75 | 100 | 135 |

RB numbering for DFT-s-OFDM waveforms is the same than the RB number for CP-OFDM it is aligned with. In the case where DFT-s-OFDM maximum RB allocation is smaller than the CP-OFDM maximum allocation, all positions within the CP-OFDM allocation are valid. This implies that RBstart values can be higher than the maximum allocation for DFT-s-OFDM. The valid RB start values follow the following equation:

RBstart DFT-s-OFDM range = 0 to (CP-OFDM maxRB) – (DFT-s-OFDM #RB)

Table 4.5.2.2-2: Range 1 NR UE transmission bandwidths in MHz for DFT-s-OFDM

| SCS [kHz] |  |  | Channel bandwidths [MHz] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 |  | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 |
| 15 | 4.5 | 9 |  | 13.5 | 18 | 23.04 | [28.80] | 38.88 | 48.6 | N.A | N.A | N.A |
| 30 | 3.6 | 8.64 |  | 12.96 | 18 | 23.04 | [27.00] | 36 | 46.08 | 58.32 | 77.76 | 97.2 |
| 60 | N.A | 7.2 |  | 12.96 | 17.28 | 21.6 | [25.92] | 36 | 46.08 | 54 | 72 | 97.2 |

For DFT-s-OFDM, when the maximum allocation is smaller than for CP-OFDM the transmit bandwidth is smaller, as a consequence the UE spectrum utilization can be smaller for DFT-s-OFDM than for CP-OFDM and in most cases equivalent to LTE.

[…]

The Lcrb values given in “Table 7.3A.1-1: Equivalent substitution relationship between different SCS UL test configuration” of TS 38.101-1 [1] and “Table 7.3B.1-1: Equivalent substitution relationship between for NR bands different SCS UL test configuration” of TS 38.101-3 [2] are based on the values given for DFT-s-OFDM in TR 38.817-01 [3] shown below.

There are two issues related to that:

- It is mentioned neither in section 7.3A.1 [1] nor in section 7.3B.1 [2] that the values are from DFT-s-OFDM. It may trouble some readers wanting to understand how the RB values in the corresponding tables have been derived.

- The reader may expect that the values are for CP-OFDM and not DFT-s-OFDM. But it seems that it has been chosen to use the Lcrb values for the DFT-s-OFDM waveform likely because RAN5 specifies the sensitivity tests to be performed using DFT-s-OFDM for the UL.

# Discussion

### 5.3.2	Maximum transmission bandwidth configuration

The maximum transmission bandwidth configuration NRB for each UE channel bandwidth and subcarrier spacing is specified in Table 5.3.2-1.

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

The TS 38.101-1 and 38.101-3 should not really point out to the DFT-s-OFDM values (without clearly mentioning it) as “Table 5.3.2-1: Maximum transmission bandwidth configuration NRB” [1] refers to the one for CP-OFDM (without here also mentioning that the values are for CP-OFDM, but CP-OFDM is generally considered as the default case) as show below.

Table 7.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range, Mid range, High range (NOTE 4) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest Lowest UL / Lowest DL, Lowest UL / Highest DL (NOTE 3) |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB allocation | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: Additional test points selected according to asymmetric channel bandwidths specified in clause 5.3.6. DL channel bandwidth shall be selected first. NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. NOTE 5: In a band where UE supports 4Rx but not supports 8Rx, the test needs to be repeated with only 2Rx antennas connected and the other antennas terminated. In a band where UE supports 8Rx, the test needs to be repeated with only 4Rx antennas connected and the other antennas terminated, and the test needs to be repeated with only 2Rx antennas connected and the other antennas terminated. |  |  |  |  |

DFT-s-OFDM values are used by RAN4 in Table 7.3A.1-1 [1] and Table 7.3B.1-1 [2], probably because the RAN4 delegate who created those tables took into account that RAN5 specifies the sensitivity tests with only the DFT-s-OFDM waveform for UL in TS 38.521-1 [4] and TS 38.521-3 [5] as shown below for single carrier sensitivity test.

The following observations can be made:

Observation 1: Lcrb values for the DFT-s-OFDM waveform (not the CP-OFDM waveform) are used by RAN4 in Table 7.3A.1-1 of TS 38.101-1 and Table 7.3B.1-1 of TS 38.101-3, probably because it is known that RAN5 uses that waveform for the UL in the corresponding test configuration tables in TS 38.521-1 and TS 38.521-3. It may be questionable if RAN4 should consider what RAN5 does for the test configuration tables for specifying its own tables in TS 38.101-1 or TS 38.101-3.

Observation 2: It is not written in the body text of TS 38.101-1 and TS 38.101-3 that the Lcrb values for the UL for reference sensitivity for CA currently specified in respectively in Table 7.3A.1-1 and Table 7.3B.1-1 are the ones corresponding to the DFT-s-OFDM waveform.

Based on them, we would like to propose:

Proposal 1: It should be clearly mentioned in the body text (including a reference to TR 38.817-01 to show how Lcrb values are calculated) that the UL Lcrb values currently specified in Table 7.3A.1-1 of TS 38.101-1 and Table 7.3B.1-1 of TS 38.101-3 are for DFT-s-OFDM (waveform used by RAN5 for sensitivity test).

Proposal 2: It should be considered if two tables should be present, one for CP-OFDM and one for DFT-s-OFDM. RAN4 would then not show awareness of which waveform for the UL, RAN5 has actually decided for the sensitivity test.

# Conclusion

Based on the discussion in section 2, the following can be noted, and the following proposal is proposed:

Observation 1: Lcrb values for the DFT-s-OFDM waveform (not the CP-OFDM waveform) are used by RAN4 in Table 7.3A.1-1 of TS 38.101-1 and Table 7.3B.1-1 of TS 38.101-3, probably because it is known that RAN5 uses that waveform for the UL in the corresponding test configuration tables in TS 38.521-1 and TS 38.521-3. It may be questionable if RAN4 should consider what RAN5 does for the test configuration tables for specifying its own tables in TS 38.101-1 or TS 38.101-3.

Observation 2: It is not written in the body text of TS 38.101-1 and TS 38.101-3 that the Lcrb values for the UL for reference sensitivity for CA currently specified in respectively in Table 7.3A.1-1 and Table 7.3B.1-1 are the ones corresponding to the DFT-s-OFDM waveform.

Proposal 1: It should be clearly mentioned in the body text (including a reference to TR 38.817-01 to show how Lcrb values are calculated) that the UL Lcrb values currently specified in Table 7.3A.1-1 of TS 38.101-1 and Table 7.3B.1-1 of TS 38.101-3 are for DFT-s-OFDM (waveform used by RAN5 for sensitivity test).

Proposal 2: It should be considered if two tables should be present, one for CP-OFDM and one for DFT-s-OFDM. RAN4 would then not show awareness of which waveform for the UL, RAN5 has actually decided for the sensitivity test.

# References

[1] TS 38-101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, version 19.1.0

[2] TS 38-101-1, “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”, version 19.1.0

[3] TR 38.817-01, “General aspects for User Equipment (UE) Radio Frequency (RF) for NR”, version 16.4.0

[4] TS 38-521-1, “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 standalone”, version 18.6.0

[5] TS 38-521-3, “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”, version 18.6.0