3GPP TSG-RAN WG4 Meeting #115	R4-2505665

Malta, 19 May – 23 May, 2025

Source: 	vivo

Title: 	Remaining issues for simulation for LP-WUR

Agenda Item:	7.28.5.1

Document for:	Discussion

Introduction

In RAN Plenary #102 meeting, WI for “Low-power wake-up signal and receiver for NR” was agreed [1]. The WI was further updated at RAN Plenary #103 meeting as [2]. The initial discussion on simulation assumptions was triggered at RAN4 111 meeting and the simulation assumptions and results were further discussed at following RAN4 meetings. In this contribution we provide our further consideration on simulation assumptions and also provide further simulation results from our side.

Discussion

The objectives of WI are duplicated as below:

The objectives of the work item are the following:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

  - Specify necessary RRM requirements

The WF and contribution summary for this WI at RAN4 113 meeting are [3] and [4]. In this contribution, we provide our considerations on LP-WUR simulation assumptions and initial simulation results.

On simulation assumptions and parameters

Based on [3], the following issues are further discussed.

Issue 2-1-4: On LP-SS sequence for simulation purpose

At RAN4 114 meeting, the following LP-SS sequence for simulation alignment is agreed.

Agreement:

For RAN4 simulation alignment purpose:

For M=1, L=4 (if supported), the set of LP-SS binary sequences is:

Serving cell:    [0 1 0 1]

Interference cell: [0 1 1 0]

For M=1, L=6 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 1 0 1 0]

Interference cell: [0 1 0 1 0 1]

For M=1, L=8 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 1 0 0 1 0 1]

Interference cell: [1 0 1 0 1 0 0 1]

For M=2, L=8 (if supported), the set of LP-SS sequence is:

Serving cell:    [0 1 0 1 1 0 0 1]

Interference cell: [0 1 1 0 0 1 0 1]

For M=2, L=12 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 0 1 1 0 0 1 1 0 0 1]

Interference cell: [0 1 1 0 1 0 0 1 1 0 0 1]

For M=2, L=16 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1]

Interference cell: [1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1]

For M=4, L=16 (if supported), the set of LP-SS sequence is down-selected between:

Serving cell:    [0110100110101010]

Interference cell: [0110101010011010]

For M=4, L=32(if supported), the set of LP-SS sequence is down-selected between:

Serving cell:    [0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

Interference cell: [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1]

At RAN1 120bis meeting, on the LP-SS sequence, the following agreement is available:

Agreement

For the length L of LP-SS binary sequence, supports

- M=1, L= {6,8}

- M=2, L= {12,16}

- M=4, L= {16,32}

Note: For each value of M, UE supports both L values – no UE capability.

Note: There is no consensus in RAN1 that SNR of -3dB can be fulfilled with the smaller value of L for each M. Definition of the proper requirements for different values of L is up to RAN4.

Based on RAN1 agreement, the sequence for simulation alignment can be the following 6 cases:

For M=1, L=6 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 1 0 1 0]

Interference cell: [0 1 0 1 0 1]

For M=1, L=8 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 1 0 0 1 0 1]

Interference cell: [1 0 1 0 1 0 0 1]

For M=2, L=12 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 0 1 1 0 0 1 1 0 0 1]

Interference cell: [0 1 1 0 1 0 0 1 1 0 0 1]

For M=2, L=16 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1]

Interference cell: [1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1]

For M=4, L=16 (if supported), the set of LP-SS sequence is down-selected between:

Serving cell:    [0110100110101010]

Interference cell: [0110101010011010]

For M=4, L=32(if supported), the set of LP-SS sequence is down-selected between:

Serving cell:    [0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

Interference cell: [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1]

Proposal 1: Suggest the following LP-SS sequence cases are used for final simulation alignment purpose.

For M=1, L=6 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 1 0 1 0]

Interference cell: [0 1 0 1 0 1]

For M=1, L=8 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 1 0 0 1 0 1]

Interference cell: [1 0 1 0 1 0 0 1]

For M=2, L=12 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 0 1 1 0 0 1 1 0 0 1]

Interference cell: [0 1 1 0 1 0 0 1 1 0 0 1]

For M=2, L=16 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1]

Interference cell: [1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1]

For M=4, L=16 (if supported), the set of LP-SS sequence is down-selected between:

Serving cell:    [0110100110101010]

Interference cell: [0110101010011010]

For M=4, L=32(if supported), the set of LP-SS sequence is down-selected between:

Serving cell:    [0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

Interference cell: [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1]

Issue 2-1-5: On ideal RSRP/RSRQ in simulation

- Proposals

  - P1: Ideal RSRP is derived from the serving cell signal after the fading channel; Ideal RSRQ is derived based on ideal RSRP divides the calculated RSSI (Ericsson)

  - P2: Suggest to discuss and align the ideal RSRP calculation (vivo)

  - Method 1: The first one is the RSRP is pass the fading channel and with the perfect channel estimation and the real RSRP pass the same channel.

  - Method 2: The second one assumes the impact of the fading channel can be completely removed hence the ideal RSRP does not need pass the channel whereas the real RSRP will pass the fading channel.

For this issue, the following agreements were achieved at previous meeting.

Agreement:

- Use on method 1 in P2 as a base further check the difference between method 1 and method 2 in P2 and check the channel estimation assumption in ideal RSRP for OOK based LR

Regarding ideal RSSI, if ideal RSRP is based on the method that the RSRP signal is passed by the fading channel with the assumption of perfect channel estimation, it is reasonable that the ideal RSSI is calculated by using the same method, i.e., signals and interference passing the fading channel.

Proposal 2:  The ideal RSSI is calculated by passing signals and interference through corresponding fading channel.

Issue 2-1-7: On OOK interference

- Proposals

  - P1: LP-SS measurement requirements are not applicable to cases where serving cell and neighbour cell are using opposite binary sequences. (Huawei)

    - Define requirements for all combinations, and the requirements are based those with opposite sequence (worst case)

    - Define the requirements without considering combinations with opposite sequence, and the requirements are not applicable for those with opposite sequence

  - P2: Discuss whether using OOK interferer is realistic or not, and if time offset should be applied (Nokia)

For this issue, based on our simulation results, some performance degradation is found for the case {M, L}={1,6} when fully opposite sequence is used for serving and neighbour cell, however the degradation level is limited. In addition this is the only case among all LP-SS sequences for all combination of {M, L} hence the impact of this case is further limited and no need to have any particular consideration for this case.

Proposal 3: For the case {M, L}={1,6} when fully opposite sequence is used for serving and neighbour cell, no need to have any particular consideration for this case.

Issue 2-1-8: On overlaid OFDM sequences in LP-SS

- Proposals

  - P1: For LP-SS overlaid sequence configuration, use L=127 ZC sequences with different root indexes: q=1 for the serving cell, and q=63 for the interfering cell. (Apple)

  - P2: No necessity to align simulation parameter for the overlaid OFDM sequence for LP-SS (vivo)

Regarding overlaid OFDM sequences in LP-SS, the following RAN1 agreements are available:

RAN1 Agreement

Support overlaid OFDM sequence(s) for LP-SS:

- LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS.

- Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable.

  - Applicable at least for OOK-1 and FFS for OOK-4

- From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

At RAN1 120bis meeting the following agreements are for roots for ZC sequence:

- Roots and CS(s) of ZC sequence(s) to generate overlaid OFDM sequence(s):

- Roots: gNB can configure up to 2 roots from any value from 1 ~ (Bzc -1), where Bzc is given by the largest prime number such that BZC<LZC, where  LZC is the overlaid OFDM sequence length. Different roots can be picked for different M values.

To our understanding, different overlaid OFDM sequences configuration has little impact on the performance of the OOK LR based on LP-SS. For the OFDM LR based only on the LP-SS, the same requirement for LP-SS OOK based LR will be reused and there will not be separate simulations for LP-SS overlaid on OFDM sequences.

Proposal 4: No need to align simulation parameter for the overlaid OFDM sequence for LP-SS since there is no separate simulation for LP-SS overlaid OFDM.

Conclusion

In this contribution, we provide our considerations on the LP-WUS/WUR WI and have the following observations and proposals.

Proposal 1: Suggest the following LP-SS sequence cases are used for final simulation alignment purpose.

For M=1, L=6 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 1 0 1 0]

Interference cell: [0 1 0 1 0 1]

For M=1, L=8 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 1 0 0 1 0 1]

Interference cell: [1 0 1 0 1 0 0 1]

For M=2, L=12 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 0 1 1 0 0 1 1 0 0 1]

Interference cell: [0 1 1 0 1 0 0 1 1 0 0 1]

For M=2, L=16 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1]

Interference cell: [1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1]

For M=4, L=16 (if supported), the set of LP-SS sequence is down-selected between:

Serving cell:    [0110100110101010]

Interference cell: [0110101010011010]

For M=4, L=32(if supported), the set of LP-SS sequence is down-selected between:

Serving cell:    [0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

Interference cell: [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1]

Proposal 2:  The ideal RSSI is calculated by passing signals and interference through corresponding fading channel.

Proposal 3: For the case {M, L}={1,6} when fully opposite sequence is used for serving and neighbour cell, no need to have any particular consideration for this case.

Proposal 4: No need to align simulation parameter for the overlaid OFDM sequence for LP-SS since there is no separate simulation for LP-SS overlaid OFDM.

- References

# [1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 102

# [2] RP-240801, Revised WID: Low-power wake-up signal and receiver for NR ((LP-WUS/WUR), RAN 103

# [3] R4-2503627, Topic summary for [114][225] NR_LPWUS, RAN4 114bis, vivo

# [4] R4-2504908, WF on RRM requirements for NR_LPWUS, RAN4 114bis, vivo

# Appendix: Simulation parameters

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | Agreement: 2.6 GHz initially |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| DRX | No applicable for LP-WUR |
| BS transmit antennas for LP-SS blocks | 1 Tx |
| UE receive antennas | 1 Rx |
| Data and control channel subcarrier spacing | [Data, SSB and LP-SS have the same SCS] |
| Subcarrier spacing | 30KHz [15 KHz up to company report] |
| Measurement period (in number of measurement samples) | [4, 5, other number could be studied upon a need] |
| LP-SS/SSB measurement interval | LP-SS: 320 ms SSB: 320 ms |
| LP-SS BW | 132 subcarriers for SCS=30kHz for LP-SS initially TBD for 15KHz SCS |
| SSS | 30KHz for SSS, TBD for 15KHz |
| Actual LP-SS transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| LP-SS |  | Refer to conclusion of the Issue 2-1-4 in [R4-2502600] | Refer to conclusion of the Issue 2-1-4 in [R4-2502600] |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OFDM based receiver [5] ppm OOK based receiver [5 10] ppm | N/A |
| Timing error |  | Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: - T= 5us for OOK-1 - T= 2us for OOK-4 with M=2 - T= 1us for OOK-4 with M=4 |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | SNR setting for serving and interference cell are derived based on agreement of Issue 2-1-1-1 |  |
|  |  | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-2.7] - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4] | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-11.7] - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4] |
|  |  | When Ês/Iot = -0.5 dB (low priority case) Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = -0.5 dB (low priority case) Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
|  |  | When Ês/Iot = 2 dB (low priority case) Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = 2 dB (low priority case) Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
|  |  | When Ês/Iot = -6 dB (low priority case) | When Ês/Iot = -6 dB (low priority case) |
| Ês/Iot | dB | -3; -0.5dB; 2dB | N/A |
| Propagation conditions | - | FR1: AWGN TDL-C 300ns |  |
| UE speed |  | 3 km/h |  |

Table 3: UE-specific parameters

| [Receiver Filter] | [3th/5th Order Butterworth with 3.96MHz bandwidth] |
| --- | --- |
| [Receiver ADC bit width] | [4 or 8-bitADC] |
| [Receiver Sampling Rate for LP-SS only] | [3.84 or 7.68MHz] |