3GPP TSG-RAN WG4 Meeting #115	R4-2505667

Malta, 19 May – 23 May, 2025

Source: 	vivo

Title: 	On remaining RRM issues for LP-WUR

Agenda Item:	7.28.5.2

Document for:	Discussion

Introduction

In RAN Plenary #102 meeting, WI for “Low-power wake-up signal and receiver for NR” was agreed [1]. The WI was further updated at RAN Plenary #103 meeting as [2] and RAN #105 [3].

The objectives of WI are duplicated as below:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

  - Specify necessary RRM requirements

The initial discussion for this topic was initially discussed at RAN4 110bis meeting. The WF on this WI of RAN4 114bis meeting and corresponding agreements can be found at [4]. The topic summary of RAN4 114bis can be found at [5].

Discussion

General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

- Proposals

Regarding case 2:

P1-1: Case#2 is a possible case, its existence depends on network configuration. No separate discussion is needed for Case#2 (CMCC);

P1-2: Prefer to consider (vivo)

P2: De-prioritize (xiaomi);

P3: Not consider (oppo ZTE Huawei Ericsson MTK);

P3-1: No consider case 2, RAN4 can revisit case 2 when further RAN1/2 conclusion conflicting with RAN4 assumptions. (CATT Apple)

P4: Up to RAN2’s decision (LG vivo Nokia)

The case to be considered for LP-WUR has been discussed, case 1 and case 3 have already been agreed.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

If a new threshold for case 1 is introduced, case 2 is a natural extension of case 3 when neighbour cell measurement is not needed, as mentioned before. Compared with case 1, case 2 create a different trade-off point between power consumption and mobility performance, since the measurement of MR on the serving cell may also help mobility performance. Hence, for the RRM relaxation for MR and serving cell RRM measurement offloaded to LP-WUR, besides case 1 and case 3, it is preferred to consider case 2. However there is no consensus for this issue for a few meetings and this issue is also discussed within RAN2.

- Case 2 is preferred. The discussion can be postponed until more RAN2’s progress.

Issue 1-1-2: Threshold for switch between different cases

- Proposals

  - P1: Up to RAN2 decision (CMCC LG CT Ericsson, Nokia)

  - P2: New LP-based threshold(s) based on LP-RSRP or LP-RSRQ used for switch into case 1 from other cases or exit from case 1 to other cases are needed (vivo Apple ZTE)

    - P2-1: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1). LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin). If RAN4 cannot make conclusion on issue 1-1-2, RAN4 sending LS to RAN2 for guidance (Apple)

    - P2-2: Details on threshold are up to RAN2 (vivo)

    - P2-3: Add an offset for existing threshold which is compared to MR based measurement for LP-RSRP and LP-RSRQ. (ZTE)

  - P3: To avoid the ping-pong, the LR and MR measurement result should be comparable or equivalent with certain offset/margin and additionally UEs are allowed to adjust the configured LR threshold within specific range; NW should configure independently the LR threshold according to LR types (OOK based LR and OFDM based LR) (LG)

  - P4: RAN4 to study switching between Case#3 and Case#1 based on a timer. Since UE need to be confident enough to enter Case#1 before deactivating MR, it is better for the UE to enter Case#3 first for a period of time (confidence period) before UE can enter Case#1. (MTK)

- Discussion on threshold should be up to RAN2.

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

- Proposals

  - P1: No LP-WUR measurement and evaluation requirement apply when LR is “on” at the legacy state; (vivo ZTE Huawei Ericsson Xiaomi)

Agreement at RAN4 114:

- At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation.

- The entry/exit conditions between different cases are summarized in the following table 1:

Table 1 Summary on entry/exit conditions for LP-WUS monitoring, case 1 and case 3

| Case | Entry conditions | Exit conditions |
| --- | --- | --- |
| LP-WUS monitoring | MR threshold and optional LR threshold | LR threshold |
| RRM measurement fully offloading (Case 1) | MR threshold and optional LR threshold | LR threshold |
| RRM measurement relaxation (Case 3) | For serving cell RRM relaxation, the entry condition can include both MR and LR measurements FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation | FFS |

- Since the entry conditions may include LP-based threshold, the LP-WUR should be ON to evaluate the LP-based threshold. In practice if the UE enters MR RRM relaxation or fully offloading cases early than enter into LP-WUS monitoring, there is no issue since the LP-WUR is already ON.

- However, one issue is when before starting LP-WUS monitoring, the UE is in the legacy scenario performing legacy serving cell or serving and neighbour cell measurement (either legacy Rel-15 scenario or legacy Rel-16 UE power saving scenario), then the LP-WUR still need “ON”, or at least to be “ON” some time before the entry conditions, following RAN2’s agreement.

- At RAN4 114 meeting we have the agreement that “ when and how to turn on LR for serving cell measurement is up to UE implementation” and when LR is “on” at legacy state, whether the LR evaluation requirement apply or not still need discussion.

- Our view is at legacy the LR evaluation requirement does not apply even when LR is “ON” and evaluating LR related threshold. The reason is the configuration of the LR threshold at legacy state seems strange since the LR need be “ON” to determine whether the LR can be “ON” or not. The logic is not clear hence it is preferred that the LR evaluation requirement does not apply when LR is “ON’ at legacy state.

- At legacy state, when LP-WUR measurement is “ON” for evaluating LR threshold, LR measurement and evaluation related requirements will not apply, i.e., no LR entry threshold evaluation requirements for the following cases: from legacy case to LP-WUR monitoring, from legacy case to RRM measurement fully offloading (case 1), and from legacy case to RRM measurement relaxation (case 3).

Issue 1-1-10: Considerations on higher priority frequency layer

Agreement:

- Support relaxed higher priority frequency layer search at case 1

Tentative agreement:

- For case 1, when the serving cell is above threshold for inter-frequency or inter-RAT neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search.

  - Note: K2 is 60

For the high priority frequency layer search, we have the former agreement and tentative agreement. Regarding the tentative agreement, it emphasizes two items, firstly the relaxed higher priority frequency searer shall be supported, secondly the requirements for the relaxed higher priority frequency searcher is fixed whereas when a UE can entry case 1 is up to thresholds designed by RAN2.

In legacy Rel-15 and Rel-16, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the higher priority frequency searcher, either perform with a rate Thigher_priority_search or K2*Thigher_priority_search, for Rel-19 LP-WUR, K2*Thigher_priority_search should be reused since firstly it should match Rel-16 UE power saving WI and secondly, the relaxed measurement is a compromise between fully no higher priority frequency searcher and have higher priority frequency searcher, which has already been extensively discussed.

- Confirm the following tentative agreement and the conditions when relaxed higher priority frequency searcher happen like Srxlev > SnonIntraSearchP used in legacy specs should wait for RAN2 progress.

For case 1, when the serving cell is above threshold for inter-frequency or inter-RAT neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search.

Note: K2 is 60

Issue 1-1-11: RRM requirements for FR2

- Proposals

  - P1: Prioritize defining RRM requirements for FR1 in this WI (Apple Nokia)

- Regarding FR2, there is not any concrete proposal on FR2 requirements so far. Suggest RAN4 to confirm whether FR2 RRM requirement needs be defined in this release, or confirm the status of FR2 requirement will not impact the completion of WI.

Issue 1-1-13: Impact on specs

The following was agreed at RAN4 114bis meeting.

|  | Clause | Impact Comment |
| --- | --- | --- |
| 3 | Definitions, symbols and abbreviations | Yes |
| 4 | SA: RRC_IDLE state mobility | Yes |
| 5 | SA: RRC_INACTIVE state mobility | Yes |
| 6 | RRC_CONNECTED state mobility | No |
| 7 | Timing | No |
| 8 | Signalling characteristics | FFS |
| 9 | Measurement Procedure | FFS |
| 10 | Measurement Performance requirements | No |
| Annex B | Conditions for RRM requirements applicability for operating bands | Yes |

Based on the WID and issue 1-4-1, the majority view from companies are there is no scope for RRM in connected state. Hence we suggest to remove FFS for section 8 and 9. In the end if any particular update on these sections, it can be handled in the maintenance phase.

- Have the following update on the impact for specs.

|  | Clause | Impact Comment |
| --- | --- | --- |
| 8 | Signalling characteristics | No |
| 9 | Measurement Procedure | No |

Issue 1-1-15 RedCap UE capability

- Proposals

  - P1: RedCap UE shall meet LP-WUS related requirement based on UE’s capability (Ericsson)

Recommendations:

For this issue, usually how one feature operates on top of the other feature should have a case by case study to determine whether they can work together and how to work together if necessary. The discussion on whether Redcap UE should support LP-WUS and how to support if necessary should not have any impact on WI completion.

- The discussion on whether Redcap UE should support LP-WUS and how to support if necessary should not have any impact on WI completion

Issue 1-1-16 UE capability

For the functionalities of the LP-WUR related to RAN4 RRM, i.e., LP-WUS UE RRM relaxation and offloading, the corresponding UE capabilities had already been discussed RAN1 hence there is no need for RAN4 RRM to discuss UE capabilities for LP-WUS UE RRM relaxation and offloading.

- There is no need for RAN4 RRM to discuss UE capabilities for LP-WUS UE RRM relaxation and offloading since they had already been discussed in RAN1.

Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements

Regarding the concrete number of accuracy, the following agreements are available at RAN4 114bis meeting:

Agreement:

For FR1:

    - [3.5] dB can be used for core requirements for LP-RSRQ accuracy and [5.5 or 6] dB can be used for core requirements for LP-RSRP accuracy, under the side conditions Ês/Iot [x, x = -3] dB

    - [3.5] dB can be used for core requirements of SSB based RSRQ accuracy and [5.5 or 6] dB can be used for core requirements for SSB based RSRP accuracy, under the side conditions Ês/Iot [y, y = -3] dB

    - Note: [2 or 2.5] dB RF impairment margin is assumed

Regarding the RF impairment margin, as we indicated in our previous contribution [R4-2503298], the RSRP accuracy requirement includes the impact of the RF impairment and a margin is added whose typical value is from 2-3 dB and 2 dB was suggested [R4-2503298]. Considering the performance of the LP-WUR, the RF impairment margin may need more hence it is ok to use 2.5 dB as suggested by other companies.

- Use 2.5 dB as the RF impairment margin for LP-RSRP and [SS-RSRP] accuracy requirements.

Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity

- Proposals

  - P1: In case the periodicity of LP-SS can be configured to small value like 80ms or 160ms (xiaomi)

    - Option 1: define the lower limit for measurement delay requirements; (Huawei)

    - Option 2: define the measurement delay requirements based on max(LP-SS periodicity, LO periodicity).

  - P2: No need for lower bound. (Nokia)

  - P3: Wait for more RAN1 progress before determining whether to have lower bound. (vivo oppo)

At RAN1 120bis meeting, the following agreements are available:

Agreement

Regarding whether to support additional sync signal to LP-SS, support the following for LP-WUS with M=1, M=2:

- Option 1A: No additional sync signal, LP-SS periodicity =320ms.

  - Additionally support LP-SS periodicity of 160ms.

Both 160ms and 320 ms are supported for LP-SS periodicity and 80ms is not introduced. Hence the necessity to have the lower bound is low. The LR requirements based on LP-SS can be classified as requirements based on 160ms and requirements based on 320ms.

- The LR requirements based on LP-SS can be classified as requirements based on 160ms and requirements based on 320ms.

Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity

- Proposals

  - P1: No need to define upper bound for SSB-based LP-WUR measurement periodicity. (xiaomi oppo CATT CT Nokia)

  - P2: Prefer the periodicity for measurement and evaluation delay requirements for LP-WUR based on SSB in IDLE/Inactive mode is 1.28s if the LO periodicity is 2.56s. (vivo ZTE)

  - P3: Postpone the discussion for the case where LP-SS periodicity > LO periodicity until RAN1 has more agreements for LP-SS periodicity (Huawei)

- For the upper bound on SSB based LR measurement periodicity, 2.56s may too longer and an upper bound is preferred.

Issue 1-2-4-1: On LR measurement filtering requirement

- Proposals

  - P1-1: The UE shall filter the LP-RSRP and LP-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by LP-SS periodicity/2 for LP-SS based LR. (vivo CT Ericsson)

  - P1-2: For LP-SS-RSRP and LP-SS-RSRQ measurement, no need to define filtering requirement (CMCC)

  - P2:The UE shall filter the [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by [LO periodicity /2] for SSB based LR. (vivo CMCC CT Ericsson Nokia)

  - P3: UE shall filter the LR measurements using at least 2 measurements with enough separation. (Huawei)

For the filtering related requirements, the following have been agreed at RAN4 114bis meeting:

Agreement:

  - The UE shall filter the [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by [LO periodicity /2] for SSB based LR.

Regarding the filtering requirement, for the SS-RSRP and SS-RSRQ, since the LP-SS periodicity is either 160ms or 320ms, there is no much implementation freedom to perform two measurements closely hence the filtering requirements need not be defined for LP-RSRP and LP-RSRQ.

- No need define filtering requirement for LP-RSRP and LP-RSRQ measurements.

Issue 1-2-4-2-1: On applicability LR evaluation requirements

- Proposals

  - P1: The LR evaluation requirements will apply for the following scenario (vivo):

    - entry to legacy case from case 1  (Xiaomi)

    - entry to case 1(RRM measurement fully offloading) from case 3 (Xiaomi)

    - [entry to legacy case from LP-WUR monitoring case] (xiaomi)

    - [entry to legacy case from case 3]  (Xiaomi)

    - [entry to case 3(RRM measurement relaxation) from case 1] (Xiaomi)

- The LR evaluation requirements will apply for the following scenario:

- entry to legacy case from case 1

- entry to case 1(RRM measurement fully offloading) from case 3

- entry to case 3(RRM measurement relaxation) from case 1

- [exit from LP-WUR monitoring case to legacy case]

- [exit to legacy case from case 3]

- [Entry to LP-WUR monitoring, to RRM measurement fully offloading (case 1), to RRM measurement relaxation (case 3) from legacy case]

Issue 1-2-4-2-3: On how to define LR evaluation requirements

- Proposals

- P1: For LR evaluation requirements, x and y can be determined by simulation results, and x1 or y1 can be N times of x or y (oppo)

- P2: Define LR evaluation requirement based on x1 = y1 = [4]. (Huawei)

- P3: For LR evaluation requirements and since OOK LR perform worse than MR, a higher number of samples than MR is required to achieve reliable evaluation. (MTK)

It was determined that the number of samples used to derive the evaluation requirement x1 and y1 for LP-SS and SSB based LR, should be larger than the value x, y, which is the number of SSBs to satisfy the accuracy and obtained through simulation campaign. However detail information still needs further investigation. To our understanding the x1 and y1 are not necessary to be multiple times of x and y, respectively, the main purpose of x1 and y1 is to introduce some margin on top of x and y. Based on the collection of simulation results so far [5], 2 or 3 samples could be the final value of x or y. The corresponding value of x1 and y1 could be 1 more sample on top of x and y.

- The value x1 for LP-SS based and y1 for SSB based LR evaluation requirement could be defined as x+1 and y + 1.

Issue 1-2-5-1: On applicability MR evaluation requirements

The MR measurement and evaluation should also be defined for LP-WUR monitoring, case 1 and case 3. Based on table 1, the application of the MR evaluation requirements is described in the following propose.

- The MR evaluation requirements will apply for the following scenario:

- Entry to LP-WUR monitoring, to RRM measurement fully offloading (case 1), to RRM measurement relaxation (case 3) from legacy case

- [exit to legacy case from case 3]

- entry to case 1(RRM measurement fully offloading) from case 3

Issue 1-2-6-1: On the number of SSBs

- Proposals

  - P1: RAN4 confirm the number of SSBs used for MR wake up delay definition assumed by RAN1, in particular, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization, is right. (vivo oppo CMCC Apple LG Huawei)

    - P1-1: Inform RAN1 that additional 3 SSB samples are assumed for SSB index acquisition. (oppo)

    - P1-2: RAN4 assumes this MR wake-up delay capability and the number of SSBs for MR synchronization shall be associated with specific target SINR condition, e.g., ≥-3dB. (Apple)

  - P2: 3SSBs could be used for AGC, timing sync and others, and 5 SSBs are not needed (ZTE)

    - Whether the types of LR would be considered. (ZTE)

    - Note: Only ramp up time is different between deep sleep state and ultra-deep sleep state. (ZTE)

  - P3: RAN4 to suggest RAN1 to update the wake-up delay capability to {[50ms], [500ms] and [900ms]}, where [50ms] assumes [2] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. (Ericsson)

  - P4: 1 SSB samples is enough for sync in good radio conditions where LR is assumed to be used. After MR wake up, MR follows SSB periodicity at least for first set of measurements Confirm that the following wake up times with sync times are ok for RAN4: (Nokia)

  - P5: RAN4 to discuss the need to have different number of SSBs (3 and 5) for different warm-up delays. (QC)

  - P6: It is suggested to use 5 number of SSBs for MR synchronization for SSB periodicity of 20ms. (Xiaomi)

Recommendations:

- For the number of samples and side conditions could consider:

  - 1) determine the SSB number based on minimum number for AGC, timing etc., without link to any particular side conditions

  - 2) determine the SSB number based on minimum number for AGC, timing etc., considering LR side condition.

For number of SSB samples for MR wake up delay, the content of R1-2501624 is copied below:

In the LS the number of SSBs assumed by RAN1 should be checked by RAN4. In our previous contribution we already provided analysis on this part [7], [8]. For a MR to get synchronous after triggered from ultra-deep sleep mode, this component will consist of at least the time for AGC, raw timing sync and other related procedure if necessary. Typically 1-2 SSBs (1-2 samples) are needed for AGC and 1-2 SSBs are needed for raw timing sync respectively. When the MR is wake up from the offloading status (ultra-deep sleep) by LP-WUS, normally its serving cell measurement should be in good status and a few samples (1-2 SSBs) is sufficient for MR measurement purpose otherwise it will exit these modes. As the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring is considered for definition in RAN1, the RAN1’s assumption of 5 SSBs from ultra-deep sleep mode is reasonable. In addition, 3 SSBs is considered when UE is wake up from deep-sleep mode, the value is reasonable since the number of samples used for AGC, raw synchronization and measurement can be reduced, if not fully removed, compared with that of the waking up from ultradeep-sleep mode scenario.

However the number of SSB is also diverse and the following options are available at ad hoc meeting [R4-2504921]

Option 1: [ 3 5 5] (Apple vivo, LG, HW, MTK, oppo, CMCC) Side condition [>= -3 dB]

Option 2: [3 3 3] (ZTE) (Side condition -3 dB)

Option 3: [5 5 5] (xiaomi)  (-6 dB)

Option 4: [2 2 2] (Nokia)  (-2 dB)

The controversial part, to some extent, is regarding to side conditions assumed for the number of SSBs. To our understanding, the SSB number derived from RAN1 is based on almost the worst case PUSCH Msg3 coverage, which matches the LR side condition -3 dB. Our preference is if the side conditions are needed, LR side conditions such as -3 dB could be used and there will be a corresponding MR side condition anyway, however it cannot be derived within a short period. Alternatively the number of SSBs can be determined based on minimum requirement, for example, there will be always at least 1 SSB for AGC no matter whether the SNR is good or bad. At least 1 SSB for measurement and 1 SSB for margin purpose.

- RAN4 confirm the number of SSBs used for MR wake up delay definition assumed by RAN1, in particular, [3 5 5] SSBs for [70]ms, [500]ms and [900] ms cases, if necessary, under the side conditions LR SINR -3 dB. Alternatively the minimum number of SSB can be derived without considering any side conditions.

Issue 1-2-8-1: Accuracy for normal or relaxed MR serving cell measurement

- Proposals

  - P1: No new accuracy requirements will be introduced for MR either normal or relaxed serving cell measurement,

    - no margin will be introduced for MR serving cell measurement in specs. (vivo)

    - same margins as existing MR neighbour cell measurement apply (Huawei)

  - P2: To enable reliable offloading from MR it is necessary to have absolute measurement accuracy requirements defined for MR. The absolute measurement accuracy already defined for idle mode can be re-used when defining idle mode measurements for LR offloading purposes (Nokia)

Recommendations:

Check P1 is agreeable

For accuracy for MR, margins, which reflect accuracy requirements, are directly defined for intra-frequency/inter-frequency and inter-RAT measurements. However margins are not explicitly defined for serving cell measurement when serving cell measurement is used to compared with a threshold, either for cell selection or for triggering intra/inter frequency measurement or for triggering Rel-16 RRM relaxation.

Since the margin has never been explicitly defined for the MR serving cell measurement before, following the same logic, it is not necessary to have any explicitly margin defined when the MR serving cell measurement is used to compare with thresholds for LP-WUR operation.

Issue 1-2-8-2: Accuracy for relaxed MR neighbour cell measurement

- Proposals

  - P1: The legacy accuracy for relaxed MR measurement should be reused if Case#2 and/or Case#3 introduced. (CMCC vivo Huawei)

For the accuracy requirements for relaxed MR measurement, the legacy requirements are used during Rel-16 UE power saving WI. It is straightforward to the use same principle, i.e., use the legacy accuracy requirements for relaxed MR measurement.

- For MR, margin is not explicitly specified in specs when MR serving cell measurement result comparing with a threshold as that of the legacy specs. For relaxed MR measurement, the legacy accuracy requirements for MR are re-used.

MR RRM relaxation

LP-WUR connected mode

Issue 1-4-1: LP-WUR at CONNECTED mode

- Proposals

  - P1: No RRM objectives is expected for connected mode in this WI. (CATT xiaomi oppo ZTE Apple CT MTK)

  - P2: Postponed until more progress in other groups (Ericsson Huawei vivo)

  - P3: RAN4 to introduce interruption requirements on the WUS monitoring in Connected mode, e.g., when the WUS MO collides with a MG. The interruption on WUS monitoring follows the current interruption requirements on DL data during MGs. (Qualcomm)

Regarding P4, the assumption is that MR and LR works at different time, when the WUS monitoring occasion collides with a MG, one possible solution is the MR uses the MG to perform measurement and LR monitors LP-WUR after MG. However the detailed design could be up to RAN1 design and since there is no RAN4 scope on CONNECTED mode, it is suggested to postpone the discussion on this issue.

- For issue 1-4-1, postpone the discussion.

Other issues

Issue 1-5-1: LR based RRM with EMR in IDLE/inactive mode

- Proposals

  - P1: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not. (Apple Nokia)

  - P2: Postpone the discussion on the EMR issue (CMCC vivo)

  - P3: RAN4 to discuss the UE’s behaviour of MR relaxation with EMR impact (Ericsson)

Regarding EMR related issue, to our understanding the principle P1 is reasonable. The issue itself is not crucial for Rel-19 requirements point of view and can be discussed later.

- For issue 1-5-1, the principle of P1 is reasonable. The issue can be discussed later.

Conclusion

In this contribution, we provide our considerations on the LP-WUS/WUR WI and have the following observations and proposals.

- Case 2 is preferred. The discussion can be postponed until more RAN2’s progress.

- Discussion on threshold should be up to RAN2.

- At legacy state, when LP-WUR measurement is “ON” for evaluating LR threshold, LR measurement and evaluation related requirements will not apply, i.e., no LR entry threshold evaluation requirements for the following cases: from legacy case to LP-WUR monitoring, from legacy case to RRM measurement fully offloading (case 1), and from legacy case to RRM measurement relaxation (case 3).

- Confirm the following tentative agreement and the conditions when relaxed higher priority frequency searcher happen like Srxlev > SnonIntraSearchP used in legacy specs should wait for RAN2 progress.

For case 1, when the serving cell is above threshold for inter-frequency or inter-RAT neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search.

Note: K2 is 60

- Regarding FR2, there is not any concrete proposal on FR2 requirements so far. Suggest RAN4 to confirm whether FR2 RRM requirement needs be defined in this release, or confirm the status of FR2 requirement will not impact the completion of WI.

- Have the following update on the impact for specs.

|  | Clause | Impact Comment |
| --- | --- | --- |
| 8 | Signalling characteristics | No |
| 9 | Measurement Procedure | No |

- The discussion on whether Redcap UE should support LP-WUS and how to support if necessary should not have any impact on WI completion

- There is no need for RAN4 RRM to discuss UE capabilities for LP-WUS UE RRM relaxation and offloading since they had already been discussed in RAN1.

- Use 2.5 dB as the RF impairment margin for LP-RSRP and [SS-RSRP] accuracy requirements.

- The LR requirements based on LP-SS can be classified as requirements based on 160ms and requirements based on 320ms.

- For the upper bound on SSB based LR measurement periodicity, 2.56s may too longer and an upper bound is preferred.

- No need define filtering requirement for LP-RSRP and LP-RSRQ measurements.

- The LR evaluation requirements will apply for the following scenario:

- entry to legacy case from case 1

- entry to case 1(RRM measurement fully offloading) from case 3

- entry to case 3(RRM measurement relaxation) from case 1

- [exit from LP-WUR monitoring case to legacy case]

- [exit to legacy case from case 3]

- [Entry to LP-WUR monitoring, to RRM measurement fully offloading (case 1), to RRM measurement relaxation (case 3) from legacy case]

- The value x1 for LP-SS based and y1 for SSB based LR evaluation requirement could be defined as x+1 and y + 1.

- The MR evaluation requirements will apply for the following scenario:

- Entry to LP-WUR monitoring, to RRM measurement fully offloading (case 1), to RRM measurement relaxation (case 3) from legacy case

- [exit to legacy case from case 3]

- entry to case 1(RRM measurement fully offloading) from case 3

- RAN4 confirm the number of SSBs used for MR wake up delay definition assumed by RAN1, in particular, [3 5 5] SSBs for [70]ms, [500]ms and [900] ms cases, if necessary, under the side conditions LR SINR -3 dB. Alternatively the minimum number of SSB can be derived without considering any side conditions.

- For MR, margin is not explicitly specified in specs when MR serving cell measurement result comparing with a threshold as that of the legacy specs. For relaxed MR measurement, the legacy accuracy requirements for MR are re-used.

- For issue 1-4-1, postpone the discussion.

- For issue 1-5-1, the principle of P1 is reasonable. The issue can be discussed later.

- References

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 102

[2] RP-240801, Revised WID: Low-power wake-up signal and receiver for NR ((LP-WUS/WUR), RAN 103

[3] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 105, vivo, NTT DOCOMO

[4] R4-2504908, WF on RRM requirements for NR_LPWUS, RAN4 114bis, vivo

[5] R4-2503627,	Topic summary for [114][225] NR_LPWUS, RAN4 114bis, vivo