# 3GPP TSG-RAN WG4 Meeting #115                                 R4-2505688

# Malta, MT, 19th – 23th May, 2025

Title: 	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Source: 	OPPO

Agenda item:	7.18.3.1

Document for:	Approval

Introduction

In last meeting, FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor was discussed and the agreements were captured in the approved WF[1].

In this contribution, we will provide our views on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor.

Discussion

In previous meetings, the trigger condition to activate Multi-Rx based L3 measurement has been agreed for scenario 1/2/3/4 which is based on serving cell quality. But the quit condition and re-evaluation mechanism are still FFS.

Based on the proposals in last meeting [2], there are three types of solutions are provided:

- Option 1: UE autonomous quitting, e.g., UE is allowed to quit Multi-Rx mode after UE has reported qualified measurement results.

- Option 2: UE indication to the NW is used to deactivate FBS based L3 measurements.

- Option 3: Timer-based solution to deactivate Rx BSF reduction.

Considering the UE power consumption is an important issue for Multi-Rx operation for L3 measurement, we prefer to have this quit condition. As discussed in the last meeting, if UE stays in Multi-Rx mode for a long time but not meets the handover condition, the power consumption would be high and may cause overheating issue. Regarding the above three approaches, we think all of them can achieve the goal, but using UE indication is the simplest one and can be aligned with Rel-18 Multi-Rx. For the solution which is based on reports, since the definition of “qualified” is not clear, we prefer not to use the measurement report to deactivate the FBS mode. For the detailed signaling, we think either the legacy signalling like the overheating indication, or new UAI indication of UE preference is feasible. We can leave it to RAN2 decision.

For the re-evaluation mechanism, we think the timer-based solution can be used, i.e., the timer starts after UE quits from Multi-Rx mode and UE re-evaluates the activation condition after the timer expires.

Proposal 1: For scenario 1/2/3/4, UE indication to the NW can be used to deactivate FBS based L3 measurements. The detailed signaling can leave to RAN2 decision.

Proposal 2: UE re-evaluates the activation condition after the timer expires where the timer starts after UE quits from Multi-Rx mode.

After discussing the activation/deactivation condition for Multi-Rx mode, there are proposals for UE to report the status of fast beam sweeping since the satisfaction of trigger condition is based on UE measurement. But we don’t think the status report is needed since NW does not need to know. The status is related to how fast UE can perform the measurement, but the NW only needs the actual results report and does not care how UE perform this measurement. After UE pass the testing, NW know that the UE will report the results under during a certain time based on configurations. Even if this status information is reported, NW has no corresponding behaviours and still needs to wait for UE to report the measurement results.

Proposal 3: No need to define UE status report of fast beam sweeping.

Regarding the capability, it was agreed to introduce the UE capability of reduced N, but the granularity is FFS. Since the implementation and capability of UE antenna on different bands may be different, to consider the flexibility of UE implementation, we think the capability can be per band.

Issue 1-2-2: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF Agreement in the ad-hoc: - Introduce the UE capability of reduced N, the values of which are {2, 4 and 6}. And UE chooses one value to report according to its capability. FFS the granularity of the UE capability in issue 1-2-4.

Proposal 4: Define the UE capability of reduced N as per band capability.

Conclusions

In this paper, we provided our views on FR2-1 SSB based L3 measurement delay reduction for optimizing BSF. And the following proposals are provided.

Proposal 1: For scenario 1/2/3/4, UE indication to the NW can be used to deactivate FBS based L3 measurements. The detailed signaling can leave to RAN2 decision.

Proposal 2: UE re-evaluates the activation condition after the timer expires where the timer starts after UE quits from Multi-Rx mode.

Proposal 3: No need to define UE status report of fast beam sweeping.

Proposal 4: Define the UE capability of reduced N as per band capability.

Reference

- R4-2504900, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2503619, Topic summary for [114bis][209] NR_RRM_Ph5_Part2, CATT