# 3GPP TSG-RAN WG4 Meeting #115                                 R4-2505693

# Malta, MT, 19th – 23th May, 2025

Title: 	Discussion on performance requirements for fast SCell activation for UE supporting Rel-18 EMR

Source: 	OPPO

Agenda item:	7.18.6

Document for:	Approval

Introduction

In last meeting, the performance requirements for fast SCell activation for UE supporting Rel-18 EMR In this contribution were not discussed due to time limit. In this paper, we will provide our views on based on the open issues captured in [1].

Discussion

Based on the endorsed draft CR in [2], the following requirements are defined for fast SCell activation for UE supporting Rel-18 for both FR1 and FR2 and the corresponding test cases need to be defined:

- Normal SCell activation

- Direct SCell activation at SCell addition

- PUCCH SCell activation

And the following conditions are needed to apply the fast SCell activation:

Requirements in this clause apply if the SCell is unknown, provided: - UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or, - UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or, - UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode,

If the SCell belongs to FR1, Tactivation_time is TFirstSSB_MAX + Trs + 5ms provided that the side condition Ês/Iot ≥ -2dB is fulfilled, if the following conditions are met: - During the period equal to 5s for FR1 before the reception of the SCell activation command: - the UE has sent a measurement report with SSB index according to the reporting requirements in 4.7.3 or 5.8.3. - the SSB measured is detectable from the earliest measurement to the end of the defined SCell activation latency (i.e., a valid CQI reporting) - If measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 is configured, the earliest measurement refers to the time instance measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 before Msg1 transmission. - If only measIdleDuration-r16 is configured, the earliest measurement refers to the time instance when the UE obtained the configuration parameter. - the SSB measured remains detectable according to the IDLE/INACTIVE mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.

To verify the fast SCell activation delay requirements, the configurations in the test cases need to meet the above conditions. E.g., the configuration of measIdleValidityDuration-r18, measReselectionValidityDuration-r18 or measIdleDuration-r16 shall be based on UE capability.

Proposal 1: RAN4 to define the following test cases for fast SCell activation:

- Normal SCell activation for FR1 and FR2

- Direct SCell activation at SCell addition for FR1 and FR2

- PUCCH SCell activation for FR1 and FR2

Proposal 2: The configuration of test cases should meet the conditions for fast SCell activation.

Regarding the test procedure, we understand the test can consist of four successive time periods, with duration T1, T2, T3 and T4, respectively. And in the test, there are two NR carriers, each with one cell. Both cells have constant signal levels throughout the test.

During T1, UE is in RRC_IDLE mode and performs cell reselection measurement or CA/DC measurement on the two cells based on the configurations. At the beginning of T2, UE is connected to Cell 1, but is not aware of Cell 2. The test configurations of T1 and T2 can use A.6.1.1/A.7.1.1 or A.6.6.9/A.7.6.23 as baseline. At the beginning of T3, UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2, and the UE now starts monitoring the SCC. At the beginning of T4 (denoted as slot #n), UE receives the MAC message at the UE antenna connector for activation of SCell. The UE should meet the fast SCell activation delay requirements during T4. For direction SCell activation, T3 and T4 can be merged into one. The test configurations of T3 and T4 can use A.6.5.3 as baseline.

Proposal 3: The test can consist of four successive time periods, with duration T1, T2, T3 and T4, respectively.

- T1 for IDLE mode measurement, T2 for transition to connected mode, T3 for SCell configuration and T4 for SCell activation.

- For direct SCell activation, T3 and T4 can be merged into one.

Proposal 4: The test configurations of T1 and T2 can use the unknown case in legacy cell reselection (A.6.1.1/A.7.1.1) or IDLE mode CA/DC measurement (A.6.6.9/A.7.6.23) as baseline.

Proposal 5: The test configurations of T3 and T4 can use the unknown case in legacy SCell activation (A.6.5.3) as baseline.

Conclusions

In this paper, we provided our views on the performance requirements for fast SCell activation for UE supporting Rel-18 EMR. And the following proposals are provided.

Proposal 1: RAN4 to define the following test cases for fast SCell activation:

- Normal SCell activation for FR1 and FR2

- Direct SCell activation at SCell addition for FR1 and FR2

- PUCCH SCell activation for FR1 and FR2

Proposal 2: The configuration of test cases should meet the conditions for fast SCell activation.

Proposal 3: The test can consist of four successive time periods, with duration T1, T2, T3 and T4, respectively.

- T1 for IDLE mode measurement, T2 for transition to connected mode, T3 for SCell configuration and T4 for SCell activation.

- For direct SCell activation, T3 and T4 can be merged into one.

Proposal 4: The test configurations of T1 and T2 can use the unknown case in legacy cell reselection (A.6.1.1/A.7.1.1) or IDLE mode CA/DC measurement (A.6.6.9/A.7.6.23) as baseline.

Proposal 5: The test configurations of T3 and T4 can use the unknown case in legacy SCell activation (A.6.5.3) as baseline.

Reference

- R4-2503619, Topic summary for [114bis][209] NR_RRM_Ph5_Part2, CATT

- R4-2420132 draftCR on Fast SCell activation for UE supporting Rel-18 eEMR

- R4-2502685 draft CR for fast SCell activation for UE supporting Rel-18 EMR

- R4-2504900, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT