# 3GPP TSG-RAN WG4 Meeting #115	R4-2505699

# Malta, MT, 19th – 23rd May, 2025

Title: 	On RRM requirements for UE-to-UE CLI handling

Source: 	OPPO

Agenda item:	7.25.4.1

Document for:	Discussion

Introduction

In last meeting, a WF on RRM requirements for NR_duplex_evo was approved [1].

In this contribution, we will provide our views on RRM requirements for UE-to-UE CLI handling.

Discussion

- Sub-topic 1-2: L1-SRS-RSRP - Issue 1-2-3: Measurement restriction - Agreements - Measurement requirements do not apply for L1-SRS-RSRP when the measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, FFS the following options - Option 1: Prioritize L1-SRS-RSRP measurement - Option 2: Prioritize L1-CLI-RSSI measurement - Option 3: Leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI

In last meeting, RAN4 discussed the measurement restriction between L1-SRS-RSRP and L1-CLI-RSSI. As L1-CLI-RSSI is based on DL timing, the two measurements cannot be performed at same time. We suggest to leave it to UE implementation which measurement to perform when measurement resources for the two measurements are partially or fully overlapping. It is similar to the existing measurement restriction for SSB and CSI-RS based L1 measurement that UE is required to perform one but not both measurements.

Proposal 1: When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI, and measurement period requirements for the two measurements do not apply.

- Sub-topic 1-2: L1-SRS-RSRP - Issue 1-2-2: Scheduling restriction - Agreement (online session on Monday with update) - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for SRS resource and Y symbols before, where Y is re-using same number as R16 requirements. - For the symbols after the SRS resource, not define scheduling restriction on symbols after symbols for SRS resource in TS 38.133, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211. - For DL, - if UE is able to support FDM-ed DL reception and SRS measurement, scheduling restriction does not apply except when SRS resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, otherwise (if UE does not support the capability), scheduling restriction applies. - The restricted symbols include symbols for SRS resource and Y symbols before symbols for SRS resource, where Y is re-using same number as R16 requirements. - FFS whether restricted symbol(s) is needed for the symbol(s) after SRS resource to consider the switching between UL subband and DL subband.

- Sub-topic 1-3: L1-CLI-RSSI - Issue 1-3-2: Scheduling restriction - Agreements (adhoc session on Tuesday with addition) - Method #1: - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for RSSI resource. - Do not define scheduling restriction on symbols after symbols for RSSI resource, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211 clause 4.3.2 - For DL, - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource - Method #3: - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for RSSI resource. - Do not define scheduling restriction on symbols after symbols for RSSI resource, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211 clause 4.3.2 - For DL, - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource. - Note: the common understanding in RAN4 if the RSSI in UL suband is too strong compare to the wanted signals in DL suband(s), UE may fail to decode the PDCCH or PDSCH. - FFS whether restricted symbol(s) is needed for the symbol(s) after RSSI resource to consider the switching between UL subband and DL subband. - Generic assumption for defining RRM requirements, not intend to add it in the core requirement. - For all Method #1, Method #2 and Method #3, - Note: The RF BW is wide enough to cover DL active BWP, including PRBs from both DL subband(s) and UL subband that fall within the DL active BWP.

For both L1-CLI-RSSI and L1-SRS-RSRP measurement, RAN4 has agreed on the scheduling restriction for UL transmission, but leave it FFS for DL reception. We are going to discuss whether restricted symbol(s) is needed for the symbol(s) especially after SRS resource or RSSI resource to consider the switching between UL subband and DL subband. Since the DL timing can be maintained after SRS resource or RSSI resource, and the requirement of transient period is already covered by RF requirements for UE switching from Tx to Rx, which is usually defined as several ‘u-seconds’. Thus we suggest not to consider the restricted symbols in DL reception after the measurements on SRS resource or RSSI resource due to the switching between UL subband and DL subband.

Proposal 2: RAN4 not to consider the restricted symbols in DL reception after the measurements on L1-CLI-SRS resource or L1-CLI-RSSI resource due to the switching between UL subband and DL subband.

Conclusions

In this paper we provided our views on RRM requirements for UE CLI measurement in SBFD.

Proposal 1: When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI, and measurement period requirements for the two measurements do not apply.

Proposal 2: RAN4 not to consider the restricted symbols in DL reception after the measurements on L1-CLI-SRS resource or L1-CLI-RSSI resource due to the switching between UL subband and DL subband.

Reference

- R4-2504904, WF on RRM requirements for NR_duplex_evo