# 3GPP TSG-RAN WG4 Meeting #115	R4-2505710

# Malta, MT, 19th – 23rd May, 2025

Title: 	On event triggered L1 measurement reporting

Source: 	OPPO

Agenda item:	7.29.3.1

Document for:	Discussion

Introduction

In last meeting, a WF was approved on RRM requirements for R19 mobility [1].

In this contribution, we provide our views on remaining issues of measurements related enhancements for LTM, including event trigger L1 measurement reporting.

Discussion

Event triggered L1 measurement reporting

- Issue 2-2: Event triggered reporting delay for L1 reporting Agreement: - Reuse the framework from L3 event triggered reporting. Discuss the details further.

RAN4 agreed to reuse the framework from L3 event triggered reporting. In our view, the safe solution to UE can be “the maximum L1-RSRP measurement period on the cells corresponding to the event”. But since RAN4 did not clarify this issue for event triggered L3 reporting from R15, it does not broke RRM requirements and related implementation of both UE and network. So we think the ambiguity here seems not a big issue. Either option is ok to UE. If we need to clarify this, the better way could be to solve it for both event triggered L3 reporting and event triggered L1 reporting.

Proposal 1: L1 event triggered measurement reporting delay shall be no larger than “the maximum L1-RSRP measurement period on the cells corresponding to the event”.

- Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category Candidate options: - Option 1: Yes (Xiaomi, CMCC, ZTE, CTC, vivo) - Option 2: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple) - Issue 2-4: whether to define a separate capability for supported number of events triggered L1 measurement reporting Candidate options: - Option 1: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM. (Xiaomi, CMCC, ZTE, E///, CTC) - Option 1a: Yes. RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS. (E///) - Option 2: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO)

We support to update the requirements for L1 event triggered reporting criteria per measurement category. Whether to define new Ecat capability for LTM L1 measurements or share Ecat capability with L3 measurements can be candidate options.

Proposal 2: Support to update the requirements for L1 event triggered reporting criteria per measurement category.

We support to include L1 reporting criteria per measurement category into current Ecat. UE can share the UE capability between L1 and L3 reporting. We suggest to use the same value of Ecat for L1+L3 intra-frequency and inter-frequency measurement category, and update the note for each of events description, as the table below for example:

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Intra-frequency Note 1,2,3,4,5 | 9 | Events for any one or a combination of intra-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells |
| Inter-frequency Note 2,3,4,5 | 10 | Events for any one or a combination of inter-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells |

Proposal 3: Update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat.

Conclusion

In the contribution we provide the following proposals:

Proposal 1: L1 event triggered measurement reporting delay shall be no larger than “the maximum L1-RSRP measurement period on the cells corresponding to the event”.

Proposal 2: Support to update the requirements for L1 event triggered reporting criteria per measurement category.

Proposal 3: Update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat.

Reference

- R4-2402601, WF on NR mobility enhancements Phase 4_part1