3GPP TSG-RAN WG4 Meeting #115                                             R4-2505715

Malta, MT, 19th – 23th May, 2025

Agenda Item:	7.31.5.1

Source: 	OPPO

Title: 	On RRM requirements of (e)RedCap UEs with NR FR1-NTN

Document for:	Approval

Introduction

In last RAN4 meeting, a WF on RRM requirements for Rel-19 NR NTN phase3 was approved [1].

In this paper, we will provide our views on the remaining issues of (e)RedCap UEs with NR FR1-NTN.

Discussion on support of (e)RedCap UE with NTN

- Issue 1-2-1-1a: The proximity distance between two SMTCs <Way Forward>: - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to Y ms. - Y=5 ms

In last meeting, RAN4 also discussed whether to relax the proximity distance between two SMTCs for (e)RedCap UEs when (e)RedCap UE is configured with more than one SMTC on a NTN carrier. The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion. We prefer to reuse the legacy value of 4ms which avoids introducing unnecessary delays or errors in measurement procedures. And it also avoids network updates to accommodate both normal and (e)RedCap UEs. If UE supports parallel processing two SMTC in TDM with proximity distance, we don’t see the big difference from legacy assumption. Otherwise, UE can report it doesn’t support the parallelSMTC capability.

Proposal 1: Prefer to reuse the legacy value of 4ms without relaxation for the proximity distance between two SMTCs for (e)RedCap UEs when (e)RedCap UE is configured with more than one SMTC on an NTN carrier.

- Issue 3-1-1: General HD-FDD related impact Agreement: - The legacy requirements and applicable conditions can be reused as baseline: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - [Minimum requirement for L1 indication for RLM and LR] - Scheduling availability of UE performing intra/inter measurements RAN4 to further discuss: - The legacy requirements and applicable conditions can be reused as baseline for defining requirements of RedCap UE in NTN scenario, and discuss whether there is spec impact for the following requirements by HD-FDD: - FFS: Scheduling availability of UE during L1-RSRP measurement - Other requirement is not precluded - Candidate options for the potential spec impact for each of the above requirements (if needed): - Option a: the requirement is extended, e.g., the requirement does not count resources, channels, SSB, etc if they overlap with the other direction as specified a valid case in RAN1 specification. - Option b: Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period. - Option c: The requirement applies when there is at least one RS within each measuremen interval. - FFS the same or different option for DRX and non-DRX - Issue 3-2-1: The impact on NR Handover due to HD-FDD Agreement: - Option 1: agree on the principle below and further discuss the wording in the CR: - For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added: - For RedCap NTN UE with HD-FDD, the requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added: - For RedCap NTN UE with HD-FDD, the requirements are met provided that - One SSB is available during T∆ - One SSB is available during TIU - Option 2: No spec impact

For the impact of HD-FDD, it was agreed that the legacy requirements and applicable conditions can be used as baseline for paging reception, handover interruption time, random access and RRC Connection Release with Redirection etc. We understand the conclusions also apply to minimum requirement for L1 indication for RLM and LR. And for the spec impact, we prefer to clarify that the requirement applies when there is at least one RS within each measurement interval.

Proposal 2: For the impact of HD-FDD on minimum requirement for L1 indication for RLM and LR, the legacy requirements and applicable conditions can be reused as baseline.

Proposal 3: For the spec impact of HD-FDD, clarify that the requirement applies when there is at least one RS within each measurement interval.

Conclusions

In this paper we provided our views on RRM requirements for support of (e)RedCap UEs with NR FR1-NTN.

Proposal 1: Prefer to reuse the legacy value of 4ms without relaxation for the proximity distance between two SMTCs for (e)RedCap UEs when (e)RedCap UE is configured with more than one SMTC on an NTN carrier.

Proposal 2: For the impact of HD-FDD on minimum requirement for L1 indication for RLM and LR, the legacy requirements and applicable conditions can be reused as baseline.

Proposal 3: For the spec impact of HD-FDD, clarify that the requirement applies when there is at least one RS within each measurement interval.

Reference

- R4-2504912 WF on RRM requirements for NR_NTN_Ph3_Part1, CATT