3GPP TSG-RAN WG4 Meeting #115	R4-2505717

St Julian’s, Malta, May 19th – 23rd, 2025

Source: 	Murata Manufacturing Co. Ltd

Title: 	MSD for DC_(n)3CA

Agenda item:	6.2.2	UE RF requirements for EN-DC and NE-DC of 2 DL with 2 UL (DC_R19_1BLTE_1BNR_2DL2UL)	[DC_R19_xBLTE_yBNR-Core]

Document for:	Approval

- Introduction

In RAN4#114-bis [1][2][3], the proponent confirmed that the downlink configuration of BCS for DC_(n)3CA is to use the CA_3C configuration for LTE, which indicates that the DC combination cannot reuse the legacy test point for DC_(n)3AA. Therefore, companies agreed to specify MSD values by introducing new test points. This contribution proposes MSD values using the new candidate test point.

- Discussion

2.1 Background Information

In RAN4#114-bis, the following test points were agreed upon by the interested companies.

Table 1: Test points for DC_(n)3CA

| EN-DC configuration | E-UTRA/NR band | FC (UL) | Channel bandwidth | UL allocation (LCRB) | FC (DL) (MHz) | MSD | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | [TBD] | FDD |
|  | 3 | N/A | 20 | N/A | 1845 | [TBD] |  |
|  | n3 | 1772.5 | 25 | 5 (RBstart = 128) | 1867.5 | N/A |  |
| DC_(n)3CA | 3 | N/A | 20 | N/A | 1840 | [TBD] |  |
|  | 3 | N/A | 5 | N/A | 1852.5 | [TBD] |  |
|  | n3 | 1772.5 | 25 | 5 (RBstart = 128) | 1867.5 | N/A |  |

This contribution evaluates MSD values for DC_(n)3CA using the above test points.

2.2 MSD evaluation

2.2.1 PA Calibration

PC3 PA is calibrated at MPR1 for -30 dBc ACLR using a fully allocated 20 MHz DFT-s-OFDM QPSK waveforms.

2.2.2 Simulation results

We evaluated MSD values for PC3 DC_(n)3CA using the following assumptions. Figure 1 shows our evaluation results.

- Tx-Rx duplexer isolation: 50dB

- Tx antenna duplex rejection in Rx range: 45dB

- Primary antenna to diversity antenna isolation: 10dB

- MRC combining and -1 dB Signal to-Noise Ratio (SNR)

Figure 1: MSD analysis for DC_(n)3CA

Focusing on MSD values for the LTE downlink component carrier closest to the NR UL component carrier, the LTE downlink configuration ‘5+20’ will represent the worst-case scenario for MSD.

Observation 1: When the LTE downlink component carrier closest to the NR uplink component carrier is configured to be smaller than the other LTE carrier, the MSD value will be worse, i.e., The LTE downlink configuration ‘5+20’ represents the worst-case scenario for DC_(n)3CA MSD.

Proposal 1: Specify MSD values and test points for DC_(n)3CA highlighted in Table 2.

Table 2: MSD for DC_(n)3CA

| EN-DC configuration | E-UTRA/NR band | FC (UL) | Channel bandwidth | UL allocation (LCRB) | FC (DL) (MHz) | MSD | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | [2.8] | FDD |
|  | 3 | N/A | 20 | N/A | 1845 | [0] |  |
|  | n3 | 1772.5 | 25 | 5 (RBstart = 128) | 1867.5 | N/A |  |

- Conclusion

Observation 1: When the LTE downlink component carrier closest to the NR uplink component carrier is configured to be smaller than the other LTE carrier, the MSD value will be worse, i.e., The LTE downlink configuration ‘5+20’ represents the worst-case scenario for DC_(n)3CA MSD.

Proposal 1: Specify MSD values and test points for DC_(n)3CA highlighted in Table 2.

Table 2: MSD for DC_(n)3CA

| EN-DC configuration | E-UTRA/NR band | FC (UL) | Channel bandwidth | UL allocation (LCRB) | FC (DL) (MHz) | MSD | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | [2.8] | FDD |
|  | 3 | N/A | 20 | N/A | 1845 | [0] |  |
|  | n3 | 1772.5 | 25 | 5 (RBstart = 128) | 1867.5 | N/A |  |

References:

[1] R4-2503367, “TP for TR 37.719-11-11 DC_(n)3”, Samsung, KT, RAN4#114-bis, Wuhan, China

[2] R4-2503241, “DC_(n)3CA MSD evaluation”, Skyworks Solutions Inc., RAN4#114-bis, Wuhan, China

[3] R4-2503724, “DC_(n)3CA configuration and MSD”, Murata Manufacturing Co Ltd., RAN4#114-bis, Wuhan, China