# 3GPP TSG-RAN WG4 Meeting #115	R4-2505719

# Malta, Malta, 19th – 23rd May, 2025

Title: 	A-MPR requirements for 7MHz channel bandwidth

Agenda item:	7.8.3

Source:	Huawei, HiSilicon

Document for:	Discussion

# Introduction

In the last meeting the WF was agreed, where the following configurations were considered for A-MPR evaluations:

- Evaluate A-MPR for: - NS_12 considering 7 MHz deployed within 814-824 MHz - NS_13 considering 7 MHz deployed in 817-824 MHz (Fc = 820.5 MHz). - NS_14 considering 7 MHz deployed within 824-849 MHz - NS_15 considering 7 MHz deployed within 814-845 MHz or within 841.5-849 MHz.

In this document we are presenting our A-MPR evaluations using these configurations

# Discussion

The PA was calibrated based on the following assumptions:

- PC3

- 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

- Carrier Leakage: 28dBc, Image: 28dBc, CIM3: 60dBc, CIM5: 70dBc

- ASE, SEM and ACLR are considered as gating factors (EVM was excluded).

In our plots the back off values below MPR level are in gray, as no A-MPR needed for them.

## NS_12

The following requirements should be considered by the UE when BS issues NS_12:

As an example, the A-MPR evaluations for QPSK with 7 MHz at 817.5 MHz (closest to the band edge) is shown in Figure 1, the remaining evaluations can be found in the Annex.

Figure 1: NS_12 power back off for7MHz CBW with QPSK and Fc= 817.5 MHz

Based on our evaluation we propose the following A-MPR regions and values.

Proposal 1: Consider the following A-MPR regions and values for NS_12 with 7MHz CBW.

Table 1: A-MPR regions for NS_12 for 7MHz CBW

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc ≤ 820.5 | ≤2.16 | >0 | A3 |

Table 2: A-MPR for NS_12

| Modulation/Waveform | A3 |
| --- | --- |
|  | Outer/Inner |
| DFT-s-OFDM QPSK | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 5 |
| DFT-s-OFDM 64 QAM | ≤ 5 |
| DFT-s-OFDM 256 QAM |  |
| CP-OFDM QPSK | ≤ 6.5 |
| CP-OFDM 16 QAM | ≤ 6.5 |
| CP-OFDM 64 QAM | ≤ 6.5 |
| CP-OFDM 256 QAM |  |
| NOTE: for 256QAM MPR would suffice and no A-MPR is needed. |  |

## NS_13

The following requirements should be considered by the UE when BS issues NS_13:

As an example, the A-MPR evaluations for QPSK with 7 MHz at 820.5 MHz is shown in Figure 2, the remaining evaluations can be found in the Annex.

Figure 2 NS_13 power back off for7MHz CBW with QPSK and Fc= 820.5 MHz

Proposal 2: Consider the following A-MPR regions and values for NS_13 with 7MHz CBW.

Table 3: A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | Fc = 820.5 MHz | ≤1.98 | >0 | A4 |

Table 4: A-MPR for NS_13

| Modulation/Waveform | A4 |
| --- | --- |
| DFT-s-OFDM QPSK | ≤ 4.5 |
| DFT-s-OFDM 16 QAM | ≤ 5 |
| DFT-s-OFDM 64 QAM | ≤ 5 |
| DFT-s-OFDM 256 QAM |  |
| CP-OFDM QPSK | ≤ 6.5 |
| CP-OFDM 16 QAM | ≤ 6.5 |
| CP-OFDM 64 QAM | ≤ 6.5 |
| CP-OFDM 256 QAM |  |
| NOTE: for 256QAM MPR would suffice and no A-MPR is needed. |  |

## NS_14

The following requirements should be considered by the UE when BS issues NS_14:

Our evaluations, in the Annex, show that no A-MPR is needed.

Proposal 3: No A-MPR for NS_14 with 7MHz CBW.

## NS_15

The following requirements should be considered by the UE when BS issues NS_15:

As an example, the A-MPR evaluations for QPSK with 7 MHz at 845.5 MHz is shown in Figure 3, the remaining evaluations can be found in the Annex.

Figure 3 NS_15 power back off for7MHz CBW with QPSK and Fc= 845.5 MHz

Table 5: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 845 < Fc ≤ 845.5 |  | ≥3.6 (Zone1) | A7 |
|  |  | ≥1.26, <2.7(Zone2) | ≥1.98, <3.6(Zone2) | A2 |
|  |  | ≥2.7, <3.6(Zone3) | ≥1.44, <3.6(Zone3) | A1 |
|  |  | ≥3.6(Zone4) | ≥0.9(Zone4) | A2 |
|  |  | ≥4.5(Zone5) | <0.9(Zone5) | A2 |
|  | 838 < Fc ≤ 841.5 |  | ≥4.5 (Zone1) | A8 |

In Table 5, the A-MPR regions is specified based on RBstart. However, the proposal 4 is based on RBend.

Proposal 4: Consider the following A-MPR regions and values for NS_15 with 7MHz CBW.

Table 6: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 845 < Fc ≤ 845.5 |  | ≥3.6 (Zone1) | A7 |
|  |  | ≥3.24, <6.3(Zone2)1 | ≥1.98, <3.6(Zone2) | A2 |
|  |  | ≥4.14, <6.3(Zone3) 2 | ≥1.44, <3.6(Zone3) | A1 |
|  |  | ≥4.5, <6.3(Zone4)3 | ≥0.9(Zone4) | A2 |
|  |  | ≥4.5 | <0.9(Zone5) | A2 |
|  | 838 < Fc ≤ 841.5 |  | ≥4.5 (Zone1) | A8 |
| NOTE 1: 1.26 ≤ (RBend- LCRB) *12*SCS (MHz) < 2.7 NOTE 2: 2.7 ≤ (RBend- LCRB) *12*SCS (MHz) < 3.6 NOTE 3: 3.6 ≤ (RBend- LCRB) *12*SCS (MHz) |  |  |  |  |

Table 7: A-MPR for NS_15

| Modulation/Waveform | A1 | A2 | A3 | A7 | A8 |
| --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 |  |  |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | 11.5 | 2 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | 12.5 | 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | 12.5 | 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | 12.5 | 4 |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 12 | 5 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 12.5 | 5 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 13 | 5 |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13 | 5.5 |

Table 7 is a modified version of Table 6.2.3.24-2 in TS 38.101-1

# Conclusion

Proposal 1: Consider the following A-MPR regions and values for NS_12 with 7MHz CBW.

Table 1: A-MPR regions for NS_12 for 7MHz CBW

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc ≤ 820.5 | ≤2.16 | >0 | A3 |

Table 2: A-MPR for NS_12

| Modulation/Waveform | A3 |
| --- | --- |
|  | Outer/Inner |
| DFT-s-OFDM QPSK | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 5 |
| DFT-s-OFDM 64 QAM | ≤ 5 |
| DFT-s-OFDM 256 QAM |  |
| CP-OFDM QPSK | ≤ 6.5 |
| CP-OFDM 16 QAM | ≤ 6.5 |
| CP-OFDM 64 QAM | ≤ 6.5 |
| CP-OFDM 256 QAM |  |
| NOTE: for 256QAM MPR would suffice and no A-MPR is needed. |  |

Proposal 2: Consider the following A-MPR regions and values for NS_13 with 7MHz CBW.

Table 3: A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | Fc = 820.5 MHz | ≤1.98 | >0 | A4 |

Table 4: A-MPR for NS_13

| Modulation/Waveform | A4 |
| --- | --- |
| DFT-s-OFDM QPSK | ≤ 4.5 |
| DFT-s-OFDM 16 QAM | ≤ 5 |
| DFT-s-OFDM 64 QAM | ≤ 5 |
| DFT-s-OFDM 256 QAM |  |
| CP-OFDM QPSK | ≤ 6.5 |
| CP-OFDM 16 QAM | ≤ 6.5 |
| CP-OFDM 64 QAM | ≤ 6.5 |
| CP-OFDM 256 QAM |  |
| NOTE: for 256QAM MPR would suffice and no A-MPR is needed. |  |

Proposal 3: No A-MPR for NS_14 with 7MHz CBW.

Proposal 4: Consider the following A-MPR regions and values for NS_15 with 7MHz CBW.

Table 6: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 845 < Fc ≤ 845.5 |  | ≥3.6 (Zone1) | A7 |
|  |  | ≥3.24, <6.3(Zone2)1 | ≥1.98, <3.6(Zone2) | A2 |
|  |  | ≥4.14, <6.3(Zone3) 2 | ≥1.44, <3.6(Zone3) | A1 |
|  |  | ≥4.5, <6.3(Zone4)3 | ≥0.9(Zone4) | A2 |
|  |  | ≥4.5 | <0.9(Zone5) | A2 |
|  | 838 < Fc ≤ 841.5 |  | ≥4.5 (Zone1) | A8 |
| NOTE 1: 1.26 ≤ (RBend- LCRB) *12*SCS (MHz)<2.7 NOTE 2: 2.7 ≤ (RBend- LCRB) *12*SCS (MHz)<3.6 NOTE 3: 3.6 ≤ (RBend- LCRB) *12*SCS (MHz) |  |  |  |  |

Table 7: A-MPR for NS_15

| Modulation/Waveform | A1 | A2 | A3 | A7 | A8 |
| --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 |  |  |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | 11.5 | 2 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | 12.5 | 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | 12.5 | 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | 12.5 | 4 |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 12 | 5 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 12.5 | 5 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 13 | 5 |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13 | 5.5 |

Table 7 is a modified version of Table 6.2.3.24-2 in TS 38.101-1

# Annex