3GPP TSG-RAN WG4 Meeting #115	R4-2505726

St Julian’s, Malta, May 19th –May 23rd, 2025

Source:	Nokia

Title:	On remaining FBS requirements

Agenda item:	7.18.3.1

Document for:	Discussion

Introduction

In this contribution we discuss the remaining open issues related to L3 beam sweeping factor reduction.

Discussion

In the last meeting, RAN4 sent an LS to RAN2 to trigger the signaling work related to triggering of fast beam sweeping (FBS). The details of the signaling are to be finalized by RAN2, and we do not see a need for RAN4 to continue the discussion about the triggering condition any further.

No need for RAN4 to discuss the details of FBS condition configuration further.

In the following, we discuss the issues that remained open in the last meeting and require further RAN4 discussions.

Conditions to exit FBS

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

- Recommended WF - Discuss the option(s) to finalize quitting conditions and re-evaluation mechanism. - Note: Option 1c/1d/3a/3b includes re-evaluation mechanism, which may be coupled with the quit conditions or not, they are collected as follow: - Option 1c: After quitting fast measurement, UE shall re-activate fast measurement no longer than T2 from the quitting time. - Option 1d: (after quitting), the UE makes another exam on the criteria that are configured for the serving cell quality. If the criteria are met, the UE continues to use reduced beam sweeping factor while the UE exits the state when the quality criteria are no longer met. - Option 3a: Introduce another timer, and UE re-evaluates the serving cell quality threshold condition after it expires. This timer starts to run when UE deactivates Rx BSF reduction feature. - Option 3b: After deactivating FBS, the UE may re-evaluate the condition of enabling FBS, at least after a minimal time interval ‘time to active’ after deactivated or disabled.

Regarding the exit condition for FBS, a logical condition to exit is that the condition to trigger FBS is no longer met.

If FBS condition is no longer met, UE is allowed to return to normal beam sweeping.

However, in some scenario it is possible that the UE remains at the cell edge and the condition to trigger FBS continues to be met for a long period of time. We are open to define some kind of timer to allow UE to exit FBS in such scenario. However, as has been proposed by some companies, it should be clarified under which conditions the UE shall re-start or re-evaluate whether to continue FBS despite timer expiration.

A timer can be introduced to prevent too long period of applying FBS if the UE stops at the cell edge. However, it should be clarified whether further conditions are needed regarding restarting or evaluating whether to continue FBS after timer expiration.

Status report of fast beam sweeping

Issue 1-1-3: Status report of fast beam sweeping

- Proposals - Option 1: (QC) - Network configures report for the status of fast beam sweeping factor application. - A TTT-like time window or N310-like timer, which starts running or counting upon the first satisfaction of the condition RSRP/RSRQ is observed by the UE, can be configured to avoid frequent status transitions and reports - The existing report defined for RLM/BFD relaxation status can be reused or served as a baseline - Option 2: (Ericsson) - NW is aware of UE enabling FBS, it may need the information regarding the UE's activation or entry into the FBS state. One potential solution is for the UE to report when it meets the FBS condition or it enables FBS. - UE shall notify NW of exiting FBS.

Regarding UE reporting to the network about whether FBS is applied or not, we do not see this signalling necessary. Mobility decisions are based on reports, not UE FBS status. The network can configure event-triggered reporting conditions to align well with the FBS condition and then the report will tell the network that the UE is at cell edge and uses or has been using FBS.

No need to introduce UE signalling about FBS status.

Applicability

Issue 1-2-1: Applicability of L3 measurement delay reduction by optimizing Rx BSF

- Proposals - Option 1: (CATT) - The reduced Rx BSF feature also applies to the associated SSB in CSI-RS based measurement, CHO and DAPS HO without extra specification updates. - Option 2: (Ericsson) - For CHO, UE always applies FBS during Tmeasure. - Option 3: (Nokia) - Introduce applicability of FBS requirements in section 3. - Option 4: (MTK) - RAN4 to confirm whether FBS L3 measurement delay reduction applies to Intra/inter-frequency measurements with NCSG scenarios.

Regarding applicability issues listed above, Option 3 is already included in the big CR agreed in the last meeting. Regarding option 2, we support this option, but based on the earlier agreements about limiting the triggering conditions to the configured FBS condition, this option may no longer be in the scope based on this agreement. However, if companies are ok to agree on this option, we are supportive for it.

If considered to be in the scope, FBS to apply when CHO is configured.

Conclusion

The following Observations and Proposals were made:

Observation 1: No need for RAN4 to discuss the details of FBS condition configuration further.

Proposal 1: If FBS condition is no longer met, UE is allowed to return to normal beam sweeping.

Proposal 2: A timer can be introduced to prevent too long period of applying FBS if the UE stops at the cell edge. However, it should be clarified whether further conditions are needed regarding restarting or evaluating whether to continue FBS after timer expiration.

Proposal 3: No need to introduce UE signalling about FBS status.

Proposal 4: If considered to be in the scope, FBS to apply when CHO is configured.