3GPP TSG-RAN WG4 Meeting #115	R4-2505728

Malta, May 19th – 23rd, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	On performance requirements of L3 measurement delay reduction

Agenda item:	7.18.5

Document for:	Discussion

Introduction

In this contribution we discuss the RRM performance requirements for FR2-1 SSB based L3 measurement delay reduction, considering both CSSF optimization and FBS reduction through multi-Rx.

Test cases for L3 beam sweeping factor reduction

In RAN4#114bis meeting, the following high level test case list was agreed:

| Index | Scenario | Need or not | Detailed configurations and scopes |
| --- | --- | --- | --- |
| 1 | Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 or 2 TCs | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) |
| 3 | Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter |  | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) |
| 2 | Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 or 2 TCs | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) |
| 4 | Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter |  | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) |
| 5 | Scenario 5: Handover | 1 or 2 TCs | TBA(e.g., Intra-frequency or inter-frequency, unknown) |
| 6 | Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection | 1 or 2 TCs | TBA(e.g., re-establishment or RRC redirection, Intra-frequency or inter-frequency, [or known] or unknown) |

It remained open whether to introduce 1 or 2 test cases i.e. intra- and inter-frequency, DRX or non-DRX test cases or only one of these for listed tests. We analyze the necessary test cases for each of the listed tests here.

Test cases for L3 measurements

RAN4 has agreed that the BSF reduction applies for intra- and inter-frequency measurements with and without measurement gaps. Hence, both requirements need to be tested. Although the reduction applies for both DRX and non-DRX, we think the most relevant scenario is the non-DRX scenario. Hence, it is ok from our side to define L3 measurement delay test cases only for non-DRX.

For FBS, define the following L3 measurement delay test cases for non-DRX:
- SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra
- SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter
- SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra
- SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

In the L3 measurement test cases, it shall be verified that the UE correctly performs FBS when the condition for FBS triggering is met. Since it is not forbidden for a UE to use FBS when the condition is not met, we do not see a need to include a time period when the condition is not met in the test case. UE behavior without FBS capability is already widely tested in the existing test cases. Furthermore, if the condition stops being met, UE is still allowed to continue with FBS, so there is no need to test that the UE would stop FBS.

In the L3 measurement test cases, simply test that the UE follows FBS when the condition for FBS is met. No need to test that the UE would stop FBS when the triggering condition is not met (or under any other condition) as the UE is allowed to do FBS also when the condition is not met.

Test cases for handover

The open question about handover test cases is whether to test intra- or inter-frequency handover or both of these. Intra-frequency handover is the most typical scenario, but for inter-frequency handover the delay reduction is more significant. We think for full test coverage, both intra- and inter-frequency handover to unknown target cell should be tested.

For FBS, define 1 test case for handover to unknown intra-frequency target cell and 1 test case for handover to unknown inter-frequency target cell.

Test cases for RRC re-establishment and RRC connection release with redirection

Beam sweeping factor reduction applies to both RRC re-establishment and RRC connection release with redirection. Hence, both procedures need test cases. Regarding the known/unknown split, beam sweeping factor reduction for RRC re-establishment only applies if the target carrier is unknown, and for RRC connection release with redirection there is not distinguishing between known or unknown target cell. Furthermore, for RRC connection release with redirection there is no distinguishing between intra- and inter-frequency target cell in the impacted requirement.

For FBS, define 1 intra- and 1 inter-frequency test case for RRC re-establishment to an unknown target cell.

For FBS, define 1 test case for RRC connection release with redirection.

Test cases for CSSF optimization

At RAN4#114bis meeting, the list of test cases was agreed as below. In this paper, we are discussing the detailed test configurations taking SSB-based intra-frequency measurement without gaps for solution 1 as an example.

In legacy test cases, the measurement delay is verified by configuring event triggered measurement reporting on a neighbour cell. By setting the neighbor cell on the same frequency or different frequency with the active cell, the intra-frequency or inter-frequency measurement delay can be tested e.g. in TS 38.133 clause A.7.6 for intra-frequency measurement on FR2 cell. In these test cases, single carrier frequency is assumed hence no CSSF is applied.

In legacy test cases, the measurement delay is verified by configuring event triggered measurement reporting on a neighbour cell and no CSSF is applied.

When UE is configured to measure one serving CC per FR2 band, the enhanced CSSF shall be derived by counting one serving CC per band. Hence a scenario with at least two carrier frequencies in one band is required to verify the measurement delay with enhanced CSSF. The test case shall include at least one PSCell/PCell, one SCell and a neighbor cell in the same band. Following the legacy test cases, the test case for enhanced CSSF can be designed by configuring event triggered measurement reporting on a neighbour cell on the same frequency with PSCell (in EN-DC) or PCell (in FR2 SA).

Issue 2-2-1: TC list for CSSF optimization

- TC for solution 1:

Agreement in online session:

- Define the test cases for solution 1 of CSSF enhancement following the table below:

| TC index | Scenario | Need or not | Detailed configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them | TBA | Nokia |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them | TBA | Ericsson |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them | TBA | ZTE |

A scenario with at least two carrier frequencies in one band is required to verify the measurement delay with enhanced CSSF.

The test cases for enhanced CSSF are designed by configuring event triggered measurement reporting on a neighbour cell on the same frequency with PSCell (in EN-DC) or PCell (in FR2 SA).

In addition, the existing test cases are defined considering both DRX and non-DRX cases. To simply the testing efforts, we can configure non-DRX in one scenario e.g. EN-DC and DRX in the other scenario e.g. SA. The test cases for SSB-based measurement without gaps are exampled in [R4-2506216].

For the test cases with enhanced CSSF, to configure non-DRX in one scenario e.g. EN-DC and DRX in the other scenario e.g. SA.

If single FR2 band is assumed above, UE will measure PSCell/PCell based on the predefined rule for selecting the serving carrier to be measured on one band. Besides, we further agreed network may indicate the carrier to be measured per band in some cases. A test case is needed to verify the measurement delay with enhanced CSSF in case network indicates the carrier to be measured per band.

As the network indicated carrier only applies when there is no PCC/PSCC on the band. The test case can be designed in FR1+FR2 or FR2 inter-band scenario. The event triggered measurement reporting can be configured to verify the measurement on a neighbor cell on the same frequency of one FR2 SCell.

To define additional test case to verify the measurement delay with enhanced CSSF in case network indicates the carrier to be measured per band.

In summary, for SSB-based measurement delay with enhanced CSSF, we may define three test cases as below:

- EN-DC event triggered reporting test without gap under non-DRX for UE configured with [MeasuringoneCCperFR2band]

- SA event triggered reporting test without gap under DRX for UE configured with [MeasuringoneCCperFR2band] in FR2 intra-band CA

- SA event triggered reporting test without gap under non-DRX for UE configured with [MeasuringoneCCperFR2band] in FR2 intra-band CA] in FR1+FR2 CA

To define three test cases for SSB-based measurement delay with enhanced CSSF (i.e. solution 1):

- EN-DC event triggered reporting test without gap under non-DRX for UE configured with [MeasuringoneCCperFR2band]

- SA event triggered reporting test without gap under DRX for UE configured with [MeasuringoneCCperFR2band] in FR2 intra-band CA

- SA event triggered reporting test without gap under non-DRX for UE configured with [MeasuringoneCCperFR2band] in FR2 intra-band CA] in FR1+FR2 CA

Conclusion

In this contribution we have made the following proposals and observations:

L3 beam sweeping factor reduction:

Proposal 1: For FBS, define the following L3 measurement delay test cases for non-DRX: 
- SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra 
- SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter 
- SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra 
- SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

Proposal 2: In the L3 measurement test cases, simply test that the UE follows FBS when the condition for FBS is met. No need to test that the UE would stop FBS when the triggering condition is not met (or under any other condition) as the UE is allowed to do FBS also when the condition is not met.

Proposal 3: For FBS, define 1 test case for handover to unknown intra-frequency target cell and 1 test case for handover to unknown inter-frequency target cell.

Proposal 4: For FBS, define 1 intra- and 1 inter-frequency test case for RRC re-establishment to an unknown target cell.

Proposal 5: For FBS, define 1 test case for RRC connection release with redirection.

CSSF optimization:

Observation 1: In legacy test cases, the measurement delay is verified by configuring event triggered measurement reporting on a neighbour cell and no CSSF is applied.

Proposal 6: A scenario with at least two carrier frequencies in one band is required to verify the measurement delay with enhanced CSSF.

Proposal 7: The test cases for enhanced CSSF are designed by configuring event triggered measurement reporting on a neighbour cell on the same frequency with PSCell (in EN-DC) or PCell (in FR2 SA).

Proposal 8: For the test cases with enhanced CSSF, to configure non-DRX in one scenario e.g. EN-DC and DRX in the other scenario e.g. SA.

Proposal 9: To define additional test case to verify the measurement delay with enhanced CSSF in case network indicates the carrier to be measured per band.

Proposal 10: To define three test cases for SSB-based measurement delay with enhanced CSSF (i.e. solution 1):