3GPP TSG-RAN WG4 Meeting #115	R4-2505731

St Julian’s, Malta, May 19th –May 23rd, 2025

Source:	Nokia

Title:	On remaining LTM L1 event-triggered reporting requirements

Agenda item:	7.29.3.1

Document for:	Discussion

Introduction

In this contribution we discuss the remaining open issues of event-triggered L1 reporting for LTM.

Discussion

Event-triggered reporting delay

In the last meeting RAN4 made the following high-level agreement about how to define the L1 event-triggered reporting delay:

Issue 2-2: Event triggered reporting delay for L1 reporting Agreement: - Reuse the framework from L3 event triggered reporting. Discuss the details further.

What the above agreement means based on the discussions and our understanding is that the delay will be defined taking into account the measurement delay up to the point the report is triggered, without defining a max-function between measurement period of serving and candidate cell measurement. It remained open how this understanding is to be captured in the specification.

As has been widely discussed in RAN4, the possible scenarios for triggering the report after the condition becomes met over the air are:

- UE has to perform a new measurement for both serving beam and candidate beam before realizing the condition is met. In this scenario, the evaluation delay is the time until the latter measurement (serving or candidate), which may be up to the measurement periodicity of serving or candidate beam, whichever is measured last.

- UE realizes the condition is met based on new serving cell measurement compared to candidate cell measurement that happened before the condition became met over the air. In this scenario, the evaluation delay is the time until the UE has performed the new serving cell measurement, which may be up to the serving cell measurement period.

- UE realizes the condition is met based on new candidate cell measurement compared to serving cell measurement that happened before the condition became met over the air. In this scenario, the evaluation delay is the time until the UE has performed the new candidate cell measurement, which may be up to the candidate cell measurement period.

In our view, the following wording captures all the scenarios 1-3:

The event triggered measurement reporting delay without the UL resource waiting time taken into account shall be no longer than the L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB of the measurement that triggers the report, where TL1-RSRP_Measurement_Period_SSB is the measurement period of serving or candidate cell depending on which measurement triggers the report.

We have captured the above in our CR with the correct specification references in R4-2505732.

Requirement applicability

In the last meeting, RAN4 made the following agreement regarding requirement applicability:

Issue 2-7: Impact on other LTM requirements Agreement: - For FR1: - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, - Case 1: leaving condition of the event is configured and corresponding condition is not met, - the related cell switch delay and TCI state activation requirements are applicable - Case 2: leaving condition of the event is NOT configured, FFS - For FR2: FFS

The case of UE being configured with a leave condition leading to on leave report at the fulfilment of such condition in FR1 is clear, while the scenario without leave condition is open. FR2 applicability is also still fully open.

We see no reason why the Case 1 agreement could not be applied to FR2. Hence, we propose to extend this agreement to FR2.

Apply to both FR1 and FR2: When UE has sent event-triggered L1 report and leave condition of the event is configured and UE has not sent report on leave before receiving the cell switch command, delay requirements are applicable.

Case 2 where leave condition is not configured was considered more difficult as there is no clear indication (on leave report) to the network that would indicate whether the reporting condition continues to be met. From network point of view, if it is not an issue for the UE to consider the requirements applicable without a further condition than UE having sent the event-triggered report, this condition is sufficient from our side. However, if this is seen a dealbreaker, we would propose at least to extend the requirement applicability from the existing 1280 ms to 5 seconds for both FR1 and FR2 based on event-triggered L1 report.

Either a) introduce no further conditions than UE having sent L1 event-triggered report and condition continuing to be met for the case without leave condition configured, or b) extend applicability from the existing e.g. to 5 seconds based on event-triggered L1 report.

We have made a text proposal for the above proposals in our CR R4-2505732, where regarding Proposal 5 we have given a text proposal for the time-based applicability (option b in proposal 3).

DL synchronization with event-triggered L1 reporting

When UE is configured with event-triggered L1 reporting, the UE may send an event-triggered L1 report, which may be followed by TCI state activation. Alternatively, TCI state activation may happen based on L3 report, and L1 report may follow after this, as has been brought up also by another company in the last meeting. Additionally, there may be a second L1 report triggered after TCI state activation, if cell switch command is not sent very shortly after TCI state activation command.

For the scenario where TCI state activation is followed by a L1 report, the UE may no longer have DL synchronization with the activated TCI state, as the current requirements allow the UE to stop keeping track of the DL sync after 160/480 ms, and event-triggered reporing condition may not become fulfilled within this time. When the UE sends an event-triggered report associated to an activated TCI state, we think it makes sense that the UE would renew the DL synchronization for this TCI state based on the report, if the UE does not have the DL sync already. This should be captured as an additional condition for Tfirst-RS and TRS-proc = 0 as:

Add the following additional condition for Tfirst-RS and TRS-proc = 0: The target TCI state is on the active TCI state list, and the UE has sent an event-triggered L1 report associated to the target TCI state no longer than 160 ms before reception of the cell switch command.

Capability related

A few open issues have been discussed regarding UE capability of number of events supported:

Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category Candidate options: - Option 1: Yes (Xiaomi, CMCC, ZTE, CTC, vivo) - Option 2: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple) Issue 2-4: whether to define a separate capability for supported number of events triggered L1 measurement reporting Candidate options: - Option 1: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM. (Xiaomi, CMCC, ZTE, E///, CTC) - Option 1a: Yes. RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS. (E///) - Option 2: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO) Issue 2-5: whether and how to coordinate with R19 MIMO Agreement: - RAN4 shall develop the two features independently at current stage. Further discussion on common capability is allowed when discussing UE feature list of R19.

For L3 event-triggered reporting, RAN4 has defined capabilities for the support of event-triggered reporting in section 9.1.4, where the UE shall be able to support in parallel per category up to Ecat reporting criteria according to table 9.1.4.2-1.

We are not aware that RAN1 would have had any discussions related to the number of supported events, so RAN4 should take care of this. The L1 events the UE shall be able to support should be added on top of the existing L3 events i.e. introduction of L1 events shall not impact the existing UE capabilities to support L3 events. Hence, we think RAN4 needs to define either a new table for L1 events to be supported or add new rows to the existing Ecat table to cover intra- and inter-frequency LTM events to be supported. The supported number of events has to be discussed, and both SSB and CSI-RS based events shall be included.

Introduce the number of supported LTM intra- and inter-frequency L1 events for SSB and CSI-RS based reporting on top of the existing L3 events in section 9.1.4.

Conclusion

In this contribution we have made the following proposals:

Proposal 1: The event triggered measurement reporting delay without the UL resource waiting time taken into account shall be no longer than the L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB of the measurement that triggers the report, where TL1-RSRP_Measurement_Period_SSB is the measurement period of serving or candidate cell depending on which measurement triggers the report.

Proposal 2: Apply to both FR1 and FR2: When UE has sent event-triggered L1 report and leave condition of the event is configured and UE has not sent report on leave before receiving the cell switch command, delay requirements are applicable.

Proposal 3: Either a) introduce no further conditions than UE having sent L1 event-triggered report and condition continuing to be met for the case without leave condition configured, or b) extend applicability from the existing e.g. to 5 seconds based on event-triggered L1 report.

Proposal 4: Add the following additional condition for Tfirst-RS and TRS-proc = 0: The target TCI state is on the active TCI state list, and the UE has sent an event-triggered L1 report associated to the target TCI state no longer than 160 ms before reception of the cell switch command.

Proposal 5: Introduce the number of supported LTM intra- and inter-frequency L1 events for SSB and CSI-RS based reporting on top of the existing L3 events in section 9.1.4.