3GPP TSG-WG RAN4 Meeting #115	R4-2505738

# Malta, MT, 19th – 23rd May, 2025

Document for:	Approval

Agenda item:	7.9.4

Source: 	Qualcomm Incorporated

Title: 	On UE RF requirements for 5G broadcast utilizing a geosynchronous satellite

Document for:	Approval

# 1. Introduction

In RAN#107, the new WID for new band for 5G Broadcast Utilizing Geosynchronous satellite was approved in [1] with the following objectives:

- Specify a new SDO band for LTE-based 5G Broadcast operating in 1467 to 1492 MHz in Region 3 for GSO: - Specify related UE RF core requirements for 10MHz PMCH - UE requirements specified in TS 36.101 for LTE-based 5G Broadcast shall be considered as baseline (reuse assumption of 10 MHz UE Rx filter) - Specify related SAN RF core requirements for 10 MHz PMCH considering also applicable regulatory requirements: - Take existing SAN RF requirements specified in TS 38.108 for 10MHz as baseline - Requirements based on Resolution 761 in WRC-19 - Only consider QPSK and 16QAM modulation Note: UE only needs to support 5G broadcast for this band Note: Release independence applicability will be further discussed

In RAN4#114bis, the issues for UE RF requirements were discussed and the agreements were captured in WF [2]. In this paper, we provide our views on the remaining issues.

# 2. UE RF requirements

## 2.1 SCS options

In LTE based 5G terrestrial broadcast, several SCS options including 15kHz SCS, 2.5kHz SCS, 1.25kHz SCS and 0.37kHz SCS were specified. For LTE based 5G broadcast over Satellite, 15kHz SCS could be set as the priority based on the discussions in RAN4#114bis and other SCS options can be further specified in future release. With that, we have the following proposal:

Proposal 1: To only specify 15kHz SCS for 10MHz PMCH for the new band for 5G Broadcast Utilizing Geosynchronous satellite in Rel-19.

## 2.2 Reference measurement channels

In [2], the agreements were reached for RMC definition since only QPSK and 16QAM modulation are considered in this WI [1]. Per the link budget for GEO, it is proposed to consider 16QAM 1/2 in RMC.

Proposal 2: The RMC of 15kHz SCS to be specified in TS 36.102 based on the following table:

Table 1 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS)

| Parameter | PMCH (15 kHz SCS) |  |  |
| --- | --- | --- | --- |
|  | Unit | Value |  |
| Reference channel |  | R.PMCH.1 | R.PMCH.2 |
| PMCH bandwidth | MHz | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 |
| Modulation |  | QPSK | 16QAM |
| Target Coding Rate |  | 1/3 | 1/2 |
| Information Bit Payload (Note 2) |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 3624 | 9912 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 2 |
| Binary Channel Bits Per Subframe |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 10200 | 20400 |
| MBMS UE Category |  | ≥ 1 | ≥ 1 |
| Note 1: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |

## 2.3 Out-of-band blocking

In [2], the frequency range for out of band blocking was agreed as below to accommodate band 50 UE filter design.

Table 7.6C.3-2: Out of band blocking

| Operating band | 1 | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | FInterferer (CW) | MHz | FDL_low -40 to FDL_low -85 | FDL_low -85 to FDL_low -110 | FDL_low -110 to 1 MHz |
|  |  |  | FDL_high +30 to FDL_high + 75 | FDL_high +75 to FDL_high +100 | FDL_high +100 to +12750 MHz |
| 246 | PInterferer | dBm | -44 | -30 | -15 |

The frequency range is calculated assuming that Range 1 starts from -15MHz below or 15MHz above frequency range of B50. Another approach defining the frequency range of Out of band blocking is to start from -15MHz below or 15MHz above frequency range of B246. Two options are shown in Figure 1 below.

Figure 1: Two options for frequency range of out of band blocking requirements

Proposal 3: RAN4 to decide which option should be used to define the frequency range of out of band blocking requirements to accommodate band 50 filter design.

- Option 1: Reuse legacy frequency range of band 50 out of band blocking requirements

- Option 2: Define the Rang 1 starting from 15MHz below and above frequency range of band 246

# 3. Release independent

As stated in [1], the release independence applicability will be further discussed in RAN4. It is noted that band 107 and 108 were introduced in Rel-18 for LTE based 5G terrestrial broadcast but it can be release independent from Rel-17 since LTE based 5G broadcast feature was specified in Rel-17 in RAN1/2. Additionally, in RAN4#114, it was agreed the sub-bands of band 108 will be specified in a release independent manner from Rel-17 [3].

For NR NTN and IoT NTN, the bands and corresponding requirements were introduced in Rel-17 and Rel-18 respectively in RAN4. While similar as LTE based 5G broadcast, IoT NTN bands are also specified in a release independent manner from Rel-17 [4].

The SDO band introduced in this WI is for LTE based 5G broadcast over satellite. There is not any additional RAN1/2 impact compared to Rel-17 LTE based 5G broadcast feature. Therefore, we have the following proposal:

Proposal 4: The new proposed band for LTE based 5G broadcast utilizing satellite is specified in a release independent manner from Rel-17.

# 4. Conclusion

In this paper, the following proposals are made:

Proposal 1: To only specify 15kHz SCS for 10MHz PMCH for the new band for 5G Broadcast Utilizing Geosynchronous satellite in Rel-19.

Proposal 2: The RMC of 15kHz SCS to be specified in TS 36.102 based on the following Table 1:

Table 1 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS)

| Parameter | PMCH (15 kHz SCS) |  |  |
| --- | --- | --- | --- |
|  | Unit | Value |  |
| Reference channel |  | R.PMCH.1 | R.PMCH.2 |
| PMCH bandwidth | MHz | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 |
| Modulation |  | QPSK | 16QAM |
| Target Coding Rate |  | 1/3 | 1/2 |
| Information Bit Payload (Note 2) |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 3624 | 9912 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 2 |
| Binary Channel Bits Per Subframe |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 10200 | 20400 |
| MBMS UE Category |  | ≥ 1 | ≥ 1 |
| Note 1: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |

Proposal 3: RAN4 to decide which option should be used to define the frequency range of out of band blocking requirements to accommodate band 50 filter design.

- Option 1: Reuse legacy frequency range of band 50 out of band blocking requirements

- Option 2: Define the Rang 1 starting from 15MHz below and above frequency range of band 246

Proposal 4: The new proposed band for LTE based 5G broadcast utilizing satellite is specified in a release independent manner from Rel-17.

# 5. References

- RP-250788, “New WID: New band for 5G Broadcast Utilizing Geosynchronous Satellite”, Astrum Mobile,    Qualcomm, RAN#107, March, 2025

- R4-2504725, “Way Forward for [114bis][318] 5G_Broadcast_GSO_NTN_band”, Astrum Mobile, Qualcomm, ZTE, Huawei

- R4-2502376, “WF on 5G Broadcast bands”, EB

- TS 36.307 v18.0.0