3GPP TSG-RAN WG4 Meeting # 115	R4-2505747

Malta, MT, 19th – 23rd May 2025

Agenda Item:	7.11.3

Source: 		Qualcomm Incorporated

Title: 	Coexistence simulation results between TN and NTN for Ku band

Document for:	Approval

# Introduction

In RAN4#114-bis, NTN-TN co-existence simulation assumptions for Ku band were discussed and RAN4 agreed to consider the simulation assumptions in [1]. In this paper, we provide our coexistence simulation results between NTN and TN and vice versa for Ku band.

# 2. Discussion

In this document we provide co-existence simulation results between TN and NTN and vice versa for Ku band for the scenarios highlighted in Table 1 and as agreed in [1].

Table 1: Simulated coexistence scenarios

| No. | Combination | Aggressor | Victim | Frequency band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | 14 GHz | ACLR NTN UE to be varied/defined. ACS TN gNB fixed |
| 2 | TN with NTN | NTN DL | TN DL | 11 GHz | ACLR NTN SAN to be varied/defined. ACS TN UE fixed |
| 3 | TN with NTN | TN DL | NTN DL | 11 GHz | ACLR TN gNB fixed. ACS NTN UE to be varied/defined |
| 4 | TN with NTN | TN DL | NTN UL | 14 GHz | ACLR TN gNB fixed. ACS NTN SAN to be varied/defined |

In the coexistence results below we have used fixed VSAT at 1.5 m height for all the cases which can be considered as worst-case scenario. Satellite elevation angles of 35 and 90 degrees for the two satellites in Table 2 are used. Two types of VSAT antennas are considered as highlighted in Table 2.

FRF 2 is used for all the cases as agreed in [1]. Furthermore, flat ACIR model is considered for all DL and UL simulations. The number of TN UEs is 1 UE in both DL and UL. The number of NTN UEs is 1 UE in DL and 10 UEs in UL.

Table 2: Simulation Matrix

| VSAT model SAN | 45 x 45 Phased Array (1st step for coexistence) | 60 cm Parabolic (1st step for coexistence) |
| --- | --- | --- |
| GEO | X | X |
| LEO 600km | X | X |

In the following, the simulation results will show the average and cell edge throughput loss as a function of the ACIR.

- Scenario 1 (NTN UL to TN UL)

The simulation results for scenario 1 for both elevation angles 35 and 90 degrees are shown in Table 3 considering the VSAT at 1.5 m. From the results we can see that the effect from the VSAT UEs to the TN BS is not very high and the ACIR requirements are about 20 dB under worst case assumptions which is 35 degrees elevation angle. For elevation angle 90 degrees, the VSAT is directed towards the sky and the sidelobes towards the BS are very low so lower ACIR is observed with about 5 dB less. It can be seen also that the throughput loss is decreasing with lower altitudes because the VSAT UE will use less TX power.

Table 3: Required ACIR for Scenario 1 considering VSAT Parabolic antenna.

|  |  | ACIR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 5 | 10 | 15 | 20 | 25 |
| GEO @90 deg | Average THP-Loss [%] | 2.4 | 1.5 | 0.9 | 0.5 | 0.33 | 0.18 |
|  | Cell edge THP-Loss [%] | 16.9 | 12.1 | 6.3 | 3.1 | 1.1 | 0.7 |
| GEO @35 deg | Average THP-Loss [%] | 5.1 | 3.4 | 2.2 | 1.4 | 0.9 | 0.5 |
|  | Cell edge THP-Loss [%] | 38 | 24.3 | 15.7 | 9.2 | 5.5 | 3 |
| LEO 600 @90 deg | Average THP-Loss [%] | 0.19 | 0.09 | 0.03 | 0.01 | 0.004 | 0.001 |
|  | Cell edge THP-Loss [%] | 0.88 | 0.57 | 0.08 | 0 | 0 | 0 |
| LEO 600 @35 deg | Average THP-Loss [%] | 3.2 | 2.2 | 1.5 | 0.97 | 0.6 | 0.35 |
|  | Cell edge THP-Loss [%] | 22.6 | 15.7 | 11.4 | 6.6 | 3.1 | 1.1 |

Observation 1: The ACIR values for scenario 1 for VSAT under worst case assumption with 35 degrees of elevation can be 20 dB and the BS ACS of 30 dB can be respected. Hence the ACLR of the NTN UE VSAT can be defined with 20 dB with no problem for the Parabolic antenna case.

The simulation results for scenario 1 for both elevation angles 35 and 90 degrees are shown in Table 4 considering the VSAT at 1.5 m and phased array antenna with 45x45. From the results we can see that the effect from the VSAT using phased array antenna to the TN BS is worst compared to VSAT using parabolic antenna and stringent ACIR requirements are needed which is about 25 dB under worst case assumptions which is 35 degrees elevation angle. The effect of the sidelobes from the phased array VSAT antenna towards the TN BS is much higher compared to VSAT parabolic case, due to higher sidelobes as can be seen in Annex1.

Table 4: Required ACIR for Scenario 1 considering VSAT Phased array antenna.

|  |  | ACIR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 5 | 10 | 15 | 20 | 25 |
| GEO @90 deg | Average THP-Loss [%] | 6.13 | 4.1 | 2.7 | 1.7 | 1.1 | 0.6 |
|  | Cell edge THP-Loss [%] | 42 | 28.2 | 19.1 | 12.9 | 7.3 | 3.4 |
| GEO @35 deg | Average THP-Loss [%] | 7.6 | 5.4 | 3.7 | 2.5 | 1.6 | 1.1 |
|  | Cell edge THP-Loss [%] | 62.7 | 42.9 | 26.9 | 16.8 | 11.6 | 6.6 |
| LEO 600 @90 deg | Average THP-Loss [%] | 0.65 | 0.38 | 0.2 | 0.09 | 0.04 | 0.01 |
|  | Cell edge THP-Loss [%] | 3.1 | 2 | 0.95 | 0.57 | 0.08 | 0 |
| LEO 600 @35 deg | Average THP-Loss [%] | 5.1 | 3.6 | 2.5 | 1.7 | 1.1 | 0.7 |
|  | Cell edge THP-Loss [%] | 41.3 | 26.8 | 17.7 | 12.3 | 7.2 | 4.4 |

Observation 2: The ACIR values for scenario 1 for VSAT with phased array antenna under worst case assumption with 35 degrees of elevation can be 25 dB and the BS ACS of 30 dB can still be respected.

Proposal 1: Based on the observations 1, and 2, the phased array antenna case should consider stringent requirements compared to parabolic antenna case.

Proposal 2: Based on the observations 1, and 2, the ACLR for NTN UE VSAT is 20 dB and 25 dB for parabolic and phased array antenna respectively.

- Scenario 2 (NTN DL to TN DL)

The simulation results for scenario 2 for both elevation 35 and 90 degrees are shown in Table 5 considering VSAT phased array and parabolic antennas. From the results we can see that the effect from the satellite to the TN UE is negligible because of the very good signal from the BS compared to the satellite interference.

Table 5: Required ACIR for Scenario 2 considering VSAT phased array and parabolic antennas.

| For elevation 35 and 90 degrees |  | ACIR |
| --- | --- | --- |
|  |  | 0 |
| GEO | Average THP-Loss [%] | Less than 0.1 |
|  | Cell edge THP-Loss [%] | Less than 0.1 |
| LEO 600 | Average THP-Loss [%] | Less than 0.1 |
|  | Cell edge THP-Loss [%] | Less than 0.1 |

Observation 3: The interference from NTN DL to TN DL for scenario 2 is negligible for all the satellite altitudes and at different elevation angles. The TN UE ACS of 24 dB can be respected. Hence the ACLR of the NTN Satellite can be defined with less value than 24 dB with no problem or based on other scenarios.

Proposal 3: Based on observation 3, the ACLR of the satellite can be defined with value less than 24 dB with no problem.

- Scenario 3 (TN DL to NTN DL)

The simulation results for scenario 3 for both elevation angles 35 and 90 degrees are shown in Table 6. From the results we can see that the effect from the TN BS to the VSAT UE is very high because the antenna height of the TN BS is higher than VSAT. Furthermore, decreasing the elevation angle to 35 degree show more worse results. The “NaN” in the cell edge throughput for 35-degree elevation angle represents out of coverage and that even without TN interference, there is no link and 50% or more of the NTN UEs can’t close the link budget in the DL.

Table 6: Required ACIR for Scenario 5 considering VSAT Parabolic antenna.

|  |  | ACIR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 | 20 | 25 | 30 | 35 | 40 |
| GEO @90 deg | Average THP-Loss [%] | 28.2 | 18.1 | 11.1 | 6.4 | 3.3 | 1.5 |
|  | Cell edge THP-Loss [%] | 95.6 | 82.8 | 62.4 | 38.8 | 5.7 | 3.9 |
| GEO @35 deg | Average THP-Loss [%] | 46.3 | 33.5 | 23 | 14.7 | 8.5 | 4.4 |
|  | Cell edge THP-Loss [%] | NAN | NAN | NAN | NAN | NAN | NAN |
| LEO 600 @90 deg | Average THP-Loss [%] | 29.2 | 18.8 | 11.4 | 6.6 | 3.4 | 1.5 |
|  | Cell edge THP-Loss [%] | 95.7 | 83.9 | 66.3 | 42.6 | 9.2 | 6 |
| LEO 600 @35 deg | Average THP-Loss [%] | 50 | 36.8 | 25.9 | 17.3 | 10.5 | 5.8 |
|  | Cell edge THP-Loss [%] | NAN | NAN | NAN | NAN | NAN | NAN |

Observation 4: The ACIR value for scenario 3 is 40 dB considering different elevation angles for VSAT using parabolic antenna which is exceeding the TN BS ACLR of 37 dB. Hence this scenario will be problematic without using a coordination distance between the BS and the VSAT or using a frequency guard band between the NTN and TN operation. That will need further discussions. Furthermore, the system parameters for the elevation angle at 35 degrees should be revised either by increasing the satellite TX power or decreasing the RBs to avoid the out of coverage.

Proposal 4: Based on observation 4, in case that cell edge throughput is “NaN” due to the out of coverage, RAN4 can conclude based on average throughput loss.

The simulation results for scenario 5 for both elevation angles 35 and 90 degrees are shown in Table 7 considering the VSAT phased array antenna. From the results we can see that the effect from the TN BS to the VSAT with phased array antenna is worse than the VSAT with parabolic antenna case with about 5 dB more stringent ACIR.

Table 7: Required ACIR for Scenario 5 considering VSAT Phased array antenna.

|  |  | ACIR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| GEO @90 deg | Average THP-Loss [%] | 41.8 | 27.9 | 17.6 | 10.6 | 6 | 3.1 | 1.3 |
|  | Cell edge THP-Loss [%] | 100 | 100 | 82.9 | 56.6 | 28 | 5.5 | 0.6 |
| GEO @35 deg | Average THP-Loss [%] | 50.9 | 37.9 | 27.4 | 19.1 | 12.6 | 8 | 4.8 |
|  | Cell edge THP-Loss [%] | NAN | NAN | NAN | NAN | NAN | NAN | NAN |
| LEO 600 @90 deg | Average THP-Loss [%] | 43.5 | 29.2 | 18.4 | 11.2 | 6.4 | 3.3 | 1.4 |
|  | Cell edge THP-Loss [%] | 100 | 100 | 85.2 | 63.7 | 30.6 | 10 | 2.8 |
| LEO 600 @35 deg | Average THP-Loss [%] | 54 | 40.5 | 29.1 | 20.2 | 13.1 | 8.5 | 5.2 |
|  | Cell edge THP-Loss [%] | NAN | NAN | NAN | NAN | NAN | NAN | NAN |

Observation 5: The ACIR value for scenario 3 is 45 dB considering different elevation angles for VSAT using phased array antenna which is exceeding the TN BS ACLR of 37 dB. Similarly, to VSAT with parabolic antenna, either coordination distance needed or a frequency guard band between the NTN and TN operation bands needed. That will need further discussions. Furthermore, the system parameters for the elevation angle at 35 degrees should be revised either by increasing the satellite TX power or decreasing the RBs to avoid the out of coverage.

Proposal 5: Based on observation 4 and 5, the ACS of NTN UE for both types of VSAT antennas is difficult to be defined because the ACIR values are between 40 and 45 dB. RAN4 needs to study how to ensure coexistence between TN and NTN.

- Scenario 4 (TN DL to NTN UL)

The simulation results for scenario 4 for both elevation angles 35 and 90 degrees are shown in Table 8 and Table 9 considering VSAT with parabolic antenna and phased array antenna respectively. From the results we can see that the effect from the TN BS to the satellite is not very high for the elevation angle of 90 degrees for all the satellite altitudes. However, the effect increases with decreasing the elevation angle and stringent ACIR values are required up to 30 dB.

Table 8: Required ACIR for Scenario 4 considering VSAT Parabolic antenna.

|  |  | ACIR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 |
| GEO @90 deg | Average THP-Loss [%] | 6.2 | 2.5 | 0.9 | 0.3 | 0.09 | 0.03 |
|  | Cell edge THP-Loss [%] | 14 | 6.4 | 3.1 | 0.7 | 0.5 | 0.16 |
| GEO @35 deg | Average THP-Loss [%] | 20 | 11.6 | 6.3 | 3 | 1.3 | 0.4 |
|  | Cell edge THP-Loss [%] | NaN | NaN | NaN | NaN | NaN | NaN |
| LEO 600 @90 deg | Average THP-Loss [%] | 18.9 | 9.5 | 4.2 | 1.6 | 0.5 | 0.18 |
|  | Cell edge THP-Loss [%] | 66.9 | 44.3 | 23.4 | 10.3 | 3.7 | 1.2 |
| LEO 600 @35 deg | Average THP-Loss [%] | 35.1 | 22.1 | 13 | 7.1 | 3.5 | 1.5 |
|  | Cell edge THP-Loss [%] | 100 | 100 | 52.2 | 23.2 | 15.9 | 4.2 |

Table 9: Required ACIR for Scenario 4 considering VSAT Phased array antenna.

|  |  | ACIR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 |
| GEO @90 deg | Average THP-Loss [%] | 6.3 | 2.5 | 0.9 | 0.3 | 0.09 | 0.03 |
|  | Cell edge THP-Loss [%] | 14 | 6.4 | 3.1 | 0.7 | 0.5 | 0.16 |
| GEO @35 deg | Average THP-Loss [%] | 22.7 | 13.3 | 7.4 | 3.7 | 1.6 | 0.6 |
|  | Cell edge THP-Loss [%] | NaN | NaN | NaN | NaN | NaN | NaN |
| LEO 600 @90 deg | Average THP-Loss [%] | 18.9 | 9.5 | 4.2 | 1.6 | 0.5 | 0.18 |
|  | Cell edge THP-Loss [%] | 66.9 | 44.3 | 23.4 | 10.3 | 3.7 | 1.2 |
| LEO 600 @35 deg | Average THP-Loss [%] | 34.6 | 21.8 | 12.8 | 7 | 3.4 | 1.5 |
|  | Cell edge THP-Loss [%] | NaN | NaN | NaN | NaN | NaN | NaN |

Observation 6: The ACIR values for scenario 4 are between 15 to 30 dB for different satellite altitudes and different elevation angles. The TN BS ACLR of 30 dB can be respected. Hence the ACS of the satellite can be defined with 30 dB with no problem. Furthermore, the system parameters for the elevation angle at 35 degrees should be revised to avoid the out of coverage for UL with GEO.  Proposal 6: The system parameters for lower elevation angles at 35 degrees need to be revised either by increasing the satellite or NTN UEs TX power or decreasing the number of RBs because most of NTN UEs in DL or UL will be out of coverage even without adjacent interference from TN.

Proposal 7: Based on observation 6, the ACS of the satellite can be defined with 30 dB.

# 3.	Conclusion

In this paper, we provided a coexistence simulation result between TN and NTN and vice versa for both elevation angles of 35 and 90 degrees considering both NTN VSAT parabolic and phases array antennas. We have made the following observations as shown below:

Observation 1: The ACIR values for scenario 1 for VSAT under worst case assumption with 35 degrees of elevation can be 20 dB and the BS ACS of 30 dB can be respected. Hence the ACLR of the NTN UE VSAT can be defined with 20 dB with no problem for the Parabolic antenna case.

Observation 2: The ACIR values for scenario 1 for VSAT with phased array antenna under worst case assumption with 35 degrees of elevation can be 25 dB and the BS ACS of 30 dB can still be respected.

Observation 3: The ACIR values for scenario 2 are 0 dB for all the satellite altitudes and at different elevation angles. The TN UE ACS of 24 dB can be respected. Hence the ACLR of the NTN Satellite can be defined with less value than 24 dB with no problem or based on other scenarios.

Observation 4: The ACIR value for scenario 3 is 40 dB considering different elevation angles for VSAT using parabolic antenna which is exceeding the TN BS ACLR of 37 dB. Hence this scenario will be problematic without using a coordination distance between the BS and the VSAT or using a frequency guard band between the NTN and TN operation. That will need further discussions. Furthermore, the system parameters for the elevation angle at 35 degrees should be revised either by increasing the satellite TX power or decreasing the RBs to avoid the out of coverage.

Observation 5: The ACIR value for scenario 3 is 45 dB considering different elevation angles for VSAT using phased array antenna which is exceeding the TN BS ACLR of 37 dB. Similarly, to VSAT with parabolic antenna, either coordination distance needed or a frequency guard band between the NTN and TN operation bands needed. That will need further discussions. Furthermore, the system parameters for the elevation angle at 35 degrees should be revised either by increasing the satellite TX power or decreasing the RBs to avoid the out of coverage.

Observation 6: The ACIR values for scenario 4 are between 15 to 30 dB for different satellite altitudes and different elevation angles. The TN BS ACLR of 30 dB can be respected. Hence the ACS of the satellite can be defined with 30 dB with no problem. Furthermore, the system parameters for the elevation angle at 35 degrees should be revised to avoid the out of coverage for UL with GEO.

Based on the above observations we have the following proposals:

Proposal 1: Based on the observations 1, and 2, the phased array antenna case should consider stringent requirements compared to parabolic antenna case.

Proposal 2: Based on the observations 1, and 2, the ACLR for NTN UE VSAT is 20 dB and 25 dB for parabolic and phased array antenna respectively.

Proposal 3: Based on observation 3, the ACLR of the satellite can be defined with value less than 24 dB.

Proposal 4: Based on observation 4, in case that cell edge throughput is “NaN” due to the out of coverage, RAN4 can conclude based on average throughput loss.

Proposal 5: Based on observation 4 and 5, the ACS of NTN UE for both types of VSAT antennas is difficult to be defined because the ACIR values are between 40 and 45 dB. RAN4 needs to study how to ensure coexistence between TN and NTN.

Proposal 6: The system parameters for lower elevation angles at 35 degrees need to be revised either by increasing the satellite or NTN UEs TX power or decreasing the number of RBs because most of NTN UEs in DL or UL will be out of coverage even without adjacent interference from TN.

Proposal 7: Based on observation 6, the ACS of the satellite can be defined with 30 dB.

# 4.	References

- R4-2504723, Way Forward for [114bis][313] NR_NTN_Ku_Band_Coexistence, Moderator Intelsat

# 5. Annex 1

In Figure 1 below, the gain is plotted as a function of the elevation angle for both the phased array antenna and the parabolic antenna. For the phased array antenna, the 45x45 configuration is used with 0 deg direction/pointing to the sky and compared with the parabolic antenna with 60 cm diameter. For both cases, the maximum EIRP is 76.6 dBm however the sidelobes in the phased array case is much stronger. That explains why with phased array case, the results are more stringent in some cases as highlighted in the results section above.

Figure 1: VSAT Parabolic antenna vs Phased array antenna pattern