3GPP TSG-RAN4 Meeting #115	R4-2505748

Malta, Malta, 19th May 2025 - 23rd May 2025

Agenda item:		6.5

Source:		Union International Chemins de Fer (UIC)

Title:		PC2 and PC1.5 in bands n100 and n101

Release:		19

Work Item:	HPUE_NR_FR1_bands_R19-Core

Document for:		Discussion

# Introduction

This contribution discusses the railway FRMCS needs to support PC2 and PC1.5 HPUE power classes in bands n100 and n101 for FRMCS cab-radios, i.e. user equipment installed on trains with external antenna on top of the train roof at approximately 4m over ground, in addition to the already supported PC1.

# Discussion

Railway operators need to have the ability with FRMCS cab-radios, to deploy CA/DC using both n100 and n101, as well as UL MIMO in n100 and n101 to achieve improved data throughput at desired locations. Currently in RAN4 both CA/DC and UL MIMO are being defined with WID NR channel BW less than 5MHz for FR1 Phase 2 [1], respectively WID NR_bands_xFeature_R19-Core [2], for PC3 transmit power in bands n100 and n101.

To extend the coverage area where FRMCS cab-radios can beneficially make use of CA/DC and UL MIMO it is necessary to enable the use of CA/DC and UL MIMO in combination with a HPUE power class in bands n100 and n101.

Observation 1: railway operators have a need for CA/DC and UL MIMO with HPUE support in bands n100 and n101 for FRMCS cab-radios, i.e. user equipment installed on trains with external antenna on top of the train roof at approximately 4m over ground.

For the railway FRMCS frequency bands n100 and n101, single carrier PC1 transmit power has been introduced with WID LTE_NR_HPUE_FWVM_REL18 [3] for FRMCS cab-radios.

UIC understands that, due to constraints within the current 3GPP specifications, it is not possible to support CA/DC or UL MIMO using PC1 in both bands n100 and n101. However, specifying the use of CA/DC and UL MIMO with PC2 in FDD band n100, and PC2 and PC1.5 in TDD band n101 should be possible. A necessary first step for this is to define support of single carrier PC2 in band n100, and PC2 and PC1.5 in band n101.

Observation 2: Current 3GPP specifications do not allow specifying the use of PC1 for CA/DC and UL MIMO in bands n100 and n101.

Observation 3: To enable specifying support of CA/DC and UL MIMO with PC2 and PC1.5 HPUE, a necessary first step is to define for FRMCS cab-radios support of single carrier PC2 in FDD band n100, and PC2 and PC1.5 in TDD band n101.

Within the current RAN4 work the existing RP-241932 HPUE_NR_FR1_bands_R19-Core [4] addresses PC2 and PC1.5 HPUE. Its stated objectives are:

- Objective 1.  For TDD band: PC1.5 and PC2 for handheld UE and FWA, and PC1 for FWVM

- Objective 2.  For FDD band: PC2 for handheld UE and FWA, and PC1 for FWVM

From these objectives it can be seen that PC2 and PC1.5 power classes in bands n100 and n101 could be supported by this WID [4]. However, the FRMCS cabradio is considered to be a FWVM device in Ref. [3], whereas the current WID [4] only covers handheld UE and FWA for PC2 and PC1.5.

Observation 4: The current RP-241932 HPUE_NR_FR1_bands_R19-Core [4] could support the railway FRMCS needs for single carrier support of PC2 in FDD band n100 and PC2 and PC1.5 in TDD band n101. However, the current WID [4] blocks this for the FRMCS cabradio due to the lack of support of FWVM devices for these power classes.

# Conclusion

Proposal 1: Based on the above observations, it is proposed to extend WID HPUE_NR_FR1_bands_R19-Core [4] with support of PC2 for FDD bands and PC2 and PC1.5 for TDD bands for FWVM.

Proposal 2: Based on the above observations, it is proposed to add the support of PC2 in FDD band n100 and PC2 and PC1.5 in TDD band n101 to WID HPUE_NR_bands_xFeature_R19-Core [4].

# References

[1]	RP-240832 NR channel BW less than 5MHz for FR1 Phase 2

[2]	RP-242572 NR_bands_xFeature_R19-Core

[3]	RP-223424 LTE_NR_HPUE_FWVM_REL18

[4] 	RP-241932 HPUE_NR_FR1_bands_R19-Core