3GPP TSG-RAN WG4 Meeting #115																			R4-2505755

Malta, MT, 19th – 23rd May, 2025

Source: 	Xiaomi

Title: 	Discussion on AIoT device 1 RF requirements

Agenda item:	7.26.3.3

Document for:	Discussion

- Introduction

This contribution provides our further analysis for AIoT device 1 RF requirements based on the last RAN4 meeting’s progress [1].

- Discussion

2.1 Transmit output power

In the last RAN4 meeting, it was agreed that backscattering loss is used to define the transmit output power. For the input CW power, our understanding is that a maximum input CW power can be used to define device transmit output power. CW output power is agreed to be <=33dBm. Using the minimum 1m distance between reader and device, FSPL for 1m distance of 900MHz is ~33 dB. To have some margin, the maximum input power to device can be assumed approximately as 0 dBm.

Proposal 1: The maximum input CW power to calculate device 1 radiated power can be assumed as 0 dBm.

For the backscattering loss, although there were some assumptions in the co-existence simulation and RAN1 link budget analysis, it needs more inputs from the implementation products. There’s another aspect needs to be discussed is that how to test the output power, EIRP or TRP. Our understanding is that EIRP is more appropriate. If EIRP is used, spherical coverage may need to be discussed. We’re aware that it’s important that spherical coverage is important but may not be good to test device 1 like FR2 smart phone. FR2 BS approach may be a candidate that declaration can be considered.

Proposal 2: EIRP is used as the metric to test transmit output power. FR2 BS EIRP range declaration approach can be considered.

2.2 SFO requirement

SFO is agreed to be 10%, it is FFS if it should be defined and tested. Our original thinking was that SFO performance is included in the modulation quality requirement, but after more analysis, it should be defined that SFO is corrected in the demodulator so only residual SFO performance is included in the modulation quality requirements. For the test method, our understanding is that it’s not very difficult, it can be tested for the timing of Tx signal and can be left to the performance test stage.

Proposal 3: Define SFO requirement for device 1.

2.3 Modulation quality requirements

For the modulation quality requirement, the metric is not decided. EVM is usually used for Tx signal quality, so our thinking is that it still can be a candidate.

Proposal 4: EVM can be a candidate metric for modulation quality requirements.

The EVM requirement should be different for BPSK and QPSK. EVM can be derived from SNR using the following equation,

EVM = 1/sqrt(SNR)

Tx signal should transmit a good SNR signal to guarantee the SNR at receiver side. After the signal is traveled in the channel and combined at receiver side, Tx noise contribution should be at a relatively low level compared with receiver NF. The situation may be different for large signal and small signal. If it’s large signal, Tx SNR is large enough that there’s no DEMOD problem. If it’s small signal, receiver NF should be the dominate noise. If the EVM requirement is the same for maximum and minimum output signal, minimum output signal SNR should be taken as reference. If Tx signal’s SNR is 10 dB higher than the receiver demod threshold, Tx noise contribution to the final SNR is negligible. So we have the following proposal

Proposal 5: Device 1 Tx signal SNR is defined [10] dB higher than receiver DEMOD SNR threshold.

For the small frequency shift part, we’re not sure if it belongs to modulation quality requirement. But of course EVM can be measured for small frequency shift mode if it needs to be tested.

Observation 1: EVM can be tested for small frequency shift mode if it needs to be tested.

2.4 Emission requirements

It was agreed that reuse legacy NR UE spurious emission requirements, EIRP or TRP metric should be discussed. From the regulation aspect, our understanding is that TRP is the correct metric. But considering the test efforts for TRP, some simplified test method can be used.

Proposal 6: TRP is used for the emission requirements, some simplified test method can be used.

For the OOB boundary, it is related to what’s the reference for the boundary, the CW frequency or the TBW center. There’s another issue needs to be discussed that if SFS is used, if the emission on the frequency range between the 2SB TBWs should be tested. Our understanding is that it may not be needed.

Proposal 7: Emission requirement is only tested for no small frequency shift mode.

If the above proposal can be agreed, then the OOB boundary should be offset to the CW frequency point. The exact boundary is related to the output power.

2.5 Reference sensitivity

It was agreed that the device REFSENS is calculated based on the link budget with 15m coverage target. Using the parameters assumed in TR 38.869.

| Total Tx Power (dBm) | Ambient IoT on-object antenna penalty (dB) | Shadow fading margin (dB) | polarization mismatching loss (dB) | Frequency (GHz) | Coverage (m) | FSPL (dB) | Receiver Sensitivity (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 38 | 0.9 | 4 | 3 | 0.9 | 10 | 53.3 | -23.2 |
| 38 | 0.9 | 4 | 3 | 0.9 | 15 | 56.9 | -26.8 |
| 38 | 0.9 | 4 | 3 | 0.9 | 20 | 59.4 | -29.3 |

It can be seen with 38 dBm reader output power which is greater than what was assumed in SI phase, the coverage is increased with the same device 1 REFSENS performance. Considering some implementation tradeoff, the device 1 REFSENS can be defined with the minimum requirement [-23 ~ -30] dBm. Some input from the tag

Observation 2: Device REFSENS can be defined with the minimum requirement [-23 ~ -30] dBm.

For how to define the requirements for different direction, we propose to use the same approach with Tx power.

Proposal 8: Use the same approach with Tx power to define the requirements for different directions.

2.6 Maximum input power

It agreed that maximum input power should be defined. From system point of view, 1m distance assumption is a reasonable option. If this is agreed, then 0 dBm can be the requirement.

Proposal 9: 0 dBm is defined for the maximum input power for device 1.

2.7 Occupied bandwidth

Occupied BW usually is a regulation requirement. For backscattering devices, it seems occupied BW requirement may not have any impact to the implementation as the devices which include a filter then the filter passband can refer OBW. So from that aspect, it seems not very necessary to define OBW from technical point of view. Then we may propose not to define OBW requirement if there’s no regulation request.

Proposal 10: OBW requirement for device 1 is not defined if there’s no regulation request.

- Conclusion

This contribution provides our analysis for the device 1 RF requirements. We have the following proposals and observations.

Proposal 1: The maximum input CW power to calculate device 1 radiated power can be assumed as 0 dBm.

Proposal 2: EIRP is used as the metric to test transmit output power. FR2 BS EIRP range declaration approach can be considered.

Proposal 3: Define SFO requirement for device 1.

Proposal 4: EVM can be a candidate metric for modulation quality requirements.

Proposal 5: Device 1 Tx signal SNR is defined [10] dB higher than receiver DEMOD SNR threshold.

Observation 1: EVM can be tested for small frequency shift mode if it needs to be tested.

Proposal 6: TRP is used for the emission requirements, some simplified test method can be used.

Proposal 7: Emission requirement is only tested for no small frequency shift mode.

Observation 2: Device REFSENS can be defined with the minimum requirement [-23 ~ -30] dBm.

Proposal 8: Use the same approach with Tx power to define the requirements for different directions.

Proposal 9: 0 dBm is defined for the maximum input power for device 1.

Proposal 10: OBW requirement for device 1 is not defined if there’s no regulation request.

- References

[1] R4-2505230, “WF on A-IoT device requirements”, CMCC, RAN4#114b

[2] ETSI EN 302 208