3GPP TSG-RAN WG4 Meeting #115																			R4-2505756

Malta, MT, 19th – 23rd May, 2025

Source: 	Xiaomi

Title: 	Discussion on AIoT CW RF requirements

Agenda item:	7.26.3.3

Document for:	Discussion

- Introduction

This contribution provides our further analysis for CW requirements based on the last RAN4 meeting’s progress [1].

- Discussion

2.1 ON/OFF power

Reader OFF requirement was agreed in last RAN4 meeting that less than [-85] dBm/MHz per antenna connector is defined. It’s straightforward CW node reuse AIoT BS requirements.

Proposal 1: Define CW off power requirement following reader off power value.

2.4 Phase noise

CW Phase noise contributes to reader’s noise floor, the contribution analysis is different for different deployment scenarios. In R-19 WI, D1T1-B is assumed as the deployment scenario. The benefit for this scenario is that the CW phase noise can have some spatial isolation benefit. Some link budget analysis is conducted as follows,

Table 1: CW2R link budget

| CW2R link |  |
| --- | --- |
| CW Tx power (dBm) | 33 |
| CW Tx antenna gain | 6 |
| CW phase noise (dBc/Hz) | -120 |
| CW2R distance (m) | 20 |
| FSPL CW2R (dB) | 59.4 |
| Reader Rx antenna gain (dBi) | 6 |
| Reader Rx phase noise power at antenna port (dBm/Hz) | -134.4 |

Table 2: CW2D link budget

| CW2D link |  |
| --- | --- |
| Reader CW Tx power (dBm) | 33 |
| Reader CW Tx antenna gain | 6 |
| CW2D distance (m) | 20 |
| FSPL CW2D (dB) | 59.4 |
| Device Rx antenna gain (dBi) | 0 |
| Device Rx CW power at antenna port (dBm) | -20.4 |

Table 2: D2R link budget

| D2R link |  |
| --- | --- |
| Device antenna gain (dBi) | 0 |
| Device backscattering loss (dB) | 6 |
| Device Tx radiated power (dBm) | -26.4 |
| D2R distance (m) | 20 |
| FSPL D2R (dB) | 59.4 |
| Reader Rx antenna gain (dBi) | 6 |
| Reader Rx device power at antenna port (dBm) | -79.7 |
| Reader Rx device power at antenna port (dBm/Hz) | -131.5 |

From the above link budget, we can have the following observation,

Observation 1: If CW phase noise is -120 dBc/Hz @ 7.5kHz offset, 15kHz DSB SNR for 20m coverage for CW2D and D2R is 3 dB.

From the datasheets available, -120 dBc/Hz @ 7.5kHz for VCXO is achievable, better performance can be achieved if more implementation efforts can be paid.

Observation 2: -120 dBc/Hz @ 7.5kHz phase noise performance for VCXO is achievable.

If the CW signal source is from reader and the phase noise is not that good, reader can still use broadband leaking carrier canceller to suppress the phase noise, there’s some room for the CW phase noise performance is reader can share some penalty.

Based on the above analysis, it seems the combined CW phase noise contribution to reader Rx noise is decided by both CW phase noise and reader phase noise canceller.

Observation 3: The CW phase noise contribution to reader Rx noise is decided by both CW phase noise and reader phase noise canceller.

Therefore, it’s hard to decide CW phase noise requirement. Considering usually VCXO performance is good, the phase noise performance can be left to implementation for R19 D1T1-B scenario. Of course, this decision should be revisited for other scenarios.

Proposal 2: Leave the CW phase noise performance to implementation in R19. This decision should be revisited for other scenarios.

2.3 Unwanted emission and spurious emissions

It’s not clear how to define unwanted emission for CW signal because there’s no channel bandwidth for CW. We have the following proposal,

Proposal 3: Unwanted emission requirement is not defined.

For spurious emission requirement, it was agreed that NR requirement is baseline.

Table 6.6.5.2.1-2: General BS transmitter spurious emission limits in FR1, Category B

| Spurious frequency range | Basic limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150 kHz |  | 1 kHz | Note 1, Note 4 |
| 150 kHz – 30 MHz | -36 dBm | 10 kHz | Note 1, Note 4 |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz – 12.75 GHz |  | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz | -30 dBm | 1 MHz | Note 1, Note 2, Note 3 |
| 12.75 GHz - 26 GHz | - 30 dBm | 1 MHz | Note 1, Note 2, Note 5 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: Applies for Band for which the upper frequency edge of the DL operating band is greater than 2.55 GHz and less than or equal to 5.2 GHz. NOTE 4: This spurious frequency range applies only to BS type 1-C and BS type 1-H. NOTE 5: Applies for Band for which the upper frequency edge of the DL operating band is greater than 5.2 GHz. |  |  |  |

Referring the NR BS spurious requirements, -30 dBm/MHz for 33 dBm CW power is -123 dBc/Hz. It’s aligned with the assumption in the phase noise analysis.

Observation 4: -30 dBm/MHz spurious emission requirement is -123 dBc/Hz for 33 dBm CW power.

For the boundary of spurious emission requirements, it was proposed D2R CBW can be considered because the spurious emission requirement is defined to protect other systems. Currently, there’re many D2R TBW possibilities as discussed in our contribution [2]. Use the smallest CBW 7.5kHz is an option, there’s another option is to use 1RB which is the smallest R2D frequency resource. As FDD spectrum usually is used as paired UL/DL, so in high possibility, D2R BW in reader side is uses at least 1 RB. So we propose 100kHz as the spurious emission requirement boundary.

Proposal 4: The boundary for CW spurious emission requirement is 100kHz.

2.5 Tx Intermodulation

In our understanding, CW node can be considered as a NW similar node not a device because it’s fixed and is deployed in some place by a plan. So BS Tx intermodulation requirement motivation can be taken as a reference [3]. BS Tx intermodulation is defined for co-location scenario. R19 AIoT deployment is very simple, our understanding is no co-location deployment with other BS for CW node, so Tx intermodulation requirement is not needed.

Observation 5: No co-location deployment assumption with other BS for CW node is foreseen in R19 AIoT WI.

Proposal 5: Tx intermodulation requirement for CW node is not defined.

- Conclusion

This contribution provides our further analysis for AIoT CW requirement. We have the following proposals,

For ON/OFF power,

Proposal 1: Define CW off power requirement following reader off power value.

For phase noise,

Observation 1: If CW phase noise is -120 dBc/Hz @ 7.5kHz offset, 15kHz DSB SNR for 20m coverage for CW2D and D2R is 3 dB.

Observation 2: -120 dBc/Hz @ 7.5kHz phase noise performance for VCXO is achievable.

Observation 3: The CW phase noise contribution to reader Rx noise is decided by both CW phase noise and reader phase noise canceller.

Proposal 2: Leave the CW phase noise performance to implementation in R19. This decision should be revisited for other scenarios.

For unwanted emission and spurious emissions,

Proposal 3: Unwanted emission requirement is not defined.

Observation 4: -30 dBm/MHz spurious emission requirement is -123 dBc/Hz for 33 dBm CW power.

Proposal 4: The boundary for CW spurious emission requirement is 100kHz.

For Tx intermodulation,

Observation 5: No co-location deployment assumption with other BS for CW node is foreseen in R19 AIoT WI.

Proposal 5: Tx intermodulation requirement for CW node is not defined.

- References

[1] R4-2505097, “WF on A-IoT BS and CW requirements”, Huawei, RAN4#114b

[2] R4-2505753, “Discussion on AIoT system parameters”, Xiaomi, RAN4#115

[3] TR 38.817-2