3GPP TSG-RAN WG4 Meeting 115	R4-2505765

Malta, MT, 19th – 23rd May, 2025

Source: 	Xiaomi

Title: 	Discussion on performance requirement for AI beam management

Agenda Item:	7.21.6

Document for:	Discussion

- Introduction

In this contribution, we will discuss performance requirement for AI based beam management.

- Discussion

      - 2.1 Performance metric

2.1.1 Beam ID accuracy

In last meeting, metrics for beam ID prediction is agreed:

Issue 2-1: Metrics/KPIs for beam ID prediction Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

In last meeting, Top-M/K is also agreed as KPI for UE-assisted performance monitoring in RAN1, which is shown as below:

RAN1#120b Agreement For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signalling

Observation 1: The similar metric Top-K/M is agreed in RAN1 performance monitoring, where at least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s).

Value of X:

- When X = 0dB, it will be equal to original option 2, and will be aligned with RAN1 KPI for performance monitoring.

- When X = 1dB, it will be equal to original option 3.

We suggest to keep both options for X. Then the above definition can include the original option 2 and option 3 together.

Value of N:

From our simulation results with agreed simulation assumption, we observe that:

- When X = 0dB, it will be equal to original option 2. Then N can be relaxed to be larger than 1. When K is smaller than 4, we prefer that N=2. When K=4, N can be 1 or 2.

- When X = 1dB, N = 1.

Proposal 1: In order to align with RAN1, Beam index prediction definition should include both original option 2 and option 3:

- For X value: keep both X = 0 and X=1dB.

- For N value:

  - When X = 1 dB, N = 1.

  - When X = 0dB, N = 1 or 2 depending on K value.

2.1.2 RSRP absolute accuracy

In last meeting, it’s agreed that absolute RSRP accuracy will applied for Top-1 predicted beams and it’s FFS whether can be apply for other beams.

Issue 2-2: Absolute RSRP accuracy Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams

From RAN1 agreements in RAN1#120:

Agreement For report content of inference results for UE-sided model, where the largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size. Note: the model output is UE implementation and it doesn’t have to be RSRP subject to dBm value.

The reporting scheme is the same as legacy non-AI based beam reporting.

For K > 1 beams:

- 1st-ranked beam: Reported as absolute quantized RSRP

- 2nd-Kth beams: Reported as differential RSRP (4-bit resolution, 2dB step size) relative to the 1st-ranked beam

In legacy, absolute RSRP is defined for the largest RSRP and relative accuracy is defined for other differential RSRP in the report.

Observation 2: In legacy, absolute RSRP is only defined for the largest RSRP and relative accuracy is defined for other differential RSRP in the report.

Challenges for Non-Top-1 Beams

Extending absolute accuracy to non-top-1 beams requires reconstructing their RSRP from differential values, introducing two critical issues:

1. Cumulative Quantization Errors:

UE reports predicted RSRP for Top-K beams with format: {A₁, ΔB₁, ΔC₁}

- Beam A (predicted 1st): Absolute RSRP

- Beams B/C: Differential RSRP (ΔB₁/ΔC₁)

The reconstructed absolute RSRP for 2nd-Kth beams contains quantization uncertainty:

Rkpred=RApred-ΔRkpred±Δquantdiff

where Δquantdiff=12×differential step size (1dB for 2dB step), Considering the quantization method with rounding off.

2. Ranking Instability: Beam order changes between prediction and ground truth reports amplify inconsistencies.

These limitations lead to two fundamental observations:

1. Error Propagation:

- Single-differential beams (e.g., 2nd-ranked): 1 dB uncertainty.

- Twice-differential beams (e.g., 3rd-ranked): 2 dB uncertainty.

2. Validation Reliability:

- Direct comparison of predicted vs. measured RSRP for non-top-1 beams becomes unreliable due to:

- Ranking mismatches.

- Compounded quantization noise.

To ensure reliable validation while minimizing complexity, we suggest to define absolute RSRP accuracy only for the top-1 predicted beam to:

- Avoids compounding quantization errors from differential reporting.

- Aligns with legacy beam reporting, which focuses on the highest-ranked beam.

Proposal 2: Only define Absolute RSRP accuracy requirement for Top-1 of predicted beam.

2.1.3 RSRP relative accuracy

RAN4 agrees to further discuss the definition of relative accuracy.

- RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

In legacy, relative RSRP accuracy is defined for measured differential RSRP reporting. Therefore, similarly, we suggest to define relative RSRP accuracy for predicted differential RSRP reporting.

Therefore,

Relative accuracy for BM-case 1 is similar as legacy:

  - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest prediction RSRP.

Where predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n is predicted differential RSRP in the report.

Another issue is that whether relative accuracy defined above can apply for BM case-2.

Extension to BM-Case 2

The baseline relative RSRP accuracy definition (designed for BM-Case 1) cannot be directly applied to BM-Case 2 due to fundamental differences in reporting structure and reference point selection. Here’s a detailed breakdown of the limitations:

1. Single-Instance vs. Multi-Instance Reporting

- BM-Case 1 (Single Time Instance):

- Reports K beams at one time instance.

- The reference beam (n) is clearly defined as the highest-RSRP beam within that report.

- Relative accuracy compares all other beams (i) to this fixed reference (n) in the same snapshot.

- BM-Case 2 (Multiple Time Instances):

- Reports K beams × N time instances (e.g., beam predictions over time).

- Ambiguity of global maximum RSRP (used as reference):

- different time instance for the same beam index n

- Relative accuracy can happen in two dimensions:

- Intra-time beam comparison (same time, different beams)

- Inter-time beam comparison (same beam, different times)

The baseline formula assumes the reference (n) and target (i) beams are from the same time instance, which is invalid when n and i span multiple times. The baseline definition lacks a mechanism to handle this two-dimensional variability.

In summary:

We suggested to use a unified expression for both BM case-1 and case-2:

Relative RSRP Accuracyi,t=Predicted RSRPi,t-Predicted RSRPmax-True RSRPi,t-True RSRPmax

Where Predicted RSRPmax is the largest RSRP across all time instance and beams in the report。Predicted RSRPi,t is RSRP of beam index i at time t。True RSRPmax is the true RSRP corresponding to the beam/time of Predicted RSRPmax。Where Predicted RSRPi,t-Predicted RSRPmax is predicted differential RSRP in the report.

Proposal 3: For relative predicted RSRP accuracy, define unified definition for both BM-case 1 and BM-case 2:

Relative RSRP Accuracyi,t=Predicted RSRPi,t-Predicted RSRPmax-True RSRPi,t-True RSRPmax

Where Predicted RSRPmax is the largest RSRP across all time instance and beams in the report。Predicted RSRPi,t is RSRP of beam index i at time t。True RSRPmax is the true RSRP corresponding to the beam/time of Predicted RSRPmax。Where Predicted RSRPi,t-Predicted RSRPmax is predicted differential RSRP in the report.

      - 2.2 Measurement error

Fit RSRP delta CDF of LLS by Gaussian distribution

Step 1: calculate RSRP delta between ideal RSRP and measured RSRP by LLS and plot CDF curve.

Step 2: determine RSRP delta corresponding to 5% and 95%, which corresponding to the lower and upper limits of the 90% confidence interval. Suppose that the lower and upper limits for 5% and 95% points of CDF curve from LLS are a and b.

Step 3: Calculate the standard deviation, σ, for Gaussian distribution. Where Z0.95 is the 95th percentile of the standard normal distribution, which is approximately 1.645.

σ=b-a2∙Z0.95

First, we derive the measured RSRP accuracy CDF for AWGN channel under SNR = -3dB obtained from LLS, where a = 1.4 and b = -1.4. then we also derive the measured RSRP accuracy CDF for TDL-C channel under SNR = -3dB obtained from LLS where a = -2.2 and b = 1.6. By step 3, σ=0.6245 for AWGN channel and σ= 1.3341 for TDL-C.

Here, we plot the RSRP measurement delta CDF curve from LLS and the fitted curve by Gaussian distribution for AWGN and TDL-C in the following figure. It shows that Gaussian distribution can fit the measurement error models from LLS well.

- AWGN                                (b) TDL-C

Fig.1 RSRP measurement delta CDF curve from LLS and fitted by Gaussian

Here, we also apply the different measurement error to different TX beams with different SNR conditions. Where measurement error is modeled as Gaussian distribution.

2.2.1 Beam index prediction accuracy

The predication accuracy for all UE with largest SNR higher than -3dB is as below:

X = 0dB

Table 1: Beam index prediction accuracy % (UEs with Highest SNR >=-3dB)

|  | Top-1/1 | Top-1/2 | Top-2/1 | Top-2/2 | Top-4/1 | Top 4/2 |
| --- | --- | --- | --- | --- | --- | --- |
| No error | 66.39 | 94.54 | 88.35 | 98.50 | 97.85 | 98.12 |
| RF | 68.34 | 94.38 | 88.32 | 98.45 | 97.21 | 98.04 |
| BB | 49.25 | 69.84 | 66.45 | 78.80 | 86.79 | 92.35 |
| RF+BB | 45.68 | 66.60 | 65.76 | 77.64 | 83.17 | 91.27 |

X = 1dB

Table 2: Beam index prediction accuracy with X=1 dB % (UEs with Highest SNR >=-3dB)

|  | Top-1/1 | Top-2/1 | Top-4/1 | Top 4/2 |
| --- | --- | --- | --- | --- |
| BB | 75 | 85.86 | 89.80 | 93.15 |
| RF | 7618 | 85.64 | 90.21 | 94.08 |
| RF+BB | 76.26 | 84.42 | 85.91 | 92.58 |

2.2.2 RSRP prediction accuracy

The absolute RSRP prediction accuracy is as below:

Table 3: RSRP accuracy, dB(UEs with Highest SNR >=-3dB)

|  | Top-1 | Top-3 |
| --- | --- | --- |
| BB | 7.71 | 7.12 |
| RF | 3.54 | 3.66 |
| RF+BB | 8.23 | 7.72 |

2.2.3 Factors Impacting Simulation Result Alignment

There are several factors may have impact on the simulation result:

      - Method of Adding Measurement Error for Different TX Beams

Currently, there are two methods to add error for BB:

- Option 1: Apply the error distribution from the post-beamforming SNR of the strongest Tx-Rx beam pair to all Tx-Rx beam pairs in the SLS.

- Option 2: Generate an error distribution separately for each individual Tx-Rx beam pair’s SNRs and apply it to the corresponding Tx-Rx beam pair measurement in the SLS.

These two options will significantly affect the final performance. Observations show that the SNR range for beams in set B can vary widely, often exceeding 20 dB. For instance, if the highest SNR is -3 dB, the SNR for other TX beams will be lower than -20 dB. Consequently, the measurement error distribution will differ, with varying Gaussian standard deviations.

In order to better simulate the real implementation in real word, we suggest to generate error distributions separately for each individual Tx-Rx beam pair based on different SNRs and apply different Gaussian standard deviations to the corresponding Tx-Rx beam pair measurements in the SLS.

      - Gaussian Standard Deviation for Different SNRs

For different TX beams, different Gaussian standard deviations will be considered. RAN4 need to discuss whether and how to align the Gaussian standard deviation of measurement error for different SNRs. The difficulty is that measurement error is highly dependent on the UE implementation, such as whether advanced noise cancellation algorithms are applied.

      - Largest SNR Distribution Among All Selected UEs

- Current Assumption:

- The simulation assumes that the strongest SNR for selected UEs will be higher than -3 dB. We generate L1-RSRP for multiple UEs and corresponding SINR for all TX beams from SLS. For AI model, the input for each UE will be 8 TX beams from set B, i.e. 8 SINRs. According to maximum SINR of each UE, we split UE into 5 groups.

- UE group 1: 15dB≤maximum SINR

- UE group 2: 10dB≤maximum SINR<15dB

- UE group 3: 5dB≤maximum SINR<10dB

- UE group 4: 0dB≤maximum SINR<5dB

- UE group 5: -3dB≤maximum SINR<0dB

Table: Beam prediction accuracy for each UE groups with RF+BB error, X=0dB

| Maximum SINR range of UE | Beam index Prediction accuracy % |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Top-1/1 | Top-1/2 | Top-2/1 | Top-2/2 | Top-4/1 | Top 4/2 |
| No error | 66.39 | 94.54 | 88.35 | 98.50 | 97.85 | 98.12 |
| UE group 1 | -10.62 | -11.94 | -11.67 | -11.89 | -5.85 | -4.02 |
| UE group 2 | -13.21 | -14.7 | -14.89 | -13.09 | -7.3 | -2.87 |
| UE group 3 | -19.71 | -22.34 | -21.88 | -20.46 | -13.42 | -5.77 |
| UE group 4 | -28.26 | -41.33 | -35.99 | -37.67 | -25.09 | -17.48 |
| UE group 5 | -44.31 | -59.34 | -56.79 | -56.05 | -51.19 | -46.87 |

Note: Prediction accuracy from UE group 1 to UE group 5 is the relative degradation value compared with no error  case.

Observation 3: Significant Performance Degradation in Low SNR Groups. The performance degradation is more significant in UE groups with lower maximum SNR.

For example, the Top-1/1 prediction accuracy drops by 44.31% for UE group 5 (SINR < 0 dB), compared to a drop of only 10.62% for UE group 1 (SINR ≥ 15 dB).

Observation 4: Higher Stability in High SNR Groups. UE groups with higher maximum SNR (e.g., UE group 1 and UE group 2) show better performance stability under different measurement errors.

For instance, UE group 1's Top-1/1 accuracy drops by 10.62% with RF+BB error, while UE group 5's drops by 44.31%.

This means that the RSRP prediction performance can vary substantially depending on the distribution of the maximum SNR. For example, in one scenario, most UEs have a strongest SNR around -3 dB, while in another scenario, most UEs have a strongest SNR above 10 dB, with only a few around -3 dB. This variation in the maximum SNR distribution can significantly impact the final performance. In our simulation, the largest SNR distribution is as below:

Fig. 2 strongest SINR distribution

Therefore, RAN4 need to align the distribution of the strongest SNR for all selected UEs.

      - SNR Distribution for TX Beams in Set B:

To better align the results, it is also important to align the SNR distribution for all TX beams in set B. from our simulation, SINR distribution for 5 UE groups are plotted in Fig.3. For all UE groups. The SNR will span a large range for the best beams and worst beams in set B. if the distribution of SNR span vary a lot for different companies, it’s also difficult to align the result.

Fig.3

To better align the results, it is crucial to align the SNR distribution for all TX beams in set B. From our simulation, the SINR distribution for the five UE groups is plotted in Fig. 3. For all UE groups, the SNR spans a wide range, from the best beams to the worst beams in set B. If the SNR distribution varies significantly across different companies, it becomes challenging to align the results.

Observation 5: The SINR distribution for all UE groups spans a large range, and this distribution can significantly impact the final accuracy performance.

To ensure consistent and reliable simulation results, we recommend that RAN4 align the following factors:

Proposal 4: To ensure consistent and reliable simulation results, we recommend that RAN4 align the following factors:

- 1. Method of Adding Measurement Error for Different TX Beams：

    - Generate error distributions separately for each individual Tx-Rx beam pair based on different SNRs and apply different Gaussian standard deviations to the corresponding Tx-Rx beam pair measurements in the SLS.

- 2. Gaussian Standard Deviation for Different SNRs

    - Establish reference standard deviation values for critical SNR operating points.

- 3. Largest SNR Distribution Among All Selected UEs

    - Divide the largest SNR range into several groups and define reference percentages for each group.

- 4. SNR Distribution for TX Beams in Set B

    - Align the SNR distribution for all TX beams in set B to ensure consistency across different companies.

- Conclusion

In this contribution, we provide the following proposals:

Observation 1: The similar metric Top-K/M is agreed in RAN1 performance monitoring, where at least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s).

Proposal 1: In order to align with RAN1, Beam index prediction definition should include both original option 2 and option 3:

- For X value: keep both X = 0 and X=1dB.

- For N value:

  - When X = 1 dB, N = 1.

  - When X = 0dB, N = 1 or 2 depending on K value.

Observation 2: In legacy, absolute RSRP is only defined for the largest RSRP and relative accuracy is defined for other differential RSRP in the report.

Proposal 2: Only define Absolute RSRP accuracy requirement for Top-1 of predicted beam.

Proposal 3: For relative predicted RSRP accuracy, define unified definition for both BM-case 1 and BM-case 2:

Relative RSRP Accuracyi,t=Predicted RSRPi,t-Predicted RSRPmax-True RSRPi,t-True RSRPmax

Where Predicted RSRPmax is the largest RSRP across all time instance and beams in the report。Predicted RSRPi,t is RSRP of beam index i at time t。True RSRPmax is the true RSRP corresponding to the beam/time of Predicted RSRPmax。Where Predicted RSRPi,t-Predicted RSRPmax is predicted differential RSRP in the report.

Observation 3: Significant Performance Degradation in Low SNR Groups. The performance degradation is more significant in UE groups with lower maximum SNR.

Observation 4: Higher Stability in High SNR Groups. UE groups with higher maximum SNR (e.g., UE group 1 and UE group 2) show better performance stability under different measurement errors.

Observation 5: The SINR distribution for all UE groups spans a large range, and this distribution can significantly impact the final accuracy performance.

Proposal 4: To ensure consistent and reliable simulation results, we recommend that RAN4 align the following factors:

- 1. Method of Adding Measurement Error for Different TX Beams：

    - Generate error distributions separately for each individual Tx-Rx beam pair based on different SNRs and apply different Gaussian standard deviations to the corresponding Tx-Rx beam pair measurements in the SLS.

- 2. Gaussian Standard Deviation for Different SNRs

    - Establish reference standard deviation values for critical SNR operating points.

- 3. Largest SNR Distribution Among All Selected UEs

    - Divide the largest SNR range into several groups and define reference percentages for each group.

- 4. SNR Distribution for TX Beams in Set B

    - Align the SNR distribution for all TX beams in set B to ensure consistency across different companies.

- Reference

[1] R1-2501595, FL summary 6 for AI/ML in beam management