3GPP TSG-RAN WG4 Meeting #115				                                            R4-2505778

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.27.3.2

Title: 	Discussion on RRM requirements for adaptation of common signal/channel in NES

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1] for R19 NES was agreed and one of the objectives below was discussed in the last meeting [4].

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this meeting, we provide our further considers on the open issues below.

- L3 measurement for SSB adaptation

- SCell activation requirement for SSB adaptation

- Discussion

  - L3 measurement

In the last RAN4 meeting, the following agreements/WF were achieved.

Issue 1-3-1: L3 requirements for SSB adaptation Agreement: - RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation. - For neighbour cell L3 measurement, legacy requirements apply. Open issue for further discuss: - For SCell with SSB adaptation (deactivated SCell or activated SCell), wait for more RAN2 progress on additional SMTC configuration, and discuss following aspects: - Whether to define L3 measurement requirement at transition. - Any other RRM requirements impacts due to additional SMTC configurations.

Furthermore, according to RAN2 agreements below [5] on the new SMTC for L3 measurement based on the adaptation SSB, we can conclude that:

Agreements on SSB adaptation

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration. Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

Proposal 1: During the transition, the L3 neighbor cell measurement requirements with the legacy SMTC for the non-adapted SSB can be applied. But after the transition, the L3 neighbor cell measurement requirements with additional SMTC configuration for the adapted SSB can be applied .

  - SCell activation requirement for SSB adaptation

In the previous meeting, for the SCell activation requirement for SSB adaptation some proposals on how to define the SCell activation requirement (e.g. the start point of SCell activation command) was raised but without any requirements:

R4#114 Agreement: For SCell activation: - Define the SCell activation requirements based on the SSB periodicity after SSB adaptation. - For the case SSB adaptation command comes together with SCell activation command. The requirement can be updated based on further progress in other WG R4#114bis proposal: Recommended WF: - RAN4 to discuss the conditions for SCell activation with SSB adaptation (i.e. based SSB periodicity after SSB adaptation), considering following aspects: - SSB adaptation command (e.g. DCI/MAC-CE/RRC), which subject to conclusion from other WG - Time relation between SSB adaptation command and SCell activation command.

Figure 1a. SCell activation when UE receiving SCell activation command earlier than SSB adaptation

Figure 1b. SCell activation when UE receiving SCell activation command later than SSB adaptation

However, as illustrated in Figure 1a above, no matter whether SSB adaptation is triggered simultaneously with SCell activation command, if the processing time for SSB adaptation command (MAC) is larger than that for SCell activation command, UE receiving time for adaptation SSB command can be after that for SCell activation command. As a result, the start point of SCell activation delay defined TS38.133 can be not reused. That is the current SCell activation delay requirements in TS38.133 can be reused for the SCell activation procedure with the adaptation SSB under some conditions.

8.3 SCell Activation and Deactivation Delay 8.3.1 Introduction This clause defines requirements for the delay within which the UE shall be able to activate at least one deactivated SCell and deactivate at least one activated SCell in EN-DC, or in standalone NR carrier aggregation, or in NE-DC, or in NR-DC. The requirements shall apply for EN-DC, standalone NR carrier aggregation, NE-DC, and NR-DC. 8.3.2 SCell Activation Delay Requirement for Deactivated SCell The requirements in this clause shall apply for the UE configured with at least one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated. The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions. Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where:

Therefore, we can propose that:

Proposal 2: Define the SCell activation requirements based on the SSB periodicity after SSB adaptation.

  - The start point can be the slot UE receiving SCell activation command if UE receiving SSB adaptation command (MAC) is not later than it.

- Conclusion

In this contribution, serval issues related to RRM in Rel19 NES are discussed. The observations and proposals can be summarized as:

Proposal 1: During the transition, the L3 neighbor cell measurement requirements with the legacy SMTC for the non-adapted SSB can be applied. But after the transition, the L3 neighbor cell measurement requirements  with additional SMTC configuration for the adapted SSB can be applied .

Proposal 2: Define the SCell activation requirements based on the SSB periodicity after SSB adaptation.

  - The start point can be the slot UE receiving SCell activation command if UE receiving SSB adaptation command (MAC) is not later than it.

- References

- RP-240830

- TS38.133 v18.6.0

- TS38.331 v17.1.0

- R4-2502689

- R2-2501484