# 3GPP TSG-RAN WG4 Meeting #115	R4-250xxxx

# Malta, MT, 19-23 May, 2025

Agenda Item:	7.7.3

Source: 	Xiaomi

Title: 	LB (FDD) – LB (SDL) CA with switching

Document for:	Discussion

- Introduction

In previous RAN-P, a new WI was agreed to enable LB-LB CA with swicthing in [1]. The motivication of this WI was to efficiently use the SDL spectrum together with low FDD band via CA by swicthing to eliminate UE implemenation channllenge  of supporting co-current transmission/reception between these two carriers due to antenna performance with wide frequnecy separation (fractional bandwith), and or duplexer design w/o sufficient frequency gap between DL and UL.

Detailed WID is reproduced here for reference:

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In this contribution, we analyze the remaining open issues to UE RF requirements for LB-LB CA via switching.

- Discussion

Based on the agreed WID, it’s agreed to focus on the scope as following:

- Focus on FDD-SDL band with semi-static switching pattern

  - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

  - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier

  - Focus on collocated scenario with single TAG and aligned SCS 15kHz only

- Example band combination: band n5 + SDL band n29

RAN4 target to specify time mask and required switching delay for the transition between two carriers as case 1 to case 2 or vice versa w/o simultaneous Tx/Rx between the PCell and the SCell.

In previous RAN4 meetings, following agreements was reached:

RAN#114 agreement: Issue 2: Switching period values - Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations - 35us, 140us - RAN4 to further discuss 13us - LS will capture the currently confirmed values - It is not precluded to send another LS with more values in future meetings Issue 3: Switching period capability - Discuss whether the switching delay capability can be per band combination Issue 4: Time mask for switching - Discuss the time mask for switching based on the following options: - Option 1: Switching period can be either within carrier 1 (FDD) or within carrier 2 (SDL) - Option 2: Switching period is always within the switch-from carrier - Option 3: Switching period is contained within the switching gap configured by the network - Other options are not precluded Issue 5: Switching period location - Discuss the switching period location based on the following options: - Option 1: Switching period location is entirely up to RAN1 design - Option 2: Switching period location is defined relative to the slot boundary - Other options are not precluded RAN4#114bis agreement Issue 1: Switching period capability - The switching period capability can be defined per band pair per band combination. Issue 2: Switching period location - Await outcome of RAN1 design Issue 3: Time mask for switching - Define the Tx on-off time mask requirements - FFS on how to define the Rx switching requirements to reflect where the switching period occurs - RAN4 has understanding that RAN1 will take switching period and timing advance into account for RAN1 scheduling pattern for low band CA via switching Issue 4: Considerations related to the Tx on-off time mask - Option 1: Specify transmit on/off time mask for FDD UL carrier/Pcell for switching between LB(FDD)-LB(SDL) - Option 2: Specify on->off time mask for switching from FDD to SDL carrier and specify off->on time mask for switching from SDL to FDD carrier - Other options are not precluded Issue 5: Test methodology - Companies are encouraged to contribute proposals related to the testability of the requirement on the time mask for switching

Regarding switching period location, RAN4 agreed to wait for RAN1 progress on switch gap/switching pattern design. Following agreements reached in April Wuhan RAN1 meeting:

RAN1#120bis Agreement - For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases, - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - FDD carrier 1 is PCell and SDL carrier 2 is SCell. - SCS 15KHz on both carriers. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier. Agreement For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS). Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2. - The pattern starts from the beginning of SFN 0 of PCell. - Restriction of maximum X switch(es) within Y slot(s). - FFS: detailed value and relation of X and Y. - FFS: other restriction. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, down-select one of the following solutions: - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier. - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier. - Alt 3: Switching gap is always assumed at the SDL carrier. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed). - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL. - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA. - FFS: whether it has specification impact.

Observation 1: Semi-static switching pattern is per slot basis and switching gap/switching pattern are based on the downlink timing (DL reception time at UE side) of Pcell.

Observation 2: The duration of the switching gap is RRC configured by NW and the duration at least cover switching period and UL TA (for SDL to FDD switching case).

Observation 3: For switching gap location,

- For SDL carrier switching to FDD carrier, switching gap is assumed at SDL carrier

- For FDD carrier switching to TDD carrier, FFS in RAN1 with 3 alternatives

  - Alt1: FDD carrier

  - Alt2: NW configuration

  - Alt3: SDL carrier

Proposal 1: For SDL carrier switching to FDD carrier, switching gap is located at SDL carrier.

Proposal 2: For FDD carrier switching to TDD carrier, FFS pending on further agreements from RAN1.

Regarding detailed time mask, in existing specification RAN4 specify time mask for uplink carrier switching mask for different cases. The switching command can be RRC based, MAC-CE or DCI based as specified in TS 38.214 for uplink Tx switching and switching location are indicated by RRC uplinkTxSwitchingPeriodLocation. The switching mask requirements for inter-band UL CA with 2 carriers are described in Figure 6.3A.3.3.2-1a and Figure 6.3A.3.3.2-1b either on carrier 1 or carrier 2 based on RRC indication.

Figure 6.3A.3.3.2-1a: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1 Figure 6.3A.3.3.2-1b: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

In RAN4 UE RF specification, the general time mask for uplink Tx on/off is also specified as following:

Figure 6.3.3.2-1: General ON/OFF time mask for NR UL transmission in FR1

For time mask requirements, the key difference compared to Rel-16 UL Tx switching are highlight below:

- On FDD-SDL band combination, no uplink transmission on SDL carrier: the switching happed between carrier 1 (Pcell, FDD carrier Tx/Rx) and carrier 2 (Scell, SDL carrier RX only)

- No con-current transmission and reception between these 2 carriers (for UL Tx switching, simultaneous reception and/or transmission across two carriers allowed pending on band combinations)

- Switching scheme: In Rel-19 only focus on semi-static switching pattern based on RRC configuration

Observation 4: RAN4 time mask requirements focus on uplink carrier switching or uplink carrier on/off.

Observation 5: Compared to UL tx switching case, FDD-SDL CA via switching only have available uplink transmission over than FDD carrier (Pcell).

From UE implementation perspective for FDD-SDL CA via switching, RF returning will interrupt both downlink reception and uplink transmission across two carriers which corresponding to RAN4 uplink transmission time mask requirements in RF specification TS 38.101-1 and DL interruption requirements specified in RRM specification TS 38.133 during the transient period.  From RAN4 requirements perspective, we suggest to follow traditional way to verify FDD-SDL CA via switching with Tx on/off time mask for uplink transmission on/off switching and DL reception switching between FDD carrier and SDL carrier by RRM interruption/ measurement requirements. Joint testing for both Tx/Rx will introduce a new test framework and hybrid of Tx/Rx requirement framework which is time consuming and overcomplicated which make it impractical in such late stage in Rel-19 5G-A timeframe.

We also observed that the potential impact to RRM requirements for FDD-SDL CA via switching already discussed in RRM session.  RRM session are discussing the potential RRM measurement and interruption requirements on this scenario.  In our view, DL reception behaviour can be implicitly/explicitly verified in RRM measurement/interruption requirements and companies are encouraged to discuss this issue in RRM session instead of RF session.

Observation 6: Joint testing for Tx/Rx switching require a new test framework with hybrid of Tx/Rx requirement which is time consuming and overcomplicated for test ability.

Observation 7: DL reception behaviour can be implicitly/explicitly verified in RRM measurement/interruption requirements which are already discussed over RRM session.

Proposal 5: Decouple FDD-SDL CA via switching with separate transmission and reception requirements.

Proposal 6: DL reception switching between FDD carrier and SDL carrier shall be discussed in RRM session instead of RF session.

For the analysis simplicity, here we take carrier 1 as FDD carrier (Pcell) and carrier 2 as SDL carrier (Scell). Depending on switching time location on carrier 1 or carrier 2, the corresponding impact to time mask and DL interruption requirements is different. According to RAN 1 design, FDD-SDL CA via switching is more like a TDD DL/UL pattern with configured switch pattern and switching gap.  With above RAN 1 design, we believe Tx on-off time mask is more like the general TDD uplink transmission on/off time mask

Proposal 7: Specify transmit on/off time mask for FDD UL carrier/Pcell for switching between LB(FDD)-LB(SDL).

- Case 1: SDL carrier to FDD carrier, switching gap located in SDL carrier; FDD carrier to SDL  carrier switching gap located in SDL carrier

Figure 1: Time mask for LB(FDD)-LB(SDL) switching on Pcell/carrier 1 with switching on out of Pcell and switching off within Pcell

- Case 2: SDL carrier to FDD carrier, switching gap located in SDL carrier; FDD carrier to SDL  carrier, switching gap located in FDD carrier

Figure 2: Time mask for LB(FDD)-LB(SDL) switching on Pcell/carrier 1 with switching gap out of Pcell

- Conclusion

In this contribution, we analyze the impact to UE RF requirements to enable LB-LB CA via switching.

Observation 1: Semi-static switching pattern is per slot basis and switching gap/switching pattern are based on the downlink timing (DL reception time at UE side) of Pcell.

Observation 2: The duration of the switching gap is RRC configured by NW and the duration at least cover switching period and UL TA (for SDL to FDD switching case).

Observation 3: For switching gap location,

- For SDL carrier switching to FDD carrier, switching gap is assumed at SDL carrier

- For FDD carrier switching to TDD carrier, FFS in RAN1 with 3 alternatives

  - Alt1: FDD carrier

  - Alt2: NW configuration

  - Alt3: SDL carrier

Proposal 1: For SDL carrier switching to FDD carrier, switching gap is located at SDL carrier.

Proposal 2: For FDD carrier switching to TDD carrier, FFS pending on further agreements from RAN1.

Observation 4: RAN4 time mask requirements focus on uplink carrier switching or uplink carrier on/off.

Observation 5: Compared to UL tx switching case, FDD-SDL CA via switching only have available uplink transmission over FDD carrier (Pcell).

Observation 6: Joint testing for Tx/Rx switching require a new test framework with hybrid of Tx/Rx requirement which is time consuming and overcomplicated for test ability.

Observation 7: DL reception behaviour can be implicitly/explicitly verified in RRM measurement/interruption requirements which are already discussed over RRM session.

Proposal 5: Decouple FDD-SDL CA via switching with separate transmission and reception requirements.

Proposal 6: DL reception switching between FDD carrier and SDL carrier shall be discussed in RRM session instead of RF session.

Proposal 7: Specify transmit on/off time mask for FDD UL carrier/Pcell for switching between LB(FDD)-LB(SDL).

- Case 1: SDL carrier to FDD carrier, switching gap located in SDL carrier; FDD carrier to SDL carrier switching gap located in SDL carrier

Figure 1: Time mask for LB(FDD)-LB(SDL) switching on Pcell/carrier 1 with switching on out of Pcell and switching off within Pcell

- Case 2: SDL carrier to FDD carrier, switching gap located in SDL carrier; FDD carrier to SDL  carrier, switching gap located in FDD carrier

Figure 2: Time mask for LB(FDD)-LB(SDL) switching on Pcell/carrier 1 with switching gap out of Pcell

Reference

[1] RP-243317, New WID on low band carrier aggregation via switching, AT&T TELUS, Bell Mobility, BT plc, Skyworks Solutions Inc., Anterix, Southern Linc, AT&T, Boost Mobile Network, Murata, Ericsson, Samsung, Qorvo, Nokia, Huawei, HiSilicon, vivo, OPPO, Qualcomm, Google, Sony, ZTE, Intel Corporation, MediaTek, LGE

[2] R4-2503016 WF o low band switching, Apple

[3] R4-2505223 WF on UE RF requirements for LB CA via switching, Apple

[4] RAN1#120bis chairman note

[5] R4-2504895, WF on RRM requirements for NR_LBCA_Sw, Apple