# 3GPP TSG-RAN WG4 Meeting #115	R4-250xxxx

# Malta, MT, 19-23 May, 2025

Agenda Item:	7.7.2

Source: 	Xiaomi

Title: 	TP to TR 38.768 on Time mask for switching

Document for:	APPROVAL

- Introduction

In previous RAN-P, a new WI was agreed to enable LB-LB CA with swicthing in [1]. The motivication of this WI was to efficiently use the SDL spectrum together with low FDD band via CA by swicthing to eliminate UE implemenation channllenge  of supporting co-current transmission/reception between these two carriers due to antenna performance with wide frequnecy separation (fractional bandwith), and or duplexer design w/o sufficient frequency gap between DL and UL.

Detailed WID is reproduced here for reference:

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In this contribution, we analyze the remaining open issues to UE RF requirements for LB-LB CA via switching.

In last RAN4 meeting, TR38.768 for low band aggregation via switching version 0.0.3 was approved in [2]. This contribution provides draft text proposal on section 5.3 based our companion contribution [4].

- Text proposal

---------------------------Start of TP------------------------------------------

5	Switching time mask

<Editor’s note: capture outcome of the discussion on the switching time mask in this clause>

5.1	Switching periods

To accommodate different UE implementations, RAN4 has reached the agreement in RAN4#114 on the applicable switching periods of 35μs and 140μs for switching between Case 1 and Case 2 as an optional UE capability.

5.2	Switching period location

<TBD>

5.3	Time mask for switching

RAN4 agreed to introduce transmit on/off time mask requirements for FDD carrier -SDL carrier switching between case 1 and case 2.

Figure 5.3-1 and 5.3-2 describe the transmit on/off time masks along with the switching period for both the transition from SDL carrier to FDD carrier and vice versa.

Figure 5.3-1: Time mask for LB(FDD)-LB(SDL) switching on FDD carrier 1 (Pcell) with switching period out of FDD carrier for switching to FDD carrier and switching period within FDD carrier for switching to SDL carrier.

Figure 5.3-2: Time mask for LB(FDD)-LB(SDL) switching on FDD carrier 1 (Pcell) with switching period out of FDD carrier

--------------------------End of TP----------------------------------

Reference

[1] RP-243317, New WID on low band carrier aggregation via switching, AT&T TELUS, Bell Mobility, BT plc, Skyworks Solutions Inc., Anterix, Southern Linc, AT&T, Boost Mobile Network, Murata, Ericsson, Samsung, Qorvo, Nokia, Huawei, HiSilicon, vivo, OPPO, Qualcomm, Google, Sony, ZTE, Intel Corporation, MediaTek, LGE

[2] R4-2505146	TR38.768 for low band aggregation via switching, Apple

[3] R4-2505223 WF on UE RF requirements for LB CA via switching, Apple