3GPP TSG-RAN WG4 Meeting #115		R4-2505813

Malta, MT, 19th – 23rd May, 2025

Agenda item:		7.24.4.1

Source:		Samsung

Title:	Discussion on RRM requirements of UE-initiated beam management in Rel-19 MIMO phase 5

Document for:		Discussion

- Introduction

In last RAN4#114bis meetings, the WF was approved in [1]. In this contribution, we continue to discuss the open issues of RRM requirements for UE-initiated/event-driven beam management.

- Discussion

In RAN4#114bis meeting, the latest agreement is approved in [1] as:

#### Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)? Agreement: - For T_RS (TSSB or TCSI-RS) in L1_measurement periods - T_RS (TSSB or TCSI-RS): - the value of current beam and new beams are the same. - For M in L1_measurement periods: - If timeRestrictionForChannelMeasurement configured, M= 1, event evaluation should be based on the most recent result from both new beam and current beam - If timeRestrictionForChannelMeasurement is not configured, - Further discuss on: - Option1: Reuse the same values of legacy, measurement period is based on 3 sample. No further restriction of UE implantation of which one is used - Option 2: No requirements even the NW can configure it. - Option 3: FFS - For P (sharing factor) in L1_measurement periods: - For SSB based or CSI-RS based, - The definition is the same as legacy - The value can be the same or different for the current beam and new beam

For the report configuration, the legacy L1-RSRP are used as:

##### 5.2.1.4.3 L1-RSRP Reporting For L1-RSRP computation - the UE may be configured with CSI-RS resources, SS/PBCH Block resources or both CSI-RS and SS/PBCH block resources, when resource-wise quasi co-located with 'type C' and 'typeD' when applicable. - the UE may be configured with CSI-RS resource setting up to 16 CSI-RS resource sets having up to 64 resources within each set. The total number of different CSI-RS resources over all resource sets is no more than 128. … When the higher layer parameter groupBasedBeamReporting-r17in CSI-ReportConfig is configured, the UE shall indicate the CSI Resource Set associated with the largest measured value of L1-RSRP, and for each group, CRI or SSBRI of the indicated CSI Resource Set is present first. If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "notConfigured", the UE shall derive the channel measurements for computing L1-RSRP value reported in uplink slot n based on only the SS/PBCH or NZP CSI-RS, no later than the CSI reference resource, (defined in TS 38.211[4]) associated with the CSI resource setting. If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the channel measurements for computing L1-RSRP reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SS/PBCH or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting.

In this Release, RAN1 only added new triggered mechanism. The reporting of legacy L1-RSRP is still applied. For NW configuration, it is still can be configured. In the discussion, we understand that the motivation of fair comparison between current beam and new beam if the measurement occasions are time separated. However, in legacy releases, how to handle multiple occasions depends on UE implementation. Unless UE can pass the accuracy test (based on 1 sample), it is not mandatory to use which sample(s) to perform the measurement. Only the measurement period is extended. In the event-triggered L1-RSRP measurement, we propose no further restriction like legacy.

Proposal 1: If timeRestrictionForChannelMeasurement is not configured, reuse the same values of legacy, measurement period is based on 3 sample for SSB based and for periodic and semi-persistent CSI-RS resources.

#### Issue 1-2: Components of Event triggered measurement reporting delay? Agreement: - The event triggered measurement reporting delay includes the Tfirst UL channel - Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH Further discuss the two options and select one option in the next meeting: - The event triggered measurement reporting delay is TL1_measurement period + Tfirst UL channel - Option 1: if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. (Samsung, Xiaomi, Apple, MTK) - Option 2: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam.. (E///, Nokia, QC)

In previous meeting, we have already discussed the difference between option 1 and option 2.

For option 1, it is easily ensured the evaluation based on the latest results from both current and new beam.

In option 2, it is ambiguous for the definition. All are left to UE implementation. For the wording, "if the event is triggered by current/new beam", if the measurement occasions are different from current beam and new beam, it has ambiguous of UE implementation how to measure current beam and how to measure new beams and which ones to compare.

If event-7 is considered in following issues, it will cause the definition of "If the event is triggered by ..." more complicated.

Therefore, we support option 1.

Proposal 2: The event triggered measurement reporting delay is TL1_measurement period + Tfirst UL channel, if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams.

#### Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19? <Way forward > - Option 1: (Qualcomm, Xiaomi, Apple, LGE) - Do not define requirements for eventInstanceCount-r19 > 1 - Option 1a: (Apple): To extend the requirements for case when eventInstanceCount-r19 > 1 is configured, add note longer delay is expected if eventInstanceCount-r19 > 1 is configured. - Option 2: (Samsung, CMCC, Huawei, ZTE, MediaTek) - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19 - Option 3: (Nokia, Ericsson) - Wait for further RAN1 conclusion - For how to specify the requirements in option 2 above: - Option 2a: - Measurement reporting delay is TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where, - TL1-RSRP_event triggered_measurement_Period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS. - C: event instances threshold is configured by NW. - Ttime_window: time window is configured by NW. - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH. - Other options are not precluded

The configurable eventInstanceCount-r19, can avoid the instantaneous results to guarantee the better performance with valid counter. In addition, it can avoid like ping-pong behavior if NW use this event-triggered L1-RSRP results.

Some companies are discussed this time window are the same as TTT in existing NR therefore no need to define any requirements related to TTT.

We don’t think it is the same as TTT. TTT is a Time during which specific criteria for the event needs to be met in order to trigger a measurement report in each event. In L3 measurement, for each MO, the measurement is based on samples of SMTC. The measurement time are based on the SMTC period for both cells. During some time, UE can decide the event is met or not. In L1 measurement, the time length of the window is the configured time window length. If no definition of how to perform on each or which RS to perform the L1 measurement. The UE behavior is not clear and the performance cannot be guaranteed.

Proposal 3: RAN4 to specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19.

#### Issue 1-5: CSI-RS based L1-RSRP for inter-cell BM <Way forward > - Option 1: - Not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management - Option 2: - Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

As discussed in last meeting, we think different companies have different understanding of Rel-17 ICBM. But this is not new measurement to be introduced in Rel-19 MIMO. That is why we suggest option 2.

Proposal 4: Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

#### Issue 1-6: Whether to specify new unified TCI switching delay requirements? <Way forward > - For MAC-CE based DL/UL TCI state switch delay - Option 1: (Qualcomm, Apple, Samsung (for known case), Ericsson) - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management - Option 2: (ZTE) - Remove components of TL1-RSRP and Tfirst-SSB+TSSB-proc - Option 3: (MediaTek) - FFS on whether to remove Tfirst-SSB+TSSB-proc - For DCI based DL/UL TCI state switch delay - Option 1: - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management - Option 2: - Whether to further reduce the delay, wait for further RAN1 progress

It is related to RAN1 agreement in RAN1#119 (3 meetings before)

Agreement Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception. - Other alternatives are not precluded.

After that, in RAN1#120 and RAN1#120bis, RAN1 didn’t further discussed on whether to reduce beam application latency and no further conclusion. Due to it is closed to RAN1 last meeting, we don’t think this enhancement will be introduced in this release. If no new mechanism is introduced, RAN4 need to use legacy requirements.

Proposal 5: For both MAC-CE based DL/UL TCI state switch delay and DCI based DL/UL TCI state switching delay, The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management.

#### Issue 1-7: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time? <Way forward > - Option 1: (Samsung) - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart. - Option 2: - For such case, No RAN4 requirements. - Option 3: (Apple) - Add the applicability rule of event triggered L1 measurement: - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE. - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

For event-2/event-1, the measurement is based on current beam. The RS of current beam is coming from indicated TCI state. If UE received TCI state switching based on MAC-CE or DCI, it means the indicated TCI state has been changed. The measurement of L1 evaluation time should be changed after the MAC-CE command or DCI indicator. If the event has been triggered, the L1 evaluation time should be restart.

Proposal 6: The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

#### Issue 1-9: Requirements for event-1 and event-7? <Way forward > - Option 1: When requirements for Event-2 are agreed, agree to reuse all requirements for Event-1 and Event-7. - Option 2: others are not precluded

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. - Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7.

For event-7, it involved all active TCI states measurement to choose Q-th best quality.

If Option 1 is Issue 1-2 can be agreed, the principle can be reused for event-1 and event-7 easily.

For example, for event-7, maximum L1-RSRP measurement period are accounted of all active TCI states.

Proposal 7: The principle of Event-2 can be used for event-1 and event-7 easily. For event-7, maximum L1-RSRP measurement period are accounted of all active TCI states.

#### Issue 1-10: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)? <Way forward > Further discuss the two options. Proponents of option 2 to bring proposals on the exact numbers for the capabilities. - Option 1: - For event trigger L1 reporting, do not define such capability in parallel per category like L3. - Option 2: - Define capabilities for support of UE-initiated/event-driven beam reporting similar as L3. #### Issue 1-11: Compatibility of capabilities with LTM <Way forward > - Option 1: (Apple, Ericsson) - Discuss it later. - Option 2: (CMCC, Huawei, Nokia) - Not preferred to mix Rel-19 mobility and Rel-19 MIMO. The capability for supported number of events triggered L1 measurement reporting is defined separately for Rel-19 mobility and Rel-19 MIMO - Option 3: (MediaTek) - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

In legacy L3 measurement in section 9.1.4, the UE shall be able to support in parallel per category up to Ecat reporting criteria according. Such as

is the total number of NR reporting criteria according to table 9.1.4.2-1, and n is the number of configured NR serving frequencies, including PCell, and SCells carrier frequencies,

Unlike L3 measurement, there is no different types of events. In L3, the different types of measurement can be measured and the serving cell and neighbour cells can measure at the same MO, the decided by software for different events whether they are met or not.

In L1 measurement, the measurement type is only based on RSRP.

For different events, they are configured from the CSI config. It will not be support to measure and report the different events at the same reporting. There is no need to define such capability for support of L1 event trigger and reporting as L3.

Proposal 8: For event trigger L1 reporting, do not define such capability in parallel per category like L3.

Proposal 9: If P8 is agreed, no need to consider such capability compatibility with LTM.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for UE-initiated/event-driven beam management in Rel-19 MIMO phase 5:

Proposal 1: If timeRestrictionForChannelMeasurement is not configured, reuse the same values of legacy, measurement period is based on 3 sample for SSB based and for periodic and semi-persistent CSI-RS resources.

Proposal 2: The event triggered measurement reporting delay is TL1_measurement period + Tfirst UL channel, if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams.

Proposal 3: RAN4 to specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19.

Proposal 4: Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

Proposal 5: For both MAC-CE based DL/UL TCI state switch delay and DCI based DL/UL TCI state switching delay, The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management.

Proposal 6: The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

Proposal 7: The principle of Event-2 can be used for event-1 and event-7 easily. For event-7, maximum L1-RSRP measurement period are accounted of all active TCI states.

Proposal 8: For event trigger L1 reporting, do not define such capability in parallel per category like L3.

Proposal 9: If P8 is agreed, no need to consider such capability compatibility with LTM.

- Reference

[1] R4-2504902, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung