3GPP TSG-RAN WG4 Meeting #115                                       	                        R4-2505854

# St. Julian’s, Malta, May 19 – May 23, 2025

Agenda item:	7.1.4

Source:	Apple Inc.

Title:	On UE Demodulation Performance Requirements for 6Rx

Release:	Rel-19

Document for:	Discussion

# Introduction

A new WID on RF enhancements for NR FR1/FR2 and EN-DC has been created for Release 19. The WID covers three main areas: High power UE (HPUE) for CA in terrestrial network (TN), power boosting and/or MPR reduction, and 6Rx for handheld and FWA UEs. The main objectives for 6Rx, as highlighted in the WID [1], are captured below:

Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution

Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding

After finalizing the discussion on the maximum number of MIMO layers for 6Rx handheld UEs, in this paper we propose the scope of the 6Rx Demodulation Performance and CSI Reporting Requirements.

# Discussion

During the last RAN4#114bis meeting there were a number of agreements on the scope of demodulation and CSI reporting requirements and on specific simulation assumptions.

Observation #1: No new PDCCH or PBCH demodulation requirements are to be introduced. New PDSCH requirements will be introduced only for single carrier and no new requirements will be introduced for CA related tests.

In order to capture the outcome of the core requirements phase, companies have already started discussing how to specify applicability rules. Some companies propose that RAN4 can introduce UE declaration on the 6Rx UE type, which means whether the UE is a handheld device or a FWA device, without capability signalling. At the moment, RAN4 is already specifying separate test for each UE type, so the only contentious point is whether RAN2 will introduce this capability.

Observation #2: After the LS submitted during RAN4#114bis, discussion on capability signaling in on RAN2 discussion, and this parallel discussion does not preclude RAN4 to continue making progress.

Proposal #1: Continue defining separate requirements for 6 MIMO layers for FWA and 4 MIMO layers for handheld UEs. Wait until RAN2’s capability signaling discussion is concluded before finalizing the wording of the applicability rules.

Another important point with respect to the context of this work item is to confirm the specific test scenarios being considered. One company has proposed that in addition to the baseline scenario, also scenarios with intra-cell and inter-cell interference should be considered when defining PDSCH requirements. Since the only change between a 4Rx UE and a 6Rx 4-layers handheld UE is the number of antennas, and the baseband functionality does not change. Because of this, we propose that all inter-cell and intra-cell scenarios should be tested as part of the legacy test cases. Hence, no new test cases being defined for cases with intra/inter-cell interference.

  - Observation #3: Scenarios with intra/inter-cell interference are already well covered with the legacy test cases for up to 4Rx.

Proposal #2: Discuss inter/intra-cell interference scenarios once we finalize defining test parameters for PDSCH single-carrier requirements.

For the remainder of this work item, the most important work should be in defining the new PDSCH requirements. Based on previous experience during Rel-18 8Rx discussion, we believe only a single PDSCH test should suffice for each case. In addition, we believe that a FWA device should only pass a single test case, with 6 MIMO layers, and if this test passes, this FWA UE should be allow to omit the 4 MIMO layers test case since all new relevant functionality is already tested.

  - Observation #4: For a 6Rx FWA device, a performance requirement that focuses on 6 MIMO layers exercises the same new funcitonality that a test focusing on 4 MIMO layers.

  - Proposal #3: Define PDSCH requirements for 6Rx UE and capture in test applicability rules that 1) for 6 MIMO layers performance requirements will only apply for FWA devices, while the 4 MIMO layers test case is reserved for handheld UEs.

  - Proposal #4: Discuss the case where if a device passed the 6Rx 4 MIMO layers test case, the UE should be allowed to skip one or multiple 4Rx 4 MIMO layers test cases.

After RAN4#114bis, propagation scenarios and antenna correlation modesl have not been fully agreed. For the propagation conditions for 6 MIMO layers requirements, since this is a FWA targeted test, we believe this should be reflected in the definition of this test. Because of this, a proper propagation channel should be TDLA30-5, since it provides reasonable frequency selectivity while keeping the doppler spread to a minimum. In addition, since no mobility is present, the network does not need to configure an additional DMRS symbol since doppler spread is quite low, only accounting for scattering but no UE mobility. Configure 2+2 for 6 layer scenario is not the best. As of today, reconfiguration of additional DMRS positions remains controlled by RRC, which is too slow, and higher rank transmissions likely to be scheduled in low mobility scenarios. Finally, since FWA devices have a relatively large form factor compared to handheld UEs, it is then reasonable to assume ULA Low as the antenna correlation model.

  - Observation #5: The 6 MIMO layers PDSCH requirement should reflect in its test parameters that this is a test targeted for FWA devices that experience virtually no mobility. As of today, a single parameter/capability that indicates DCI-based dynamic AddPos scheduling across DMRS capabilities does not exist.

Proposal #5: For 6 MIMO layers, consider TDLA30-5 as the propagation channel.

Proposal #6: For 6 MIMO layers, consider ULA Low as the antenna correlation model.

Proposal #7: For 6 MIMO layers, consider 2+0 as the DMRS configuration.

For the 4 MIMO layers test, since it’s targeted for a handheld UE with limited space and relatively small form factor, parameters should be different. Because of this, a proper propagation channel should be TDLA30-10, since it provides reasonable frequency selectivity while allowing a somewhat higher doppler spread. Since most of the gains of 6Rx vs 4Rx for handheld devices are related to diversity gain, we believe that a relatively low mobility scenario should be targetted, since we still care more for the high throughput conditions. On the other hand, since handheld UEs indeed present form factor constraints, we believe a good approach would be to consider a variation of the existing MediumB antenna correlation model. However, as discussed during the feasibility study before, BS transmitted correlation in MediumB seems quite pessimistic. Hence, we believe a good compromise for testing purposes is to consider Alpha = 0.0 while Beta = 0.005154.

  - Observation #6: The 6 MIMO layers PDSCH requirement should reflect in its test parameters that this is a test targeted for FWA devices that experience virtually no mobility.

Observation #7: Type-1 and Type-2 DMRS are typical configurations observed in the field.

Proposal #8: For 4 MIMO layers, consider TDLA30-10 as the propagation channel.

Proposal #9: For 4 MIMO layers, consider ULA with Alpha = 0.0 and Beta = 0.005154 for defining the antenna correlation model.

Proposal #10: For 4 MIMO layers, consider 1+1 as the DMRS configuration.

In addition to this, we want to reiterate the antenna configuration approach used for defining performance requirements. For the 4 MIMO layers case, we don’t see a justification for entertaining the use of more than 4 transmit antennas. Since the number of layers is 4, this is enough for the purpose of the test.

Observation #8: For testing and performance requirement purposes, there’s no justification to increase the number of transmit antennas beyond 4 when considering 6Rx and 4 MIMO layers.

Proposal #11: Consider a transmit/receive antenna configuration of 8Tx/6Rx for 6 MIMO layers, and a transmit receive antenna configuration of 4Tx/6Rx for 4 MIMO layers.

Finally, to complete the setup for PDSCH requirements we need to consider initial simulation results. Find below the relative throughput curves for the configurations proposed for 4 MIMO layers and 6 MIMO layers.

Figure 1: Relative throughput for Rank-4 and Rank-6 transmission.

In both cases we observe that 256QAM data points operate at a too high SNR to be tested, but we would like to see curves to achieve full throughput well before the 30dB mark. For this, we believe 64QAM MCSs are sufficient for the purposes of these tests.

Observation #9: Our simulation results suggest that 256QAM modulation is not feasible for testing purposes with the proposed test configurations for 4 and 6 MIMO layers.

Proposal #12: For 4 MIMO layers PDSCH requirements, consider Table 1 MCS 19.

Proposal #13: For 6 MIMO layers PDSCH requirements, consider Table 1 MCS 19.

Furthermore, we would like to address the purpose of the SDR requirements, and two aspects have been raised. First, about the static channel and relevant precoding for determine the requirement for 6 MIMO layers with 64QAM and 256QAM. We believe the easiest way to move forward is to fix the precoder to determine the maximum MCS for each case. SDR were already defined for 8Rx so we believe they should be defined for 6Rx as well.

During the past meeting it was proposed to determine a specific static channel matrix. We believe RAN4 should consider that matrix as a good starting point, unless other companies can propose something more appropriate. From our view, we believe this matrix is sufficient.

Observation #10: SDR requirements were defined for 8Rx UEs.

Proposal #14: Define SDR requirements for 6Rx UEs and consider the folliwing matrix to define the static channel

H=wHw-1wH=34*11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

In addition, it has been proposed that SDR requirements are defined for 1024QAM with 2 layers. However, due to previous work item decisions, we believe 2 layers is consistent and should be maintained.

Proposal #15: Do not define SDR requirements for 6Rx UEs for more than 2 layers with 1024QAM modulation.

Finally, we propose to based the CSI requirements discussion again on previous experience with Rel-18 8Rx. For CQI reporting requirements, we are not sure about the specific motivation. At best, we believe static channel should be the minimum consideration. At this point, we prefer not to define new CQI reporting requirements for 6Rx unless a clear motivation is confirmed.

Proposal #16: Do not define CQI reporting requirements specifically for 6Rx UEs.

In addition, we don’t see much value in RI reporting requirements since we did not define such requirements in Rel-18 for 8Rx.

Proposal #17: Do not define RI requirements for 6Rx UEs.

# Conclusions

Our observations and proposals are summarized below:

Observation #1: No new PDCCH or PBCH demodulation requirements are to be introduced. New PDSCH requirements will be introduced only for single carrier and no new requirements will be introduced for CA related tests.

Observation #2: After the LS submitted during RAN4#114bis, discussion on capability signaling in on RAN2 discussion, and this parallel discussion does not preclude RAN4 to continue making progress.

Proposal #1: Continue defining separate requirements for 6 MIMO layers for FWA and 4 MIMO layers for handheld UEs. Wait until RAN2’s capability signaling discussion is concluded before finalizing the wording of the applicability rules.

  - Observation #3: Scenarios with intra/inter-cell interference are already well covered with the legacy test cases for up to 4Rx.

Proposal #2: Discuss inter/intra-cell interference scenarios once we finalize defining test parameters for PDSCH single-carrier requirements.

  - Observation #4: For a 6Rx FWA device, a performance requirement that focuses on 6 MIMO layers exercises the same new funcitonality that a test focusing on 4 MIMO layers.

  - Proposal #3: Define PDSCH requirements for 6Rx UE and capture in test applicability rules that 1) for 6 MIMO layers performance requirements will only apply for FWA devices, while the 4 MIMO layers test case is reserved for handheld UEs.

  - Proposal #4: Discuss the case where if a device passed the 6Rx 4 MIMO layers test case, the UE should be allowed to skip one or multiple 4Rx 4 MIMO layers test cases.

  - Observation #5: The 6 MIMO layers PDSCH requirement should reflect in its test parameters that this is a test targeted for FWA devices that experience virtually no mobility. As of today, a single parameter/capability that indicates DCI-based dynamic AddPos scheduling across DMRS capabilities does not exist.

Proposal #5: For 6 MIMO layers, consider TDLA30-5 as the propagation channel.

Proposal #6: For 6 MIMO layers, consider ULA Low as the antenna correlation model.

Proposal #7: For 6 MIMO layers, consider 2+0 as the DMRS configuration.

  - Observation #6: The 6 MIMO layers PDSCH requirement should reflect in its test parameters that this is a test targeted for FWA devices that experience virtually no mobility.

Observation #7: Type-1 and Type-2 DMRS are typical configurations observed in the field.

Proposal #8: For 4 MIMO layers, consider TDLA30-10 as the propagation channel.

Proposal #9: For 4 MIMO layers, consider ULA with Alpha = 0.0 and Beta = 0.005154 for defining the antenna correlation model.

Proposal #10: For 4 MIMO layers, consider 1+1 as the DMRS configuration.

Observation #8: For testing and performance requirement purposes, there’s no justification to increase the number of transmit antennas beyond 4 when considering 6Rx and 4 MIMO layers.

Proposal #11: Consider a transmit/receive antenna configuration of 8Tx/6Rx for 6 MIMO layers, and a transmit receive antenna configuration of 4Tx/6Rx for 4 MIMO layers.

Observation #9: Our simulation results suggest that 256QAM modulation is not feasible for testing purposes with the proposed test configurations for 4 and 6 MIMO layers.

Proposal #12: For 4 MIMO layers PDSCH requirements, consider Table 1 MCS 19.

Proposal #13: For 6 MIMO layers PDSCH requirements, consider Table 1 MCS 19.

Observation #10: SDR requirements were defined for 8Rx UEs.

Proposal #14: Define SDR requirements for 6Rx UEs and consider the folliwing matrix to define the static channel

H=wHw-1wH=34*11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

Proposal #15: Do not define SDR requirements for 6Rx UEs for more than 2 layers with 1024QAM modulation.

Proposal #16: Do not define CQI reporting requirements specifically for 6Rx UEs.

Proposal #17: Do not define RI requirements for 6Rx UEs.

# References

- RP-240720 “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, by RAN4 chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024