3GPP TSG-RAN WG4 Meeting #115 	R4-2505859

# St. Julian’s, Malta, May 19 – May 23, 2025

Agenda item:	7.3.5

Source:	Apple Inc.

Title:	On UE Demodulation Performance Requirements for NR Channel BW less than 5MHz for FR1 Phase 2

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

3GPP Rel-8 TS 36.101 specified various channel bandwidths for 4G LTE systems including 1.4, 3, 5, 10, 15 and 20 MHz channel sizes. Global regulators and mobile network operators allocated and deployed networks occupying these various channel bandwidths that are still operational today. Allocation of additional spectrum is prohibited in nearly all cases due to existing use of the adjacent spectrum. Most of these networks do not utilize NB-IoT but are fully mobile networks that utilize CAT-4 and higher devices. Certain licensees operate regionally and nationally in the USA, internationally across Europe and in parts of SE Asia and Africa. These networks support the family of 3GPP technologies including GSM and LTE for mission critical applications. These operational networks are now looking to invest in the future of 3GPP by deploying 5G services. This includes the use of NR to support multiple low latency use cases (see 3GPP TR 38.825) resulting from various vertical industry domains, e.g., electrical power distribution grid & rail communication. These uses cases often require less than the latency offered by LTE technology and thus need NR to fully support them.

During Rel-18 the feature Dedicated Spectrum of Less than 5MHz was finalized and RAN4 specified requirements to support deploying NR in spectrum allocations from approximately 3 MHz up to below 5 MHz, in bands including n100, n106, n26, n28, n31, n72 and n85. For this Phase 2, the Carrier Aggregation scenario of a 3MHz carrier with one or more 3MHz or wider bandwidth carriers will increase the total available network capacity and hence the value of the 3MHz carrier. In fact, it in some cases enables the use of spectrum blocks that otherwise would not be used at all for 5G. In addition, Dual Connectivity of a 3MHz carrier with one or more 3MHz or wider bandwidth carriers will increase radio coverage and cell hand-over performance. For railways the latter is particularly relevant for handovers for example between tunnel entries or exits, and the open field network coverage areas.

# 2. Discussion

The main objective of this work item is to define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band. The latest WID states an example band combination of CA/DC of “3MHz or 5MHz” in band n100 and 5MHz or 10MHz in band n101 - with the caveat that the 5MHz channel bandwidth in the above mentioned “3MHz or 5MHz” is specific to band n100 and does not apply to other bands.

During the past Release 18, more specifically in RAN4#109, it was agreed that for the purpose of demodulation performance requirements and coverage, it is assumed there will be no UEs that exclusively support a maximum a channel bandwidth of 5MHz. Hence, the conclusion of this work item was that all previous PDSCH requirements of past releases can be applied to a UE that supports both Less than 5MHz channel bandwidth and legacy bandwidths of larger size.

Observation 1: For the purposes of Rel-18 demodulation performance requirements, it was assumed there will be no UEs that exclusively support a maximum channel bandwidth of 5 MHz. A UE that supports <5MHz will also support 10MHz or more.

Observation #2: The PDSCH requirements implication is that since in such UE PDSCH is already exercised for larger CBWs, there is no need for additional tests for smaller CBWs — any small difference in performance is already expected due to reduced different transport block size and different number of code blocks.

On the other hand, the performance requirement in TS 38.101-4 for CA UE demodulation tests in Clause 5.2A are defined independent of CA configurations and bandwidth combination sets specified in Clause 5.5A of TS 38.101-1. For UEs supporting different CA configurations and bandwidth combination sets, the applicability and test rules are defined in TS 38.101-4 in Tables 5.1.1.7.2-1 and 5.1.1.7.2-2.

Observation #3: As per TS 38.101-4, PDSCH CA requirements are not tied to specific band combinations.

From the table above, we see that we select the CA configuration that contains CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate, among all selected CA configurations.

Observation #4: For CA requirements, in Rel-18 we test the "Largest aggregated CA bandwidth combination”. For UEs supporting 10MHz or more, it’s not the case that the largest aggregated CA bandwidth will be obtained by one or two carriers being of the type 'Less than 5MHz”.

Hence, by the above, we believe that for this Phase 2 nothing has fundamentally changed from what has been discussed in Rel-18, and this issue should be concluded during this meeting.

Proposal #1: Maintain current assumption that no product is expected to launch that supports only <5MHz channels. RAN4 to confirm the case that if a UE has implemented a CA configuration with for example CC1 (10MHz) + CC2 (3MHz), it most likey also support another CA configuration with CC1 (10MHz) + CC2 (>3MHz), which will be selected for CA test.

Because of the above, we believe our proposal remains valid for Rel-19.

Proposal #2: Do not introduce new single-carrier PDSCH requirements for <5MHz channels, and do not define new PDSCH CA requirements for <5MHz channels.

In addition, during the past meeting, there was a suggestion that even though the existing test applicability rule for PDSCH CA requirements in 5.1.1.7 in TS38.101-4 can be reused, other options are not precluded. In our opinion, existing applicability ruse are to be reused.

Proposal #3: Do not modify existing test applicability rules for PDSCH CA requirements in 5.1.1.7 in TS38.101-4 can be reused since they remain valid.

In addition, during the last meeting it was agreed that one of the candidate options is for RAN4 to discuss whether there are SDR requirement impact for UE supporting 3MHz channel bandwidth in CA. The purpose of the SDR test is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. More specifically, the sustained downlink data rate shall be verified in terms of the success rate of delivered PDCP SDU(s) by Layer 2. The SDR test case specifies the RF conditions and the required success rate of delivered TB by Layer 1 to meet the sustained data rate requirement which shall be higher than 85% when PDSCH is scheduled with MCS defined for the selected CA. But again, this test is inherently testing the baseband handling of the CA operation, which is already tested with higher bandwidths of other CA combinations, so a UE that support higher bandwidths in addition to 3MHz will always use a CBW other than 3MHz to perform this test.

Proposal #4: Do not define new SDR requirements for <5MHz channels.

Finally, companies have also propose to consider CA CQI reporting requirements. As taken from TS 38.101-4, the performance requirement for CA CQI tests in clause 6.2A, similar to CA PDSCH tests, are defined independent of CA configurations and bandwidth combination sets specified in clasue 5.5A in TS 38.101-1.

Moreover, for UEs supporting multiple CA capabilities, requirements allow testing any one of the supported CA capabilities with largest aggregated CA bandwidth combination, and UEs supporting multiple CA configurations from the selected CA capability, they can test any one of the supported CA configurations with largest aggregated CA bandwidth combination.

Observation #5: For CA CQI requirements, there’s enough flexibility in TS 38.101-4 to test any supported CA capability and any supported CA configuration already.

Proposal #6: Do not define new CA CQI reporting requirements specifically for 3MHz channel bandwidth.

# 3. Conclusion

Our observations and proposals are stated below:

Observation 1: For the purposes of Rel-18 demodulation performance requirements, it was assumed there will be no UEs that exclusively support a maximum channel bandwidth of <5 MHz. A UE that supports <5MHz will also support 10MHz or more.

Observation #2: The PDSCH requirements implication is that since in such UE PDSCH is already exercised for larger CBWs, there is no need for additional tests for smaller CBWs — any small difference in performance is already expected due to reduced different transport block size and different number of code blocks.

Observation #3: As per TS 38.101-4, PDSCH CA requirements are not tied to specific band combinations.

Proposal #1: Maintain current assumption that no product is expected to launch that supports only <5MHz channels. RAN4 to confirm the case that if a UE has implemented a CA configuration with for example CC1 (10MHz) + CC2 (3MHz), it most likey also support another CA configuration with CC1 (10MHz) + CC2 (>3MHz), which will be selected for CA test.

Proposal #2: Do not introduce new single-carrier PDSCH requirements for <5MHz channels, and do not define new PDSCH CA requirements for <5MHz channels.

Observation #4: For CA requirements, in Rel-18 we test the "Largest aggregated CA bandwidth combination”. For UEs supporting 10MHz or more, it’s not the case that the largest aggregated CA bandwidth will be obtained by one or two carriers being of the type 'Less than 5MHz”.

Proposal #4: Do not define new SDR requirements for <5MHz channels.

Observation #5: For CA CQI requirements, there’s enough flexibility in TS 38.101-4 to test any supported CA capability and any supported CA configuration already.

Proposal #6: Do not define new CA CQI reporting requirements specifically for 3MHz channel bandwidth.

# References

- R4-2321064, “WF on NR_FR1_lessthan_5MHz_BW_demod”, Nokia.