3GPP TSG-RAN WG4 Meeting #115						R4-2505900

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.8.6

Source:	Apple

Title:	On UE Demodulation and CSI Reporting Requirements for 7MHz

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

The performance part for Rel-19 NR FR1 7MHz Channel Bandwidth WI starts in RAN4#115. In this contribution we propose the scope of UE demodulation requirements for this WI.

# 2. Discussion

Based on the latest WID [1] the performance part objectives of the work item are:

We discuss the scope of UE demodulation and CSI reporting requirements for 7 MHz channel bandwidth.

7 MHz is supported in FDD bands. There are several channel BWs supported in FDD, but we introduce single carrier demodulation and CSI reporting requirements only with 10MHz CBW. To cover all the supported CBWs in FDD we introduce CA requirements. For the new 7MHz CBW, we propose to use the same approach and introduce only CA requirements in FDD.

- Introduce CA requirement for PDSCH with 7MHz CBW in FDD for coverage.

The CA requirements for PDSCH cover PDSCH mapping Type A, HST-SFN and HST-DPS features. We think it is sufficient to introduce CA requirement with 7 MHz only for PDSCH mapping Type A. Further discuss if HST-SFN and HST-DPS should be covered for 7MHz.

- Introduce CA requirement for PDSCH mapping Type A with 7MHz CBW.

- Further discuss if PDSCH CA requirements for HST-SFN and HST-DPS should be introduced with 7MHz CBW.

Since 7MHz support is introduced in bands n5, n26, which are low frequency bands and not bands for mandatory 4RX operation, it is sufficient to introduce CA requirements with 7MHz only for 2RX.

- Introduce CA requirement with 7MHz CBW for 2RX only.

The same test parameters for PDSCH CA requirements in 38.101-4 §5.2A can be reused. For 7MHz the PDSCH bandwidth and CORESET bandwidth need to be updated. Maximum allocation of 35 PRB is supported for 7 MHz CBW. Use PDSCH BW of 35 PRBs and CORESET BW of 30 PRBs.

- Reuse the same test parameters as PDSCH CA requirements, with the following update:
Num PRB PDSCH: 35
CORESET Bandwidth: 30

# 3. Conclusion

In this paper, we propose the scope of UE demodulation requirements for 7MHz CBW. Our observations and proposals are captured below:

- Introduce CA requirement for PDSCH with 7MHz CBW in FDD for coverage.

- Introduce CA requirement for PDSCH mapping Type A with 7MHz CBW.

- Further discuss if PDSCH CA requirements for HST-SFN and HST-DPS should be introduced with 7MHz CBW.

- Introduce CA requirement with 7MHz CBW for 2RX only.

- Reuse the same test parameters as PDSCH CA requirements, with the following update:
Num PRB PDSCH: 35
CORESET Bandwidth: 30

# Reference

- RP-250373, “Revised WID on NR FR1 7 MHz Channel Bandwidth”, T-Mobile USA, China Telecom.