3GPP TSG-RAN WG4 Meeting #115						R4-2505902

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.16.3

Source:	Apple

Title:	Simulation results for Spatial Channel Modeling study

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#114bis spatial channel model for demodulation requirements was discussed and way forward [1] was approved. In this contribution present simulation results for spatial channel model study.

# 2. Simulation Results

For the simulation parameters below, we present our results.

Table 1: Simulation Parameters for Spatial Channel Model Study

| Parameter |  | SU-MIMO PDSCH (FR1) | SU-MIMO PMI (FR1) |
| --- | --- | --- | --- |
| Duplex mode |  | TDD |  |
| TDD Slot Configuration Pattern |  | 7DS2U |  |
| FR / Carrier frequency |  | FR1 / 3.5 GHz |  |
| UE receiver type |  | IRC Baseline | IRC Baseline |
| Number of Tx Ports |  | 4 | 8,16,32 |
| Number of Rx Ports/Antennas |  | 4 | 4 |
| Number of layers |  | 4 | 4,2 |
| PMI |  | Random | Follow, Random |
| Waveform |  | CP-OFDM with normal CP |  |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |  |
| MCS |  | 13 (64 QAM table) |  |
| PDSCH configuration | Mapping type | Type A |  |
|  | k0 | 0 |  |
|  | Starting symbol (S) | 2 |  |
|  | Length (L) | 12 |  |
|  | PDSCH aggregation factor | 1 |  |
|  | Resource allocation type | Type 0 |  |
|  | VRB-to-PRB mapping type | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |  |
| PDSCH DMRS configuration | DMRS Type | Type 1 |  |
|  | Number of additional DMRS | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) 1 (for rank <= 4) |  |
| Codebook configuration | CodebookType | type-I-SP | type-I-SP (for follow/ random), eType-II (for follow) |
|  | Codebook configuration | For 4Tx (N1,N2,O1,O2) = (2,1,4,1) | For 8TX: (N1,N2,O1,O2) = (4,1,4,1) For 16 TX: (N1,N2,O1,O2) = (8,1,4,4) For 32 TX: (N1,N2,O1,O2) = (8,2,4,4) |
| PDSCH DMRS Precoding Configuration |  | For Random precoding: for every PRB Bundle (size=2) | WB PMI for follow Type I WB PMI for random Type I SB PMI for eType II |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type | Aperiodic |  |
|  | Number of CSI-RS ports (X) | 4 CSI-RS Ports (2,1) for 4 Layer | 8 CSI-RS Ports (4,1) 16 CSI-RS Ports (8,1) 32 CSI-RS Ports (8,2) |
|  | Density (ρ) | 1 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) | (13) |  |
| CQI/RI/PMI delay |  | N/A | 7 ms |
| Number of HARQ Processes |  | 8 |  |
| Maximum HARQ transmissions |  | 4 |  |

Channel Models:

- TDLA30

- TDLC300

- UMa-CDL-C-A2: CDL (TR 38.753) based on untruncated CDL-C from Table 7.7.1-3 in TR 38.901(provided in Appendix)

  - With ray-splitting for cluster 2-4, 6-8

- UMa-CDL-C-A4: CDL (TR 38.753) based on truncated CDL-C from Table 7.7.1-3 in TR 38.901 (provided in Appendix)

Doppler/Speed:

For TDL Models: 10, 100 Hz

For CDL Models: 3, 30 kph

AAV Configuration:

AAV with Sub-array AAV 1Y

  - (M,N,P,Ms,Ns) = (8,4,2,8,1) for 4Tx CSI-RS Ports

Pass through – AAV 3

  - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

  - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

  - (M,N,P,Ms,Ns) = (1,8,2,1,1) for 16Tx CSI-RS Ports

  - (M,N,P,Ms,Ns) = (2,8,2,1,1) for 32Tx CSI-RS Ports

SU-MIMO PDSCH Performance

In the table we provide simulation results for alignment and comparison.

Table 2:Simulation results for SU-MIMO PDSCH for alignment and comparison

| Channel Model w/ Ant Corr | Doppler/ Speed | AAV | SNR @ 70% Max TP | SNR @ 30% Max TP |
| --- | --- | --- | --- | --- |
| UMa-CDLC-A2 | 3 kph | AAV3 | 15.3 | 8.1 |
| UMa-CDLC-A4 | 3 kph | AAV3 | 15.2 | 8.8 |
| UMa-CDLC-A2 | 3 kph | AAV1Y | 15.8 | 7.6 |
| UMa-CDLC-A4 | 3 kph | AAV1Y | 15.6 | 7.6 |
| TDLA-30 ULA Low | 10 Hz | N/A | 13.2 | 6.2 |
| TDLA-30 XP Med | 10 Hz | N/A | 14.6 | 7.5 |
| UMa-CDLC-A2 | 30 kph | AAV3 | 16.5 | 8.9 |
| UMa-CDLC-A4 | 30 kph | AAV3 | 16.2 | 8.8 |
| UMa-CDLC-A2 | 30 kph | AAV1Y | 16.3 | 7.7 |
| UMa-CDLC-A4 | 30 kph | AAV1Y | 16.0 | 7.4 |
| TDLC-300 ULA Low | 100 Hz | N/A | 14.6 | 7.0 |
| TDLC-300 XP Med | 100 Hz | N/A | 16.0 | 7.8 |

Performance with UMa-CDL C is not better than TDL channel model.

SU-MIMO PMI-PDSCH Performance

The TP gain with Type I and eType II are presented in the table below. For PMI reporting, only AAV3 is used.

Table 3: SU-PMI simulation results with 8TX

| Channel /MIMO Corr | Doppler/Speed | Num Layers | g Type I |  | SNR @ 90% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | eType II | Type I | eType II | Type I |
| TDLC300 XP-Low | 10 Hz | 2 | 1.33 | 1.01 | 5.4 | 7.0 |
| TDLC300 XP-Med |  |  | 2.15 | 2.22 | 3.7 | 3.6 |
| TDLC300 XP-High |  |  | 2.63 | 2.96 | 4.3 | 3.6 |
| UMa-CDL-C-A2 | 3 kph |  | 1.66 | 1.59 | 6.6 | 6.9 |
| UMa-CDL-C-A4 |  |  | 1.57 | 1.55 | 7.6 | 7.7 |
| TDLC300 XP-Low | 10 Hz | 4 | 1.06 | 1.00 | 13.6 | 13.7 |
| TDLC300 XP-Med |  |  | 1.99 | 1.96 | 13.8 | 14.3 |
| TDLC300 XP-High |  |  | 2.09 | 2.00 | 19.2 | 21.0 |
| UMa-CDL-C-A2 | 3 kph |  | 1.16 | 1.38 | 15.8 | 14.7 |
| UMa-CDL-C-A4 |  |  | 1.23 | 1.42 | 16.5 | 15.5 |
| TDLC300 XP-Low | 10 Hz | 2 | 1.00 | 1.00 | 7.9 | 8.5 |
| TDLC300 XP-Med |  |  | 2.00 | 2.05 | 5.5 | 4.3 |
| TDLC300 XP-High |  |  | 2.36 | 2.61 | 5.3 | 4.5 |
| UMa-CDL-C-A2 | 3 kph |  | 1.26 | 1.14 | 9.3 | 9.7 |
| UMa-CDL-C-A4 |  |  | 1.40 | 1.25 | 8.7 | 9.3 |
| TDLC300 XP-Low | 10 Hz | 4 | 1.00 | 1.00 | 16.5 | 15.5 |
| TDLC300 XP-Med |  |  | 1.90 | 1.92 | 15.6 | 15.4 |
| TDLC300 XP-High |  |  | 2.09 | 2.00 | 19.7 | 22.3 |
| UMa-CDL-C-A2 | 3 kph |  | 1.02 | 1.30 | 18.1 | 16.5 |
| UMa-CDL-C-A4 |  |  | 1.18 | 1.46 | 17.2 | 15.9 |

Table 4: SU-PMI simulation results with 16TX

| Channel /MIMO Corr | Doppler/Speed | Num Layers | g Type I |  | SNR @ 90% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | eType II | Type I | eType II | Type I |
| TDLC300 XP-Low | 10 Hz | 2 | 1.98 | 1.05 | 3.9 | 5.9 |
| TDLC300 XP-Med |  |  | 2.79 | 2.54 | 2.3 | 2.9 |
| TDLC300 XP-High |  |  | 13.83 | 38.90 | 1.9 | 1.4 |
| UMa-CDL-C-A2 | 3 kph |  | 2.82 | 2.39 | 3.3 | 4.4 |
| UMa-CDL-C-A4 |  |  | 2.61 | 2.55 | 4.7 | 4.9 |
| TDLC300 XP-Low | 10 Hz | 4 | 1.15 | 1.00 | 13.2 | 13.7 |
| TDLC300 XP-Med |  |  | 2.02 | 1.97 | 11.9 | 13.0 |
| TDLC300 XP-High |  |  | 3.32 | 2.97 | 14.5 | 15.5 |
| UMa-CDL-C-A2 | 3 kph |  | 2.23 | 1.99 | 12.9 | 15.0 |
| UMa-CDL-C-A4 |  |  | 2.48 | 1.97 | 13.3 | 16.3 |

Table 5: SU-PMI simulation results with 32TX

| Channel /MIMO Corr | Doppler/Speed | Num Layers | g Type I |  | SNR @ 90% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | eType II | Type I | eType II | Type I |
| TDLC300 XP-Low | 10 Hz | 2 | 1.99 | 1.16 | 3.7 | 5.7 |
| TDLC300 XP-Med |  |  | Inf | Inf | -0.7 | -0.6 |
| TDLC300 XP-High |  |  | Inf | Inf | -0.9 | -2.0 |
| UMa-CDL-C-A2 | 3 kph |  | 21.35 | 5.98 | 1.1 | 2.4 |
| UMa-CDL-C-A4 |  |  | 9.14 | 5.76 | 2.4 | 3.3 |
| TDLC300 XP-Low | 10 Hz | 4 | 1.42 | 1.00 | 12.1 | 13.6 |
| TDLC300 XP-Med |  |  | 5.85 | 4.18 | 9.2 | 9.7 |
| TDLC300 XP-High |  |  | Inf | 149.75 | 11.4 | 13.0 |
| UMa-CDL-C-A2 | 3 kph |  | 3.21 | 2.22 | 10.3 | 12.8 |
| UMa-CDL-C-A4 |  |  | 3.47 | 2.43 | 10.9 | 13.2 |

Based on the results, we have the following observations:

- eType II performance is better than Type I with larger CSI-RS ports even with no AAV

- The performance improvement with eType II can be seen in legacy TDL and well as CDL channel models

- X-pol with Medium / High antenna correlation are not suitable for large number of TX ports/ antenna for > 2layer

eType II performance is better than Type I with larger CSI-RS ports even with no AAV.

The performance improvement with eType II can be seen in legacy TDL and well as CDL channel models

X-pol with Medium / High antenna correlation are not suitable for large number of TX ports/ antenna for > 2layer

For eType II we compare performance between different channel models – existing TDL channel models and Uma-CDL-C.

Figure 3: Follow PMI with eType II performance comparison – 8TX

Figure 4: Follow PMI with eType II performance comparison – 16 / 32TX

Based on the results in figure 2, the performance with follow PMI with eType II is not significantly better than existing TDL channel models with/ without correlation.

The performance of follow PMI with eType II is not significantly better than existing TDL channel models.

# 3. Conclusion

In this paper, we provide our simulation results for study of spatial channel models. Our observations are captured below:

SU-MIMO PMI PDSCH performance

- eType II performance is better than Type I with larger CSI-RS ports even with no AAV.

The performance improvement with eType II can be seen in legacy TDL and well as CDL channel models

X-pol with Medium / High antenna correlation are not suitable for large number of TX ports/ antenna for > 2layer

The performance of follow PMI with eType II is not significantly better than existing TDL channel models.

# Reference

- R4-2504756, “Way Forward for [114bis][320] NR_SCM”, Nokia.

# Annex A: CDL Tables

Table A-1: UMa-CDLC-A2 with 2ns delay quantization

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 77 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 81 | -3.5 | -21.8 | 118.9 | 98.5 | 71.1 |
| 4 | 85 | -5.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 5 | 79 | -2.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 6 | 232 | 0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 7 | 235 | -2.2 | -5.9 | 171.2 | 99.2 | 76.6 |
| 8 | 239 | -3.9 | -5.9 | 171.2 | 99.2 | 76.6 |
| 9 | 240 | -7.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 10 | 290 | -7.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 11 | 300 | -10.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 12 | 341 | -11.1 | -73.0 | 43.1 | 92.3 | 50.8 |
| 13 | 448 | -5.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 14 | 478 | -6.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 15 | 792 | -8.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 16 | 989 | -13.2 | 64.6 | 26.3 | 105.2 | 121.0 |
| 17 | 1554 | -13.9 | 62.2 | -22.6 | 94.0 | 53.9 |
| 18 | 1679 | -13.9 | 55.0 | -1.6 | 91.8 | 61.6 |
| 19 | 2004 | -15.8 | -76.3 | -19.8 | 90.7 | 37.5 |
| 20 | 2047 | -17.1 | 57.4 | 4.5 | 108.2 | 53.1 |
| 21 | 2302 | -16 | 58.1 | 0.2 | 91.7 | 35.8 |
| 22 | 2423 | -15.7 | 67.3 | -4.9 | 91.6 | 52.9 |
| 23 | 2571 | -21.6 | 76.2 | -28.3 | 106.4 | 46.4 |
| 24 | 3158 | -22.8 | -91.4 | 29.3 | 109.5 | 44.7 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 1.4 | 15.6 | 3.6 | 12.2 | 7 |

Table A-2: UMa-CDLC-A4 with 2ns delay quantization (used for both AAV options)

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 132 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 138 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 402 | 0.0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 416 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 502 | -10.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 7 | 518 | -13.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 8 | 776 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 9 | 826 | -9.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 10 | 1372 | -11.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 11 | 1712 | -16.2 | 64.6 | 26.3 | 105.2 | 121.0 |
| 12 | 2906 | -16.9 | 55.0 | -1.6 | 91.8 | 61.6 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 1.4 | 15.6 | 3.6 | 12.2 | 7 |