3GPP TSG-RAN WG4 Meeting #115						R4-2505908

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.24.4.1

Source:	Apple

Title:	On RRM Requirements for UE initiated beam management

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#114bis the RRM requirements for UE initiated beam management was discussed and WF [1] was approved. In this contribution we present our views on RRM core requirements for UE initiated beam management.

# 2. Discussion

In [1] the following agreements were made for RRM core requirements with UE initiated beam management:

#### Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)? Agreement: - For T_RS (TSSB or TCSI-RS) in L1_measurement periods - T_RS (TSSB or TCSI-RS): - the value of current beam and new beams are the same. - For M in L1_measurement periods: - If timeRestrictionForChannelMeasurement configured, M= 1, event evaluation should be based on the most recent result from both new beam and current beam - If timeRestrictionForChannelMeasurement is not configured, - Further discuss on: - Option1: Reuse the same values of legacy, measurement period is based on 3 sample. No further restriction of UE implantation of which one is used - Option 2: No requirements even the NW can configure it. - Option 3: FFS - For P (sharing factor) in L1_measurement periods: - For SSB based or CSI-RS based, - The definition is the same as legacy - The value can be the same or different for the current beam and new beam #### Issue 1-2: Components of Event triggered measurement reporting delay? Agreement: - The event triggered measurement reporting delay includes the Tfirst UL channel - Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH Further discuss the two options and select one option in the next meeting: - The event triggered measurement reporting delay is TL1_measurement period + Tfirst UL channel - Option 1: if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. (Samsung, Xiaomi, Apple, MTK) - Option 2: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam.. (E///, Nokia, QC) #### Issue 1-4: Whether/how to specify the UE behaviour if new L1-RSRP during the time period between first PUCCH and second PUSCH? Agreement: - No further discussion on this issue in RAN4.

Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)?

The remaining open issue for event triggered measurement reporting delay is

- If timeRestrictionForChannelMeasurement is not configured,

  - Further discuss on:

    - Option1: Reuse the same values of legacy, measurement period is based on 3 sample. No further restriction of UE implantation of which one is used

    - Option 2: No requirements even the NW can configure it.

    - Option 3: FFS

The timeRestrictionForChannelMeasurement is defined by RAN1. We don’t think there has been any discussion on this in RAN1 yet. Hence, we prefer to wait for RAN1 discussion to understand if timeRestrictionForChannelMeasurement will be applicable for new beam configured for UEIBR and how it will be configured/applicable to the current beam. We recommend monitoring RAN1 progress and send LS to RAN1 in case there is no update in this meeting.

- There has been no RAN1 discussion on applicability of timeRestrictionForChannelMeasurement for UEIBR.

Wait for RAN1 discussion on applicability of timeRestrictionForChannelMeasurement for UEIBR.

RAN4 monitor progress in RAN1#121 and if necessary, send LS to RAN1 seeking clarification on applicability of timeRestrictionForChannelMeasurement for UEIBR in RAN4#115.

Sharing factor for current and new beam

  - For P (sharing factor) in L1_measurement periods:

    - For SSB based or CSI-RS based,

      - The definition is the same as legacy

      - The value can be the same or different for the current beam and new beam

For SSB from serving cell, SSB from cell with different PCI and CSI-RS the sharing factor ‘P’ is already defined. The definition of the sharing factor would be the same as agreed in [1]. If the current beam and new beam are in the same slot, then the sharing factor for current and new beam would be the same. But if the current and new beam are in different slots, then the sharing factors could be different depending on whether slot with RS for current or new beam overlaps with SMTC and/or MG.  This could happen for SSB based – if current and new beam are from different cells – current beam from serving cell and new beam from cell with different PCI, or vice versa. For CSI-RS based, the CSI-RS could be configured on different slots for current and new beam.

Based on definition of existing sharing factors, the sharing factor for current beam can be PC and the sharing factor for new beam can be PN.

Using the existing definition of sharing factor ‘P’, the sharing factors for new beam and current beam are PN and PC respectively.

Assuming same ‘M’ for current and new beam, the measurement period for current beam and new beam are:

TL1-RSRP_currBeam = M*N*PC*TRS

TL1-RSRP_newBeam = M*N*PN*TRS

Where TRS is the SSB or CSI-RS periodicity depending on whether it is SSB or CSI-RS based measurement. For FR1 and FR2 where RS for current beam and new beam are not overlapping or adjacent, the above measurement period can be used.

In FR2 when the RS for current beam and new beam are overlapping or adjacent, additional sharing factor is needed, and the measurement period is –

TL1-RSRP_currBeam = M*N*2*PC*TRS

TL1-RSRP_newBeam = M*N*2*PN*TRS

The measurement period for current beam and new beam each with periodicity TRS for FR1 and non-overlapping and non-adjacent in FR2 are defined as - 
TL1-RSRP_currBeam = M*N*PC*TRS 
TL1-RSRP_newBeam = M*N*PN*TRS

The measurement period for current beam and new beam each with periodicity TRS for overlapping or adjacent in FR2 are defined as -
TL1-RSRP_currBeam = M*N*2*PC*TRS 
TL1-RSRP_newBeam = M*N*2*PN*TRS

Issue 1-2: Components of Event triggered measurement reporting delay?

Agreement:

- The event triggered measurement reporting delay includes the Tfirst UL channel

  - Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH

Further discuss the two options and select one option in the next meeting:

- The event triggered measurement reporting delay is TL1_measurement period + Tfirst UL channel

  - Option 1: if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. (Samsung, Xiaomi, Apple, MTK)

  - Option 2: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam.. (E///, Nokia, QC)

In [1] the agreement for Tfirst UL channel is not definable, since the RS that triggered event is only known at the UE side. The

- The agreement from last meeting - Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH is not definable.

- The RS that triggered the event is only known at the UE.

Tfirst UL channel should be defined as time to first PUCCH after L1 measurement period for UEIBR. And this measurement allows for evaluation of both the current beam and the new beam. The minimum requirement should allow evaluation of both current and new beam, but The UE could trigger the event after measurement of either of them.

The figures below illustrate possible cases when the UE might trigger the event after either current beam or new beam measurement, but it cannot be guaranteed.

Figure 1: Example of UEIBR with different measurement period

Figure 2: Example of UEIBR with same measurement period

- The minimum requirements should allow measurement period for new beam and current beam.

- Tfirst UL channel should be defined as time to first PUCCH after L1 measurement period for UEIBR.

In case the TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. This covers the case when

The L1 measurement period for UEIBR is the maximum of L1 measurement periods to measure both current beam and new beams.

The  Tfirst UL channel is defined as time to first PUCCH after L1 measurement period for UEIBR.

Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19?

<Way forward >

- Option 1: (Qualcomm, Xiaomi, Apple, LGE)

  - Do not define requirements for eventInstanceCount-r19 > 1

- Option 1a: (Apple): To extend the requirements for case when eventInstanceCount-r19 > 1 is configured, add note longer delay is expected if eventInstanceCount-r19 > 1 is configured.

- Option 2: (Samsung, CMCC, Huawei, ZTE, MediaTek)

  - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

- Option 3: (Nokia, Ericsson)

  - Wait for further RAN1 conclusion

- For how to specify the requirements in option 2 above:

  - Option 2a:

    - Measurement reporting delay is TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where,

    - TL1-RSRP_event triggered_measurement_Period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

    - C: event instances threshold is configured by NW.

    - Ttime_window: time window is configured by NW.

    - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

  - Other options are not precluded

It is not feasible to define delay requirements based on evaluation window and can only be defined based on no time window configured or eventInstanceCount-r19 =1. Also, the purpose of defining requirements with eventInstanceCount-r19 >1 is un-clear. What does it help verify at the UE?

- It is not feasible to define core requirement for reporting delay with time window-based triggering

- The purpose of defining requirements based on evaluation window/ counter is unknown.

Measurement reporting delay requirement for UE initiated beam report is defined for eventInstanceCount-r19 =1.

To extend the requirements to window based evaluation, we can add a note that longer reporting delay is expected if eventInstanceCount-r19 > 1 is configured.

To extend the requirements for case when eventInstanceCount-r19 >1 is configured based on UE capability, add note longer delay is expected if eventInstanceCount-r19 >1 is configured.

Issue 1-5: CSI-RS based L1-RSRP for inter-cell BM

<Way forward >

- Option 1:

  - Not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management

- Option 2:

  - Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

The CSI_RS resources configured for new beam measurement cannot be identified as from serving cell or cell with different PCI. For event triggered reporting based on CSI-RS, the requirements are defined for CSI-RS without specifying inter-cell or intra-cell. We propose option 2 above.

- CSI-RS based new beam measurement cannot be distinguished between inter-cell and intra-cell.

For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirement.

Issue 1-6: Whether to specify new unified TCI switching delay requirements?

<Way forward >

- For MAC-CE based DL/UL TCI state switch delay

  - Option 1: (Qualcomm, Apple, Samsung (for known case), Ericsson)

    - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management

  - Option 2: (ZTE)

    - Remove components of TL1-RSRP and Tfirst-SSB+TSSB-proc

  - Option 3: (MediaTek)

    - FFS on whether to remove Tfirst-SSB+TSSB-proc

- For DCI based DL/UL TCI state switch delay

  - Option 1:

    - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management

  - Option 2:

    - Whether to further reduce the delay, wait for further RAN1 progress

Issue 1-6: Whether to specify new unified TCI switching delay requirements?

<Way forward >

- For MAC-CE based DL/UL TCI state switch delay

  - Option 1: (Qualcomm, Apple, Samsung (for known case), Ericsson)

    - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management

  - Option 2: (ZTE)

    - Remove components of TL1-RSRP and Tfirst-SSB+TSSB-proc

  - Option 3: (MediaTek)

    - FFS on whether to remove Tfirst-SSB+TSSB-proc

- For DCI based DL/UL TCI state switch delay

  - Option 1:

    - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management

  - Option 2:

    - Whether to further reduce the delay, wait for further RAN1 progress

For DCI based switching delay until now the RAN1 agreements don’t indicate update to the beam application time. Unless there is any update to beam application time from RAN1 for UEIBR, RAN4 shall assume that the existing requirements are applicable to UEIBR.

For DCI based DL/UL TCI state switch, RAN4 shall assume the current requirements are applicable unless there is any agreement in RAN1 on update to beam application delay.

For MAC-CE based TCI state switch, the components of delay are time for TO/FO tracking if target TCI is not in active TCI state list, time for L1-RSRP measurement if

After the UE sends event triggered beam report, the network could send a TCI state switch command to the new beam. The existing TCI state switching delay requirements for unified TCI state or legacy DL active TCI state would still be appliable. The TCI state switching delay components are

MAC CE decoding time

L1-RSRP measurement period for unknown TCI state

TO/FO tracking if TCI state is not in active list

UEIBR doesn’t impact any of the delay components. In case the target TCI is not in active TCI state list, additional time for TO/FO is needed, as UE triggered L1-RSRP report doesn’t guarantee that RS is tracked for PDSCH reception. In case the time between the UEIBR and TCI state switch is > 1280 ms, the TCI state would be unknown, and UE would need time for L1-RSRP measurement.

- UEIBR doesn’t guarantee that RS Is tracked for PDSCH reception.

- In case the time between the UEIBR and TCI state switch is > 1280 ms, the TCI state would be unknown.

Unless we define the requirements based on only known TCI state, and only if the target TCI state is in active TCI state list, we don’t see how the switching delay for MAC-CE based unified TCI state switch can be different from R17.

There is no need to specify new unified TCI state switching delay requirements unless the TCI state switch is only defined for known TCI state, and TCI state in active TCI state list.

Issue 1-7: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time?

<Way forward >

- Option 1: (Samsung)

  - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

- Option 2:

  - For such case, No RAN4 requirements.

- Option 3: (Apple)

  - Add the applicability rule of event triggered L1 measurement:

    - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE.

    - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

The assumption or applicability for event triggered L1 measurement needs to be defined. We propose the following conditions for the reporting delay requirements to be applicable:

- The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE.

- The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

Measurement reporting delay requirement applicability
- The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE 
- The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

Issue 1-9: Requirements for event-1 and event-7?

<Way forward >

- Option 1: When requirements for Event-2 are agreed, agree to reuse all requirements for Event-1 and Event-7.

- Option 2: others are not precluded

RAN4 to prioritize and complete requirements definition for Event 2 and re-use them for Event 1/7 where applicable.

RAN4 prioritize requirements for Event 2 and discuss Event 1/7 requirements later.

Issue 1-10: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)?

<Way forward >

Further discuss the two options. Proponents of option 2 to bring proposals on the exact numbers for the capabilities.

- Option 1:

  - For event trigger L1 reporting, do not define such capability in parallel per category like L3.

- Option 2:

  - Define capabilities for support of UE-initiated/event-driven beam reporting similar as L3.

There is no necessity to define capabilities for Support of Event Triggering and Reporting Criteria for UEIBR.

For event trigger L1 reporting, do not define such capability in parallel per category like L3.

Issue 1-11: Compatibility of capabilities with LTM

<Way forward >

- Option 1: (Apple, Ericsson)

  - Discuss it later.

- Option 2: (CMCC, Huawei, Nokia)

  - Not preferred to mix Rel-19 mobility and Rel-19 MIMO. The capability for supported number of events triggered L1 measurement reporting is defined separately for Rel-19 mobility and Rel-19 MIMO

- Option 3: (MediaTek)

  - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

In parallel WI in R19 for L1 based mobility, UE initiated beam reporting for neighbor cell is being discussed. It is very early in the WI stage to discuss UE capabilities. The RRM requirements for the respective WIs should proceed independently and while UE features are discussed, revisit discussion on compatibility of capabilities with LTM.

Discuss later in the WI on compatibility of capabilities with LTM.

# 3. Conclusion

In this paper, we provide our views on impact to RRM requirements from UE initiated beam management. Our observations and proposals are captured below:

Reporting Delay Requirements

- There has been no RAN1 discussion on applicability of timeRestrictionForChannelMeasurement for UEIBR.

- Wait for RAN1 discussion on applicability of timeRestrictionForChannelMeasurement for UEIBR.

RAN4 monitor progress in RAN1#121 and if necessary, send LS to RAN1 seeking clarification on applicability of timeRestrictionForChannelMeasurement for UEIBR in RAN4#115.

Using the existing definition of sharing factor ‘P’, the sharing factors for new beam and current beam are PN and PC respectively.

The measurement period for current beam and new beam each with periodicity TRS for FR1 and non-overlapping and non-adjacent in FR2 are defined as - 
TL1-RSRP_currBeam = M*N*PC*TRS 
TL1-RSRP_newBeam = M*N*PN*TRS

The measurement period for current beam and new beam each with periodicity TRS for overlapping or adjacent in FR2 are defined as -
TL1-RSRP_currBeam = M*N*2*PC*TRS 
TL1-RSRP_newBeam = M*N*2*PN*TRS

- The agreement from last meeting - Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH is not definable.

- The RS that triggered the event is only known at the UE.

- The minimum requirements should allow measurement period for new beam and current beam.

- Tfirst UL channel should be defined as time to first PUCCH after L1 measurement period for UEIBR.

The L1 measurement period for UEIBR is the maximum of L1 measurement periods to measure both current beam and new beams.

The  Tfirst UL channel is defined as time to first PUCCH after L1 measurement period for UEIBR.

- It is not feasible to define core requirement for reporting delay with time window-based triggering

- The purpose of defining requirements based on evaluation window/ counter is unknown.

Measurement reporting delay requirement for UE initiated beam report is defined for eventInstanceCount-r19 =1.

To extend the requirements for case when eventInstanceCount-r19 >1 is configured based on UE capability, add note longer delay is expected if eventInstanceCount-r19 >1 is configured.

CSI-RS Based measurements

- CSI-RS based new beam measurement cannot be distinguished between inter-cell and intra-cell.

For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirement.

TCI State Switching Requirements

For DCI based DL/UL TCI state switch, RAN4 shall assume the current requirements are applicable unless there is any agreement in RAN1 on update to beam application delay.

- UEIBR doesn’t guarantee that RS Is tracked for PDSCH reception.

- In case the time between the UEIBR and TCI state switch is > 1280 ms, the TCI state would be unknown.

There is no need to specify new unified TCI state switching delay requirements unless the TCI state switch is only defined for known TCI state, and TCI state in active TCI state list.

Applicability

Measurement reporting delay requirement applicability
- The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE 
- The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

RAN4 prioritize requirements for Event 2 and discuss Event 1/7 requirements later.

Capabilities

For event trigger L1 reporting, do not define such capability in parallel per category like L3.

Discuss later in the WI on compatibility of capabilities with LTM.

# Reference

- R4-2504902, “WF on RRM requirements for NR_MIMO_Ph5_Part1”, Samsung.