3GPP TSG-RAN WG4 Meeting #115						R4-2505912

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.24.6

Source:	Apple

Title:	On UE Demodulation and CSI Reporting Requirements for MIMO Phase 5

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

The performance part for Rel-19 MIMO Phase 5 WI starts in RAN4#115. In this contribution we present our views on the scope of UE demodulation and CSI reporting requirements for this WI.

# 2. Discussion

Based on the latest WID [1] the objectives of the work item are listed below:

- UE Initiated beam management

- CSI enhancements for up to 128 CSI-RS ports

- CJT Reporting enhancements

- Non-coherent UL codebook for 3-antenna port transmissions

- Asymmetric DL sTRP/UL mTRP

We discuss below the impact to UE demod CSI requirements based on each of the features introduced in this WI.

### UE Initiated Beam Management

For this feature RAN1 will specify the following enhancement:

UE initiated/event triggered beam reporting enhancement is to introduce event triggered L1-RSRP measurement reporting. L1-RSRP reporting is related to RRM and has no UE demodulation/ CSI reporting requirements impact.

- No UE demodulation/CSI requirement impact with UE initiated/ event triggered beam reporting.

### CSI Enhancements

For this feature RAN1 will specify the following enhancement:

For CSI enhancements, the following are introduced in RAN1:

- PMI reporting for 48, 64, 128 ports for the following codebooks

  - Rel-15 Type I-SP (Single panel)

  - Rel-15 Type I-MP (multi panel)

  - Rel-16 eType II

  - Rel-17 FeType II PS (port selection)

  - Rel-18 Type II-Doppler

  - Rel-18 Type II-CJT

- CRI reporting enhancement for up to 128 ports for Type I and eType II codebooks

- Fixed SRS port group mapping for SRS antenna switching when > 4 layers/ 2 2CW is supported

There is CSI reporting requirement impact with up to 128 ports codebook and CRI enhancement.

There is no demodulation/CSI requirement impact with SRS port grouping

- There is CSI reporting requirement impact with up to 128 ports codebook and  CRI reporting enhancement in MIMO Phase 5.

- There is no demodulation/CSI reporting requirement impact with SRS port grouping.

Until Rel-18 RAN4 has introduced PMI reporting requirements for the following codebooks and TX/CSI RS ports

  - Rel-15 Type I-SP (Single panel) – 4TX, 8TX, 16TX, 32 TX ports

  - Rel-15 Type II – 16 TX ports

  - Rel-16 eType II – 16 TX ports

  - Rel-18 Type II-Doppler – 16 TX ports

  - Rel-18 Type II-CJT – 8 TX ports

Until Rel-18 we have 2, 4, 8, 16, 24, 32 TX ports for all codebooks, but we don’t introduce PMI reporting requirements for all TX ports and all codebooks. In Rel-19 the increased TX ports support for 48, 64, 128 ports are introduced for 6 codebooks. We don’t think it is necessary to introduce PMI reporting requirements for all codebooks with > 32 TX ports.

- It is not necessary to introduce PMI reporting requirements for all codebooks with increased number of TX ports

It would be sufficient to introduce requirements with > 32 TX ports for 1 codebook, and for 1 TX antenna configuration.

- It is sufficient to introduce requirements with > 32 TX ports for 1 codebook and 1 TX antenna configuration.

We propose to consider defining requirement with Type I codebook with 64 ports in Rel-19.

Define requirements with Type I codebook with 64 TX ports in Rel-19.

For CRI reporting, RAN4 hasn’t introduced any requirements for up to 32 TX ports which is supported since Rel-15. With no baseline CRI reporting requirements, we don’t see the necessity to introduce CRI reporting requirements for > 32 ports. Hence we propose not to intrdcie any CRI reporting requirements for > 32 TX in Rel-19.

- RAN4 has not defined any CRI reporting requirements for up to 32 TX ports which is supported since Rel-15.

Do not introduce CRI reporting requirements with > 32 TX ports in Rel-19.

### CJT Reporting Enhancement

For this feature RAN1 will specify the following enhancement:

Requirements for CJT reporting enhancements is discussed in RRM with evaluation for number of samples for defining delay and accuracy requirements for delay and frequency offset. It would not be necessary to define any UE CSI reporting requirements for CJT reporting enhancements.

- There is extensive discussion in RRM for defining delay and accuracy requirements for CJT reporting.

- There is no necessity to define CSI reporting requirements for CJT reporting enhancement

Do not introduce any CSI reporting requirements for CJT reporting enhancement.

### Non-coherent UL codebook for 3-antenna port transmissions

For this feature RAN1 will specify the following enhancement:

For non-coherent UL codebook for 3-TX antenna, there is no impact to UE demodulation/CSI requirements.

- No UE demodulation/CSI requirement impact with non-coherent UL codebook for 3-TX antenna.

### Asymmetric DL sTRP/ UL mTRP

For this feature RAN1 will specify the following enhancement:

For asymmetric DL sTRP/ UL mTRP RAN1 has introduced 2 closed-loop power control adjustment with PL-offset configured for UL only TRP, and 2 TA support for this feature. Either of these don’t have impact to UE demodulation/CSI requirements.

- No UE demodulation/CSI requirement impact with asymmetric DL sTRP/ UL mTRP.

# 3. Conclusion

In this paper, we present our views on the scope of UE demodulation and CSI reporting requirements for MIMO Phase 5 WI. Our observations and proposals are captured below:

- No UE demodulation/CSI requirement impact with UE initiated/ event triggered beam reporting.

- There is CSI reporting requirement impact with up to 128 ports codebook and CRI reporting enhancement in MIMO Phase 5.

- There is no demodulation/CSI reporting requirement impact with SRS port grouping.

- It is not necessary to introduce PMI reporting requirements for all codebooks with increased number of TX ports

- It is sufficient to introduce requirements with > 32 TX ports for 1 codebook and 1 TX antenna configuration.

- Define requirements with Type I codebook with 64 TX ports in Rel-19.

- RAN4 has not defined any CRI reporting requirements for up to 32 TX ports which is supported since Rel-15.

Do not introduce CRI reporting requirements with > 32 TX ports in Rel-19.

- There is extensive discussion in RRM for defining delay and accuracy requirements for CJT reporting.

- There is no necessity to define CSI reporting requirements for CJT reporting enhancement

Do not introduce any CSI reporting requirements for CJT reporting enhancement.

- No UE demodulation/CSI requirement impact with non-coherent UL codebook for 3-TX antenna.

- No UE demodulation/CSI requirement impact with asymmetric DL sTRP/ UL mTRP.

# Reference

- RP-242394, “WID revision: NR MIMO Phase 5”, Samsung.