3GPP TSG-RAN WG4 Meeting #115						R4-2505913

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.31.7

Source:	Apple

Title:	On UE Demodulation Requirements for NTN Phase 3

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

The performance part for Rel-19 NTN Phase 3 WI starts in RAN4#115. In this contribution we present our views on the scope of UE demodulation requirements for this WI.

# 2. Discussion

Based on the latest WID [1] the objectives of the work item are listed below:

- Downlink Coverage Enhancements

- Uplink Capacity Enhancement

- Support of broadcast service

- Regenerative payload

- Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN

We discuss below the impact to UE demod requirements based on each of the features introduced in this WI.

For UL capacity enhancement, support for broadcast service and support of regenerative payload, there is no impact to UE demodulation requirements.

There is no UE demodulation requirement impact with the following features introduced in R19 NTN 
- Uplink Coverage Enhancement
- Support for broadcast service
- Support for regenerative payload

### Downlink Coverage Enhancements

The core part objectives for this feature from [1]:

For downlink coverage enhancements several enhancements are introduced of which the impact to UE demodulation requirements is potentially from the following link level enhancements –

- Link level enhancements are to be specified for the following channels:

  - PDCCH at least for CSS (except for Type-3) via PDCCH repetition

  - PDSCH with Msg4 via PDSCH repetition

  - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration

The link level enhancements for FR1-NTN introduce enhancements for PDCCH and PDSCH for which we define demodulation requirements.

There is potentially impact to UE demodulation requirements with link level enhancements for FR1-NTN.

The enhancements are for PDCCH are for CSS except for Type-3 CSS. We typically don’t define requirements for PDCCH on CSS. We propose to discuss if RAN4 should define requirements for PDCCH repetition for CSS.

PDCCH repetition enhancements for NTN are for CSS which we typically don’t define requirements for.

- RAN4 to discuss if we should introduce requirements for PDCCH repetition for NTN.

The enhancements to PDSCH are for SIB1 and Msg4, which we don’t define requirements for in RAN4. We propose to discuss if RAN4 should define requirements for PDSCH repetition for SIB1 and Msg4.

PDSCH repetition enhancements for NTN are for SIB1 and Msg4 which we don’t define requirements for.

- RAN4 to discuss if we should introduce requirements for PDSCH -SIB1 / Msg4 repetition for NTN.

### Support for RedCap/ eRedCap

The enhancement to support RedCap/eRedCap include:

The enhancements introduced in Rel-19 for HD-FDD RedCap/ eRedCap UEs for NTN do not have UE demodulation requirements impact. But the features for RedCap/eRedCap such as reduced number of RX, reduced bandwidth, reduced data rate for NTN needs to be covered by demodulation requirements.

For Rel-17 RedCap RAN4 introduced requirements for the following:

- Requirements with 1RX (PDCCH, PDSCH, PBCH)

- Requirements with reduced BW (20MHz BW in TDD) with 1RX/ 2RX

For Rel-18 eRedCap RAN4 introduced requirements for the following:

- Requirements with reduced baseband bandwidth for 1RX/ 2RX

- Requirements without reduced baseband bandwidth for 1RX/2RX

For FR1 NTN in Rel-17 RAN4 specified the following PDSCH requirements in FDD:

- Requirements with larger HARQ processes (16, 32)

- Requirements with HARQ disabled

In Rel-19 for RedCap/ eRedCap UEs support in NTN, we would need to introduce requirements for NTN features of HARQ enhancement with RedCap/eRedCap features of reduced number of RX, reduced bandwidth, reduced data rate.

RAN4 would need to discuss introducing requirements for NTN features of HARQ enhancement with RedCap/eRedCap features of reduced number of RX, reduce bandwidth, reduced data rate.

In Rel-17 NTN, we introduced a few test cases with NTN channel models and different HARQ enhancement features. Since a FR1 NTN UE also supports TN access, the requirements for the features it supports would be tested. For the new requirements in Rel-19, we propose to only introduce PDSCH demodulation requirements. Do not introduce requirements for PDCCH, PBCH and CSI reporting.

- Do not introduce PDCCH, PBCH and CSI reporting requirements for Rel-19 NTN for RedCap/eRedCap UE.

There is no necessity to introduce all the PDSCH requirements for RedCap/eRedCap from Rel-17 and Rel-18 with all the HARQ enhancement features. We propose to choose a combination of NTN features and RedCap/eRedCap features for requirements introduced in Rel-19.

- Choose a combination NTN features and RedCap/eRedCap features for PDSCH demodulation requirements introduced in Rel-19.

The PDSCH demodulation requirements from R17 NTN are for 10MHz with 2 RX. The reduced bandwidth (< 20MHz) for RedCap is already covered in existing FR1 NTN requirements. We propose to introduce requirements with 1 RX in Rel-19 NTN.

The reduced bandwidth for RedCap UEs with NTN is already covered in existing R17 NTN PDSCH demodulation requirements with 2RX.

- Introduce requirements with 1RX for NTN in Rel-19.

For eRedCap features of reduced bandwidth and without reduced bandwidth with reduced data rate, we propose to introduce test case with 1RX and 1 with 2RX.

- Introduce requirements with 1RX and 2RX for eRedCap UEs for reduced bandwidth and without reduced bandwidth for NTN.

For PDSCH requirements use the following configuration, same as that used in R17 NTN requirements:

- MCS & Rank: MCS 4, MCS13; Rank 1

- Antenna config: 1T1R, 1T2R

- Channel Model: NTN-TDLA100-200, NTN-TDLC5-200

- For PDSCH requirements use the following config-
- MCS 4, MCS13; Rank 1
- Antenna config: 1T1R, 1T2R
- Channel Model: NTN-TDLA100-200, NTN-TDLC5-200.

Proposed test cases:

RedCap:

| BW/SCS | Ant Config | Prop. Channel | MCS | HARQ Config | Notes |
| --- | --- | --- | --- | --- | --- |
| 10MHz/ 15 KHz | 1x1 | NTN-TDLA100-200 | 4 | Disabled HARQ | Also applicable to eRedCap without reduced BW |
| 10MHz/ 15 KHz | 1x1 | NTN-TDLC5-200 | 13 | 16 HARQ Proc |  |
| 10MHz/ 15 KHz | 1x1 | NTN-TDLC5-200 | 4 | 32 HARQ Proc | Also applicable to eRedCap without reduced BW |

eRedCap:

| BW/SCS | Ant Config | Prop. Channel | MCS | HARQ Config | PRB size | TBS | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLA100-200 | 4 | Disabled HARQ | 25 | 1928 | With reduced BW |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLC5-200 | 13 | 16 HARQ Proc | 25 | 6272 | With reduced BW |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLC5-200 | 4 | 32 HARQ Proc | 25 | 1928 | With reduced BW |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLA100-200 | 13 | 16 HARQ Proc | 40 | 9992 | Without reduced BW |

- Consider the following test cases/ parameters for RedCap with NTN

| BW/SCS | Ant Config | Prop. Channel | MCS | HARQ Config | Notes |
| --- | --- | --- | --- | --- | --- |
| 10MHz/ 15 KHz | 1x1 | NTN-TDLA100-200 | 4 | Disabled HARQ | Also applicable to eRedCap without reduced BW |
| 10MHz/ 15 KHz | 1x1 | NTN-TDLC5-200 | 13 | 16 HARQ Proc |  |
| 10MHz/ 15 KHz | 1x1 | NTN-TDLC5-200 | 4 | 32 HARQ Proc | Also applicable to eRedCap without reduced BW |

- Consider the following test cases/ parameters for eRedCap with NTN

| BW/SCS | Ant Config | Prop. Channel | MCS | HARQ Config | PRB size | TBS | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLA100-200 | 4 | Disabled HARQ | 25 | 1928 | With reduced BW |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLC5-200 | 13 | 16 HARQ Proc | 25 | 6272 | With reduced BW |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLC5-200 | 4 | 32 HARQ Proc | 25 | 1928 | With reduced BW |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLA100-200 | 13 | 16 HARQ Proc | 40 | 9992 | Without reduced BW |

# 3. Conclusion

In this paper, we present our views on the scope of UE demodulation requirements for NTN Phase 3 WI. Our observations and proposals are captured below:

General

- There is no UE demodulation requirement impact with the following features introduced in R19 NTN 
- Uplink Coverage Enhancement
- Support for broadcast service
- Support for regenerative payload

Downlink Coverage Enhancements

There is potentially impact to UE demodulation requirements with link level enhancements for FR1-NTN.

PDCCH repetition enhancements for NTN are for CSS which we typically don’t define requirements for.

- RAN4 to discuss if we should introduce requirements for PDCCH repetition for NTN.

PDSCH repetition enhancements for NTN are for SIB1 and Msg4 which we don’t define requirements for.

- RAN4 to discuss if we should introduce requirements for PDSCH -SIB1 / Msg4 repetition for NTN.

Support for RedCap/eRedCap UEs

RAN4 would need to discuss introducing requirements for NTN features of HARQ enhancement with RedCap/eRedCap features of reduced number of RX, reduce bandwidth, reduced data rate.

- Do not introduce PDCCH, PBCH and CSI reporting requirements for Rel-19 NTN for RedCap/eRedCap UE.

- Choose a combination NTN features and RedCap/eRedCap features for PDSCH demodulation requirements introduced in Rel-19.

The reduced bandwidth for RedCap UEs with NTN is already covered in existing R17 NTN PDSCH demodulation requirements with 2RX.

- Introduce requirements with 1RX for NTN in Rel-19.

- Introduce requirements with 1RX and 2RX for eRedCap UEs for reduced bandwidth and without reduced bandwidth for NTN.

- For PDSCH requirements use the following config-
- MCS 4, MCS13; Rank 1
- Antenna config: 1T1R, 1T2R
- Channel Model: NTN-TDLA100-200, NTN-TDLC5-200.

- Consider the following test cases/ parameters for RedCap with NTN

| BW/SCS | Ant Config | Prop. Channel | MCS | HARQ Config | Notes |
| --- | --- | --- | --- | --- | --- |
| 10MHz/ 15 KHz | 1x1 | NTN-TDLA100-200 | 4 | Disabled HARQ | Also applicable to eRedCap without reduced BW |
| 10MHz/ 15 KHz | 1x1 | NTN-TDLC5-200 | 13 | 16 HARQ Proc |  |
| 10MHz/ 15 KHz | 1x1 | NTN-TDLC5-200 | 4 | 32 HARQ Proc | Also applicable to eRedCap without reduced BW |

- Consider the following test cases/ parameters for eRedCap with NTN

| BW/SCS | Ant Config | Prop. Channel | MCS | HARQ Config | PRB size | TBS | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLA100-200 | 4 | Disabled HARQ | 25 | 1928 | With reduced BW |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLC5-200 | 13 | 16 HARQ Proc | 25 | 6272 | With reduced BW |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLC5-200 | 4 | 32 HARQ Proc | 25 | 1928 | With reduced BW |
| 10MHz/ 15 KHz | 1x1, 1x2 | NTN-TDLA100-200 | 13 | 16 HARQ Proc | 40 | 9992 | Without reduced BW |

# Reference

- RP-243300, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, Thales, CATT.