# 3GPP TSG-RAN WG4 Meeting #115	R4-2505924

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	5.17.2

Source:	Apple

Title:	On R18 NTN RRM core and performance requirement maintenance

Document for:	Discussion

# Introduction

In last RAN4#114 meeting, the WF for eNTN has been agreed in [1] and moderator has captured all open issues in the topic summary [2]. There are still some opens issues for further discussion, and in this contribution, we continue the discussion on remaining RRM performance requirements for eNTN.

In addition, there is one in-coming LS from RAN2[3], and we also discuss the potential reply LS to RAN2 in this paper.

# Discussion on Core part maintenance

### In RAN4 #110 meeting WF R4-2403492, RAN4 agreed that,

### Issue 5-2-S2: (‘Soft’ Satellite switch) Interruption time Agreement: (Needs online discussion) - The starting point of the downlink synchronization time/interruption time. - Between t-serviceStart and t-Service, the exact starting time is up to UE implementation - The starting point of the interruption time. - t-Service

### And RAN2 also has similar definition in their spec TS38.331,

### However, in the current RAN4 requirement for soft satellite switching without PCI change, it was specified that,

### The above equation means RAN4 assumed the DL synchronization starts from t-seviceStart, and it’s conflicting with the previous RAN4 agreements and RAN2 spec.

Figure 1. the existing equation is unclear on when UE to start DL sync

In the equation, Tsearch is the time required to search the target NR SAN cell assuming the target cell is not already known when the handover command is received by the UE. If the target cell Es/Iot ≥ -2 dB, then Tsearch = Tfirst_SSB ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. Tfirst_SSB is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from SSB-TimeOffset reference point as defined in 38.331 [2] to UE.

In our view, the existing requirement applicability shall be clarified to avoid the ambiguity for UE implementation, i.e., whether or not when to perform DL synchronization of new SAT is up to implementation. There might be two solutions without changing the current equation,

Alt 1: clarify the current requirement equation for soft satellite switching is applied for the case when UE starts DL synchronization from t-serviceStart.

Alt 2: clarify the Tfirst_SSB is the time to the end of the target satellite’s first complete SSB burst where UE starts performing DL synchronization.

Proposal 1: to reflect “when to start DL synchronization between t-serviceStart and t-Service is up to UE implementation”, extra clarification is needed for existing soft satellite switching latency requirement. Either of the following alternatives is fine to us:

- Alt 1: clarify the current requirement equation for soft satellite switching is applied for the case if UE starts DL synchronization from t-serviceStart.

- Alt 2: clarify the Tfirst_SSB is the time to the end of the target satellite’s first complete SSB burst where UE starts performing DL synchronization.

# Discussion on LS[3] from RAN2

RAN4 received the reply LS[3] from RAN2, and the text is duplicated below,

RAN4 Question 1: RAN4 would like RAN2 to confirm which one of the following is the correct understanding of parameter ssb-TimeOffset, - Option 1: the relative offset between the SSB of the source satellite and the SSB of the target satellite, and the starting boundaries of SFN0 of the source satellite and the target satellites are aligned at the uplink time synchronization reference point - Option 2: the relative offset between the starting boundaries of SFN0 of the source satellite and the target satellite RAN2 answer: Option 1 is aligned with RAN2 understanding. RAN4 Question 2: Regarding the assumptions above, if option 1 is the correct understanding, RAN4 kindly ask RAN1/2: 1) Whether it is feasible to configure ssb-TimeOffset with values other than multiple of 5. 2) How the half-frame indication and SFN indication bits are configured during [t-ServiceStart, t-Service]. RAN2 answer: The starting boundaries of SFN0 of the source satellite and the target satellite are aligned at the uplink time synchronization reference point and ssb-TimeOffset can only be configured as 5ms or multiples of 5ms. RAN2 understanding is that SSB from the target satellite will indicate SFN and half frame bit corresponding to the actual time domain position as in legacy. Furthermore, RAN2 has agreed for Rel-18 RAN2 will not specify a specific solution but leave to NW implementation to reconfigure SMTCs as needed.

As shown in the following figure, the ssb-TimeOffset is to avoid the colliding between source satellite SSB and target satellite SSB during [t-ServiceStart, t-Service], and then after t-Service, the new serving satellite SSB offset is restored to the same as the previous source satellite. That means, the SSB offset restoring will only offset back the SSB absolute timing by a fixed time value (e.g., the ssb-TimeOffset of target satellite during [t-ServiceStart, t-Service]).

Figure 2. illustration of soft satellite switching with SSB offset reset

According to the RAN2 reply LS, we don’t see any change is needed for the principle of delay requirement design for the existing requirement. However, in RAN4 requirement for soft satellite switching as following, the Tsearch is the time required to search the target NR SAN cell assuming the target cell is not already known when the handover command is received by the UE. Cell search can determine the coarse timing information for symbol/slot boundary for the target satellite and it shall be maintained even after t-Service. The only difference is UE can determine the SSB location on time domain by using this coarse timing and restored SSB offset (it implies 0 t-Service after in our view).

Additionally, we used maximum function for the requirement to consider whether UE can complete the timing acquisition before t-Service. Thus, the only thing we need to clarify in RAN4 is: if UE does not complete the cell searching when t-service is expired, we need to clarify in the current definition of Tfirst_SSB.

The current definition of Tfirst_SSB is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from [SSB-TimeOffset reference point as defined in 38.331 [2]] to UE.

If the ssb-TimeOffset is restored after the t-Service and UE does not complete the cell searching when t-service is expired, that actually means ssb-TimeOffset shall not be applied to the SSB timing for target satellite, and therefore it could be:

is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from [SSB-TimeOffset reference point as defined in 38.331 [2]] to UE; or

Proposal 2: if UE does not complete the cell searching when t-service is expired, RAN4 to discuss whether the clarification is needed for Tfirst_SSB, e.g., Tfirst_SSB can be revised to:

is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from [SSB-TimeOffset reference point as defined in 38.331 [2]] to UE.

# Conclusion

In this contribution, we discuss the NR-NTN core requirement maintenance.

Proposal 1: to reflect “when to start DL synchronization between t-serviceStart and t-Service is up to UE implementation”, extra clarification is needed for existing soft satellite switching latency requirement. Either of the following alternatives is fine to us:

- Alt 1: clarify the current requirement equation for soft satellite switching is applied for the case if UE starts DL synchronization from t-serviceStart.

- Alt 2: clarify the Tfirst_SSB is the time to the end of the target satellite’s first complete SSB burst where UE starts performing DL synchronization.

Proposal 2: if UE does not complete the cell searching when t-service is expired, RAN4 to discuss whether the clarification is needed for Tfirst_SSB, e.g., Tfirst_SSB can be revised to:

Tfirst_SSB is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from [SSB-TimeOffset reference point as defined in 38.331 [2]] to UE.

# References

[1] R4-2500519, Topic summary for [114][205] NR_NTN_enh, Qualcomm Incorporated, RAN4 #114

[2] R4-2502687, WF on RRM requirements for NR_NTN_enh_enh, Qualcomm Incorporated, RAN4#114

[3] R2-2503069, Reply LS on soft satellite switch with re-sync, Huawei, RAN2#129bis