3GPP TSG-RAN WG4 Meeting #115                                                        R4-2505942

# Malta, MT, 19th – 23rd May, 2025

Source:	Apple

Title:	Discussion on Beam Management: RRM core requirement and testing framework for beam management

Agenda Item:	7.21.4

Release:            Rel-19

Document for:	Discussion

Introduction

Rel-18 Study Item was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface with the target to study the 3GPP framework for AI/ML for air-interface corresponding to each targeted use cases (i.e., CSI feedback enhancement, beam management, and positioning accuracy enhancements for different scenarios) regarding aspects such as performance, complexity, and potential specification impact [1] [2].

According to latest WID in [2], RAN4 is required to study the interoperability and testability aspects for each use case: Specifically, RAN4 is expected to study the requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable.

The current agreements on how to perform the RAN4 study on general issues for AI/ML, and issues related to interoperability/testing have been captured in the latest TR [3]

In this contribution, we provide our viewpoints on some of the interoperability and testability aspects for beam management.

RRM core requirement for model Inference

## 2.1 Prediction delay requirement including measurement period

Measurement Period for prediction

The legacy L1-RSRP measurement period and accuracy requirement is based on 1 shot measurement. The measurement period has resource sharing factor ‘P’, number of sample ‘M’ configured by measurement configuration, RX beam sweeping factor ‘N’ in FR2. The measurement period for Set-B would highly depend on the accuracy needed for prediction. We propose to use the legacy measurement period as a starting point and further evaluate if it is sufficient to achieve good prediction accuracy.

Observation 1: The measurement period for Set-B would depend on the accuracy needed for prediction.

Proposal 1: Use legacy measurement period as a starting point for Set-B for evaluation if sufficient to achieve good prediction accuracy.

During RAN4#114bis the following agreements were reached:

Issue 2-4: Measurement period for inference

Agreement:

- For measurement delay component of the prediction delay:

  - Use “legacy” L1-RSRP measurement as the baseline for Case 1

    - FFS whether same values for M,N,P should be reused

  - FFS for case 2

For AI-BM Case 2, the UE may perform multiple L1-RSRP measurement occasions on Set-B before making AI-based predictions. The number of such measurement occasions should be configurable and is subject to further study in RAN1.

Proposal 2: For AI-BM Case 2, the number of L1-RSRP measurement occasions on Set-B should be configurable and studied further in RAN1.

## 2.2  Known Condition for TCI state

The known condition for TCI state is defined based on the period between TCI state switch and the last transmission of RS or RS QCLed with target TCI state. If the target TCI state is a predicted beam, and the RS was not transmitted to, measured and reported by UE, then we need to introduce a new definition for known condition. We also need to differentiate between UE side prediction and NW side prediction. For the case when target TCI state is a RS based on beam prediction -

UE side model: The UE predicts and reports the top K beams based on measurements on set B. Since the UE is predicting the beam(s) reported, it is known to the UE if a TCI state is received within a certain period. The known condition can be determined based on last transmission of set-B.

NW side model: The UE reports the measurements for Set B and target TCI state is based on NW side prediction, the TCI state is unknown to the UE. In case the RS of target TCI state is transmitted to the UE, measured and reported prior to the TCI state switch, then it is known to the UE.

Figure 3: Known condition for UE side prediction

Observation 2: The legacy definition for known TCI state cannot be used when target TCI state is based on predicted beam and not transmitted to and measured & reported by UE.

Observation 3: RAN4 needs to discuss known TCI for UE side and NW side prediction.

Proposal 3: For UE side model known TCI state is determined based on Set B transmission and UE report of top-K.

Proposal 4: For NW side model - 
(1) TCI state is unknown if RS of target beam is not transmitted to UE prior to TCI state switch
(2) If RS of target beam is transmitted to the UE and reported, legacy definition of known TCI state is applicable

For UE side Rx beam assumption, there are also different assumptions:

Alt 1: UE always uses the best Rx beam to measure set B beams and set A beams and the corresponding L1-RSRP

This imply beam sweeping in the data collaboration procedure at the UE side for set A and set B measurement.

In training data collection for NW side model based on MDT framework, the UE report the gNB beam with corresponding L1-RSRP with best UE Rx beam.

During inference, UE report the L1-RSRP and corresponding NW beam index(es) (CRI/SSBRI) assuming the best UE Rx beam.

How best UE Rx beam is determined requires advanced UE side Rx beam management implementation. For example,

UE record the best Rx beam in global coordinates during training,

During inference, based on sensor information and UE orientation, to figure out the best Rx beam to use for the set A.

During performance monitoring, best Rx beam for set A and set B respectively is used.

Alt 2: UE always use the best Rx beam for set B measurement, and UE used the best Rx beam of set B for set A measurement during training and inferencing.

This imply that UE beam sweeping for set B measurement during training and inference.

In addition, using data collection for training procedure, the RS configuration in gNB needs to indicate the root RS of set A configuration with respect to set B RS.

There can be proprietary information disclosures concern at the NW vendor. There is no agreement in RAN1 (no related proposals either in this case).

In training data collection for NW side model based on MDT framework, the UE report the gNB beam with corresponding L1-RSRP with best UE Rx beam for set B, and set A L1-RSRP with best UE Rx beam QCLed to set B beam.

During inference, UE report the set B L1-RSRP and corresponding NW beam index (CRI/SSBRI, can be more than 4) assuming the best UE Rx beam for set B.

The same QCL type D root RS to set B should be signaled by the NW, as the training phase.

Proposal 5: NW side data collection, QCL type D is signaled between set A and set B RS.

Proposal 6: For NW side model, TCI state is known if QCL source is signaled in both data collection and inference (to ensure UE Rx consistency), it is known. Otherwise, it is unknown.

Proposal 7: (for Alt 1):  Define UE capability for Alt 1 of UE implementation. When UE does not support this capability, the TCI state is unknown. Unknown TCI state switching delay is applied.

Proposal 8: (for Alt 2): When gNB signal the QCL relationship between set A and set B, in training RS configuration, inference RS configuration and performance monitoring RS configuration (example of Alt 2 NW beam implementation of set A and set B),

The TCI state is known when the following is met:

Target TCI state is based on RS from set A beams with QCL relationship configured to a set B beam

The time between TCI state switch command and transmission of the QCLed RS from set B shall not exceed X ms

X = 1280 ms (legacy)

UE shall send measurement report for the QCLed RS from Set B between transmission and TCI state switch

TCI state and SSB of TCI state remain detectable during TCI state switching period

Otherwise, it is unknown.

In the past few meetings, RAN4 discussed whether Rx beam information is known for predicted beams.

Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

Based on our discussion above we would like to make the following proposal:

Proposal 9: The ability to identify the Rx beam associated with an unmeasured predicted Tx beam may rely on the UE’s supported features

## 2.3 RRM core requirement for performance monitoring

With respect to performance monitoring of UE-side models, key topics discussed in RAN1 include: selection of performance metrics, identification of benchmarks or references for comparison, and the type of monitoring approach. The following analysis outlines potential impacts on specifications related to performance metrics and benchmarking, viewed from a physical layer perspective:

For the performance monitoring of BM-Case1 and BM-Case2: - Performance metric(s) with the following alternatives: - Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy - Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER - Alt.3: Performance metric based on input/output data distribution of AI/ML - Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP - Benchmark/reference for the performance comparison, including: - Alt.1: The best beam(s) obtained by measuring beams of a set indicated by gNB (e.g., Beams from Set A) - Alt.4: Measurements of the predicted best beam(s) corresponding to model output (e.g., Comparison between actual L1-RSRP and predicted RSRP of predicted Top-1/K Beams) - Signalling/configuration/measurement/report for model monitoring, e.g., signalling aspects related to assistance information (if supported), Reference signals

For the configuration for monitoring, based on the two options given in RAN1#118bis, Option 2 (dedicated resource set(s)) are agreed to be supported:

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

Proposal 10: RAN4-defined prediction accuracy requirements is proposed in the context of performance monitoring Type 1, and for Option 2 (UE-assisted performance monitoring)

## 2.3 RRM core requirement for LCM

Based on the preceding analysis of various performance monitoring types for UE-side models (whether monitoring is conducted by the network or the UE), control over the model or its associated functionality may be exercised by either the UE or the gNB:

- Management: - For UE-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the UE when the monitoring resides within the UE. - For UE-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the gNB when the monitoring resides within the gNB or UE.

In RAN2#126, the following agreements are achieved on LCM for UE-sided model:

Agreement:

- Whether transition from AI to non-AI would be UE initiated or network initiated

  - This will be a decision for other groups

  - RAN4 to assess what RRM requirements are needed for each case.

Agreement:

- The discussion should consider use-cases specific aspects

Agreements from RAN4#113 and agreements from RAN2#127 meetings:

## The management of AI/ML beam management (BM) operations, such as activation, deactivation, and fallback (e.g., transitioning from AI/ML-based beam prediction to traditional modes), can be facilitated through CSI report configurations. These operations can be initiated, for instance, based on the output from performance monitoring. Utilizing the current capability framework, the network has the ability to configure CSI report settings to support beam prediction for both BM-Case1 and BM-Case2. The CSI reports for these cases can be configured as periodic, semi-persistent, or aperiodic, depending on the operational requirements.

Proposal 11: For UE-side AI/ML beam management, LCM operations such as activation, switching, and fallback can be supported via CSI report configurations. RAN4 should evaluate the latency impact of these operations for periodic, semi-persistent, and aperiodic CSI reporting modes. The analysis should cover BM-Case1 and BM-Case2, considering processing time, measurement intervals, and model availability in the UE.

## 2.4 Beam Prediction Testability Discussion

During RAN4#114 the following agreement has been reached:

Issue 4-1: Baseline test setup

Agreement:

- Discuss both single AoA based and multiple AoA based set up

  - Continue to study the two multiple AoA candidate setups

    - One option to continue the discussion proposed as following:

      - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability

        - AI/ML BM performance centric, not CDL channel emulation centric.

        - Define test system candidates which are already feasible

      - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance.

  - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs

Issue 4-2: Channel model

Agreement:

- Use CDL-based channel model as starting point

According to the discussion during RAN4#111 [ R4-2410571] the following agreements have been reached:

Agreement:

Maximum number of set B Tx beams that test system should be able to emulate: [8-16]

Maximum number of set A Tx beams that test system should be able to emulate:  [64-128]

FFS on AoAs

UE rotation during the test: FFS

UE rotation/repositioning between different tests: Yes

Issue 2-3: Channel models

Agreement: Further discuss and down-select the following options considering TE feasibility

- Option 2:  TDL

- Option 5: simplified CDL

  - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed

Issue 2-4: Test setup needs

Issues listed below to be further investigated in order to determine the capabilities of the test equipment:

- set B Tx beams; set A Tx beams

      - number of beams transmitted simultaneously

      - overall number of beams transmitted during the test

- AoA

- AoD

- UE rotation during the test

Other aspects can also be considered.

During RAN4#113 the following agreement was reached:

Agreement: Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs.

We will discuss two options for the Beam Prediction Testability:

(1) Multiple AoA based Testing Setup

To verify the accuracy of the L1-RSRP prediction for BM case 1 and case 2, we will need to introduce enough randomness in the spatial and temporal domain.  As it is shown in Fig 2, the propagation environment characterized by the different AoAs and AoDs, effectively sample the radiation beam patterns across the mainlobe and sidelobes of each of the beams in set B/set A at Tx and Rx antenna arrays. Fig 2 shows the radiation pattern of one beam inside set B or set A. The spatial mapping from set B to set A is learnt through all possible samplings of set B Tx beam patterns according to the AoDs along with samplings of the Rx beams across AoAs. AI/ML can learn the mapping from set B samples to set A samples by exploiting the correlations of the radiation patterns. For this learning to be complete we should sample the radation patterns at many points in spatial domain. This could be achieved by introducing randomness in AoA/AoD domain since the channel response is determined by the Tx beam response at the direction of the AoD.

In the presence of multiple AoAs and AoDs, the resulting L1-RSRP is the superposition of these spatial beam samples at both Rx and Tx. The spatial mapping of set B L1-RSRP to set A L1-RSRP is learnt by proving enough random data  to capture the spatial correlations across different angles of departures and arrivals as well as the different superpositions of those angles. The correlations inferred will depend on the specific mainlobe as well as the sidelobe shapes of the setA/B patterns which will have to configured in the test and emulated by the test equipment. Therefore, the AI/ML model should be trained and tested with enough randomness.

Observation 4: The testing environment for Beam Management (BM) case 1 and BM case 2 should replicate conditions that sufficiently capture the correlations of the transmit (Tx) and receive (Rx) beam patterns across the entire spectrum of propagation conditions in both spatial (angles of arrival and departure - AoAs and AoDs) and temporal domains.

Observation 5:  The following conditions will introduce randomness and variations in propagation conditions across both time and spatial domains for the computation of L1-RSRP

- Different AoDs with respect to the Tx antenna array

- Different AoAs with respect to the Rx antenna array

- Different superpositions of {AoA,AoD} pairs

- Fading/Variation in time domain ( different {AoA,AoD} pairs per resolvable delay bin path)

- UE movement (including rotation)

It is expected that during deployment in the field, all the aforementioned randomness and variations will be inherent in the real propagation environment. Therefore, the AI/ML model should be tested under similar conditions, otherwise, the UE could easily pass the test and fail during real deployment scenarios.

Observation 6:  During real-world deployment, UE will encounter random radio propagations characterized by variations in both spatial and temporal domains (fading). Testing UE under similar conditions is important to reflect the realities of deployment accurately.

Fig. 2: Beam pattern spatial domain sampling across AoA/AoD

The baseline measurement setup for RRM characteristics and SNR referenece point for RRM testing procedure based  on TR 38.810 [4] for the study on test methods specifically for FR2 UE beam management is shown in Fig 3.

Fig. 3:  UE RRM test methodology for FR2 Beam Management

For the testing goals of BM AI/ML use case as described in the paragraph above, there are two main limitations with the existing test methods:

- Limitation on the number of simultaneously active AoAs (=2)

- Limitation of the relative angular separation between AoA pairs: 30°, 60°, 90°, 120° and 150°.

There are questions regarding how to meet the testing goals and RAN4 requirements using the FR2 OTA chamber, particularly with the necessity of sweeping beam patterns across Set-A and Set-B, but with the limitations of only two Angle of Arrivals (AoAs) and coarse angular separation. Moreover, for BM-Case2, to evaluate temporal DL Tx beam prediction, there should be some variation in beam direction over time, including UE rotation.

Observation 7: The following questions need to be answered to evaluate the feasibility of the FR2 OTA-based test procedure:

- How can we generate multiple beams from the Set-B and Set-A Tx beams given the limitation of two AoAs?

- What assumption is made regarding Rx beam sweeping? Does the UE utilize a fixed Rx beam, or does it sweep to find the optimal Rx beam?

- How can we simultaneously emulate different AoAs (Rx beam) and AoD (Tx beams)?

- How can we achieve dynamic variation in the AoD domain (Tx beam sweeping) for BM case 2 prediction?

In [5] it was concluded that we would neeed for the current 3D MPAC to approx. >100 probes i.e. also  >200 RF channels (to count for dual polarized probes) for emulating CDL-C for AI/ML based BM testing. This would be simply unfeasible. Thus, further simplifications of the test scenarios are necessary. For feasible implementation of an OTA test setup for AI/ML based beam management, further simplifications are inevitable.

For beam management, the primary goal is not to develop an exact or near-perfect representation of the resulting CDL channel model in static scenarios (such as FR2 MIMO OTA analyzed in earlier sections). Instead, the focus is on emulating a time-varying power environment with limited resolution, thereby avoiding the need for physical simulation of angular spread at both the cluster and overall levels

Proposal 12: Multi AoA based Testing Setup for BM: Evaluate the testability requirements for simulating time-varying input power to a sparse probe layout based on a simplified channel CDL model.  The key aspects to be considered are:

- How to determine the minimum number of clusters to be emulated without introducing bias in the results. Investigate a quantitative “goodness” criterion to selecting the number of probes/clusters.

- Consider incorporating UE rotation for the test

- How many different CDL channels we need to support (considering generalization purposes as well)

For RAN 4 testing we should have the following goals:

(1) Provide confidence that if a DUT passes the test it will also perform well in the field

(2) Reliability of using synthetic channels for test data in evaluating models trained on real data.

Observation 8: For RAN 4 testing we should have the following goals:

(1) Provide confidence that if a DUT passes the test it will also perform well in the field (2) Ensure reliability of using synthetic channels for test data in evaluating models trained on real data.

Based on the above goals we have the following:

Proposal 13: To establish a criterion for assessing the feasibility of a simplified sparse probe layout (Option 5) for beam management (BM) testing, the following procedure can be considered:

- Train a model using the reference CDL channel and test it with the simplified sparse layout-based CDL channel.

- Train a model using the simplified CDL channel and test it with the reference CDL channel model.

- Train and test a model using the reference CDL channel.

A similarity metric should be defined to compare the RSRP accuracies across these test cases. This metric would indicate the effectiveness of testing with the simplified CDL model. The similarity metric can be derived through simulations.

Fig. 4:  Channel Emulator Based Spatial channel for testing BM

Below we provide some details on how to emulate a CDL channel with a time varying power across the AoA probes.

During RAN4#110-bis a spatial model was proposed to be implemented by the channel emulator as shown in Fig 5.

Fig. 5   Multipath based Testing Setup with a simplified CDL channel and probe layout

The following parameters that can be programmed:

Tx codebook and antenna pattern. The Tx gain is known given the codebook and antenna pattern (across 2 polarizations) The AoD angles from the Tx side. We assume that the AoA angles are fixed and they are part of the probes in the MIMO OTA testing setup. The coupling factors that couple the AoDs clusters to the AoAs clusters Gc  AoDs represent the first stage of clusters and AoAs represent the last stage of clusters. Any intermediate clusters and interactions can be abstracted by the coupling factors Gc,ij(AoAi,AoDj). The different AoDs that couple to AoA_i are denoted as AoDij.  A Tx beam (from set A or set B) is denoted as wk. A UE RX beam is denoted as um. The RX codebook is denoted as U.

By taking into consideration the testing set-up shown in Fig 5, the RSRP for a particular Tx beam is derived as shown below:

Fig. 6:  Proposed BM-Case 1 spatial prediction testing setup ( multiple static channels)

For testing the RSRP accuracy for BM-Case1 spatial prediction we can configure multiple static configuration tests, where each static scenario corresponds to a set of parameters for generating the channel realizations. For each test we configure UE to perform measurements on the set A and we treat those measurements as the genie measurements. The whole procedure is shown in Fig 6.

Proposal 14: For BM-Case 1 spatial prediction and for verifying RSRP accuracy we propose to use the testing setup and channel emulator functionality as described in Fig 6. (multiple static configurations)

For testing the RSRP accuracy for BM-Case2 temporal prediction we can configure a non-static configuration test, where the set of parameters for generating the channel realizations vary over time.Therefore, the channel realization vary due to inherent propagation conditions that should alter the optimum Tx beam over time and introduce some randomnsee and correlation over time to be captured by the AI/ML. Similar to BM-Case 1 testing procedure, we configure UE to perform measurements on the set A and we treat those measurments as the genie measurements. The whole procedure is shown in Fig 7.

Fig. 7:  Proposed BM-Case 2 temporal prediction testing setup ( non- static channels)

Proposal 15:  For testing the beam prediction/RSRP accuracy for BM-Case 2 temporal prediction we propose the framework described in this section and visualized in Fig 7, through a non static configuration of the channel emulator.

(2) Single ΑοΑ based Testing Setup

For DL Tx beam prediction, the model inputs consist of the L1-RSRP values of Tx beams in Set B, along with their corresponding Tx beam ID information, either explicitly provided or implicitly indicated. Essentially, only the final L1-RSRP value is critical, as all spatial characteristics are inherently captured in the RSRP measurements. Rather than simulating CDL channels and [64-128] Tx beams using multiple angles of arrival (AoAs) and departure (AoDs), the spatial-selective and time-varying propagation conditions can be effectively represented by adjusting the Tx power at the test equipment (TE) for each beam at different times. This approach ensures that the resulting RSRP values received at the test UE reflect the desired conditions. Notably, this method is already widely adopted in legacy RAN4 RRM testing.

Observation 9: Rather than simulating CDL channels and [64-128] Tx beams using multiple angles of arrival (AoAs) and departure (AoDs), the spatial-selective and time-varying propagation conditions can be effectively represented by adjusting the Tx power at the test equipment (TE) for each beam at different times

To simplify the testing setup, we can design it under the assumption of near-LOS conditions, which can be emulated using a single cluster and a single AoA probe. This is shown in Fig 7. Note that the multiple AoD links are not active simultenously. Only one link is active at a time, the purpose is to emulate the gains across different angles of the Tx antenna pattern to have multiple samples across the AoD domain and help making different Tx beams distinguishable. We also have one AoA/probe.

Fig. 8:  Single path/cluster (LOS condition) based Testing Setup

For each AoA (spatial sample) we collect RSRP inputs across setB beams (x,y) and we predict the RSRP at the same AoD for beams in set A (z). Let’s assume we consider the angle AoDj.  The received power at UE by using beam x is given by:

RSRPx,AoDj=PtGtx(x,AoDj)×Gpath(AoDj,AoA)×Grx(AoA)

Similarly the received power at UE by using beam y is given by:

RSRPy,AoDj=PtGtx(y,AoDj)×Gpath(AoDj,AoA)×Grx(AoA)

The AI/ML model will receive as inputs the RSRPs at set B beams and make a prediction as beam z (in set A):

RSRPz,AoDj=fAI/MLRSRPx,AoDj,RSRPx,AoDj

During test, we can generate multiple realizations of gains by varying the AoD as well as the gain Gpath(AoDj,AoA) to collect multiple RSRP samples across the set B beams and test the prediction against the RSRP of set A beams. The gain Gpath(AoDj,AoA) should remain constant during one realization of testing (sweeping of set B, set A beams).

In contribution [5] it was discussed that the multi-AoA test environment could be converted to a single AoA setup: The multi-AoA environment can be transformed into a single‑AoA test for AI/ML BM testing, by including the UE Rx beam/antenna pattern in the combination of BS Tx beams with the CDL channel model to be emulated, without impacting the number of BS Tx beams or the parameters of the CDL channel model.

The following options were identified:

We can differentiate 4 options how for AI/ML BM, a multi-AoA test can be transformed into a single-AoA test:

- Option 1: RAN4 defines a Reference Fixed UE Rx Beam/Antenna pattern. In this case, since the testing becomes primarily a “baseband test”, a second test would be required to address the UE spatial/directional performance.

- Option 2: RAN4 defines a Reference Complex UE Rx Beam/Antenna pattern (Codebook). Also in this case, testing becomes primarily a “baseband test”, thus a second test would be required to address the UE spatial/directional performance.

- Option 3: UE manufacturer provides the Complex UE Rx Beam/Antenna pattern (Codebook) to the TE prior to testing. Assuming that the pattern is realistic, the UE spatial/directional performance is covered in the test.

- Option 4: TE measures the Complex UE Rx Beam/Antenna pattern (Codebook) before starting test. This approach would rely on the basics of Radiated Two Stage Method (RTS) defined in [7]. In this case, the UE spatial/directional performance is covered in the test.

Similar concepts have been discussed in [6]. According to that contribution and for the setup B the following Pros and Cons were discussed:

Setup B) 1AoA + Fading simulator with wireless (virtual) cable connection

Pros

Though the number of test probe is physically one and DL signals are transmitted from one direction, it is possible to test with CDL channel models by using a fading simulator.

There are no limits to channel models (AoDs, AoAs, clusters, UE positions, etc.), and even possible to use the actual field data for the tests.

Possible to reuse the existing 1AoA OTA test system. i.e. Simpler test system compared to Setup A.

Cons

There is a need to include UE Rx beam pattern to a fading simulator, either standardized one or need to obtain actual UE beam patterns.

If the actual UE Rx beam patterns are necessary, there is a need to measure at least one beam pattern before the test to include it in the fading simulator. The number of necessary beam patterns are FFS. i.e. Longer test time is expected in a case the actual Rx beam pattern is necessary.

In a case the UE autonomously aims the Rx beam and then measures the RSRP and inputs it to the AI model in the field, it may not be possible to simulate the behaviour of the UE.

The measurements are made while beam-locked by the UBF (UE Beamlock Function) command, so they do not necessarily reproduce the UE in operation on the real network.

By considering the AI/ML spatial prediction as a purely power (RSRP) prediction problem and by considering that these power signal levels can be emulated by the fading channel emulator, we have the following proposal for a single AoA testing setup as it is shown in the flowchart in Figure 9.

Fig. 9:  Proposal for single AoA BM testing setup (flowchart)

The number of channel realizations (=640K) in the figure only serves as an example and for motivating the discussion on the tradeoff between testing complexity and sufficient number of channels to derive a test metric (top-K beam accuracy)

Proposal 16: RAN4 to consider the set of procedures/steps shown in the flowchart for adopting a single AoA BM testing setup

Proposal 17: RAN4 to evaluate the tradeoff between testing complexity (number of different channel realizations for power measurements, different number of parameters that need to be emulated in the fading simulator) and establishing accuracy on the test metric (top-K) beam prediction accuracy

Proposal 18: RAN4 to evaluate if a power measurement should be performed with a single Rx beam or the optimum Rx beam should be selected from an Rx codebook to measure the power of set B.

Proposal 19: RAN4 to discuss Option2 (a Reference Complex UE Rx Beam/Antenna pattern) for emulation of Rx patterns in the fading simulator

The equations described for the received power are based on powers (CDL power-based channel model). However, the channel model is described in the complex I-Q domain.

Proposal 20: Distinguish between the descriptions of power based CDL channel model and the original complex domain CDL channel model

Proposal 21: RAN4 to evaluate the prediction performance difference between a reference complex UE Rx beam/antenna pattern and its own complex pattern to ensure that testing will provide confidence that UE will perform well in the field if it passes the test

# Conclusion

In this section we summarize our discussion by listing the observations and proposals discussed in this contribution:

Observation 1: The measurement period for Set-B would depend on the accuracy needed for prediction.

Observation 2: The legacy definition for known TCI state cannot be used when target TCI state is based on predicted beam and not transmitted to and measured & reported by UE.

Observation 3: RAN4 needs to discuss known TCI for UE side and NW side prediction.

Observation 4: The testing environment for Beam Management (BM) case 1 and BM case 2 should replicate conditions that sufficiently capture the correlations of the transmit (Tx) and receive (Rx) beam patterns across the entire spectrum of propagation conditions in both spatial (angles of arrival and departure - AoAs and AoDs) and temporal domains.

Observation 5:  The following conditions will introduce randomness and variations in propagation conditions across both time and spatial domains for the computation of L1-RSRP

- Different AoDs with respect to the Tx antenna array

- Different AoAs with respect to the Rx antenna array

- Different superpositions of {AoA,AoD} pairs

- Fading/Variation in time domain ( different {AoA,AoD} pairs per resolvable delay bin path)

- UE movement (including rotation)

Observation 6:  During real-world deployment, UE will encounter random radio propagations characterized by variations in both spatial and temporal domains (fading). Testing UE under similar conditions is important to reflect the realities of deployment accurately.

Observation 7: The following questions need to be answered to evaluate the feasibility of the FR2 OTA-based test procedure:

- How can we generate multiple beams from the Set-B and Set-A Tx beams given the limitation of two AoAs?

- What assumption is made regarding Rx beam sweeping? Does the UE utilize a fixed Rx beam, or does it sweep to find the optimal Rx beam?

- How can we simultaneously emulate different AoAs (Rx beam) and AoD (Tx beams)?

- How can we achieve dynamic variation in the AoD domain (Tx beam sweeping) for BM case 2 prediction?

Observation 8: For RAN 4 testing we should have the following goals:

(1) Provide confidence that if a DUT passes the test it will also perform well in the field (2) Ensure reliability of using synthetic channels for test data in evaluating models trained on real data.

Observation 9: Rather than simulating CDL channels and [64-128] Tx beams using multiple angles of arrival (AoAs) and departure (AoDs), the spatial-selective and time-varying propagation conditions can be effectively represented by adjusting the Tx power at the test equipment (TE) for each beam at different times

complexity (number of different channel realizations for power measurements, different number of parameters that need to be emulated in the fading simulator) and establishing accuracy on the test metric (top-K) beam prediction accuracy

Proposal 1: Use legacy measurement period as a starting point for Set-B for evaluation if sufficient to achieve good prediction accuracy.

Proposal 2: For AI-BM Case 2, the number of L1-RSRP measurement occasions on Set-B should be configurable and studied further in RAN1.

Proposal 3: For UE side model known TCI state is determined based on Set B transmission and UE report of top-K.

Proposal 4: For NW side model - 
(1) TCI state is unknown if RS of target beam is not transmitted to UE prior to TCI state switch
(2) If RS of target beam is transmitted to the UE and reported, legacy definition of known TCI state is applicable

Proposal 5: NW side data collection, QCL type D is signaled between set A and set B RS.

Proposal 6: For NW side model, TCI state is known if QCL source is signaled in both data collection and inference (to ensure UE Rx consistency), it is known. Otherwise, it is unknown.

Proposal 7: (for Alt 1):  Define UE capability for Alt 1 of UE implementation. When UE does not support this capability, the TCI state is unknown. Unknown TCI state switching delay is applied.

Proposal 8: (for Alt 2): When gNB signal the QCL relationship between set A and set B, in training RS configuration, inference RS configuration and performance monitoring RS configuration (example of Alt 2 NW beam implementation of set A and set B),

The TCI state is known when the following is met:

Target TCI state is based on RS from set A beams with QCL relationship configured to a set B beam

The time between TCI state switch command and transmission of the QCLed RS from set B shall not exceed X ms

X = 1280 ms (legacy)

UE shall send measurement report for the QCLed RS from Set B between transmission and TCI state switch

TCI state and SSB of TCI state remain detectable during TCI state switching period

Otherwise, it is unknown.

Proposal 9: The ability to identify the Rx beam associated with an unmeasured predicted Tx beam may rely on the UE’s supported features

Proposal 10: RAN4-defined prediction accuracy requirements is proposed in the context of performance monitoring Type 1, and for Option 2 (UE-assisted performance monitoring)

Proposal 11: For UE-side AI/ML beam management, LCM operations such as activation, switching, and fallback can be supported via CSI report configurations. RAN4 should evaluate the latency impact of these operations for periodic, semi-persistent, and aperiodic CSI reporting modes. The analysis should cover BM-Case1 and BM-Case2, considering processing time, measurement intervals, and model availability in the UE.

Proposal 12: Multi AoA based Testing Setup for BM: Evaluate the testability requirements for simulating time-varying input power to a sparse probe layout based on a simplified channel CDL model.  The key aspects to be considered are:

1.     How to determine the minimum number of clusters to be emulated without introducing bias in the results. Investigate a quantitative “goodness” criterion to selecting the number of probes/clusters.

2.     Consider incorporating UE rotation for the test

3.     How many different CDL channels we need to support (considering generalization purposes as well)

Proposal 13: To establish a criterion for assessing the feasibility of a simplified sparse probe layout (Option 5) for beam management (BM) testing, the following procedure can be considered:

- Train a model using the reference CDL channel and test it with the simplified sparse layout-based CDL channel.

- Train a model using the simplified CDL channel and test it with the reference CDL channel model.

- Train and test a model using the reference CDL channel.

A similarity metric should be defined to compare the RSRP accuracies across these test cases. This metric would indicate the effectiveness of testing with the simplified CDL model. The similarity metric can be derived through simulations.

Proposal 14: For BM-Case 1 spatial prediction and for verifying RSRP accuracy we propose to use the testing setup and channel emulator functionality as described in Fig 6. (multiple static configurations)

Proposal 15:  For testing the beam prediction/RSRP accuracy for BM-Case 2 temporal prediction we propose the framework described in this section and visualized in Fig 7, through a non static configuration of the channel emulator.

Fig. 9:  Proposal for single AoA BM testing setup (flowchart)

Proposal 16: RAN4 to consider the set of procedures/steps shown in the flowchart for adopting a single AoA BM testing setup

Proposal 17: RAN4 to evaluate the tradeoff between testing complexity (number of different channel realizations for power measurements, different number of parameters that need to be emulated in the fading simulator) and establishing accuracy on the test metric (top-K) beam prediction accuracy

Proposal 18: RAN4 to evaluate if a power measurement should be performed with a single Rx beam or the optimum Rx beam should be selected from an Rx codebook to measure the power of set B.

Proposal 19: RAN4 to discuss Option2 (a Reference Complex UE Rx Beam/Antenna pattern) for emulation of Rx patterns in the fading simulator

Proposal 20: Distinguish between the descriptions of power based CDL channel model and the original complex domain CDL channel model

Proposal 21: RAN4 to evaluate the prediction performance difference between a reference complex UE Rx beam/antenna pattern and its own complex pattern to ensure that testing will provide confidence that UE will perform well in the field if it passes the test

- References

[1] RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#94e.

[2] RP-242399 - Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,

[3] 3GPP TR 38.843, “Technical Specification Group Radio Access Network;Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface,

[4] TR 38.810, v16.7.0, “Study on test methods”.

[5] R4-2501726, “ Discussion on test setup for AI/ML based beam management”, R&S Feb 2025

[6] R4-2500167,  “FR2 test setup for AI/ML beam management evaluation”, Anritsu Feb 2025