3GPP TSG-RAN WG4 Meeting #115                            R4-2505943

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.22.2

Source:	Apple

Title:	General aspects for NR air interface Phase 2

Release:       Rel-19

Document for:	Discussion

# 1 Introduction

Rel-19 Work Item (WI) was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (WID in [1]). The application of AI/ML techniques to NR air interface has been studied in FS_NR_AIML_Air.

This work item provides normative support for the general framework of AI/ML concerning air interfaces. It also enables the implementation of recommended use cases outlined in the previous study. Furthermore, several study objectives within this project aim to address outstanding issues identified during the study, with the goal of enhancing understanding in preparation for future normative effort.

The current agreements on how to perform the RAN4 study on general issues for AI/ML, and issues related to interoperability/testing have been captured in the latest TR [2]

In this contribution, we provide our viewpoints on some of the interoperability and testability aspects for AI/ML for NR air interface for CSI compression and prediction.

# 2 Discussion

## 2.1 Generalization/scalability aspects

The performance of generalization is of importance for a successful AI/ML feature deployment and the necessity and feasibility of defining requirements or test to verify the generalization of AI/ML has been studied. The content below from the TR discusses achieving minimum performance under certain conditions while ensuring that performance is not significantly degraded under other conditions Accordingly for the generalization verification aspects, the following contents are agreed to be captured in TR 38.843 [1]:

The goals of generalization test are to verify whether the minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations. The following aspects should be considered for generalization/scalability related testing: - details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality - what the minimum level performance for each identified scenario and/or configuration is - what the significant degradation for other scenarios and/or configurations is It should also be considered that generalization and/or scalability related requirements for different scenarios/ configurations can be implicitly handled in the test case definition. As for the handling of generalization tests, the following option is considered as baseline: Signaling based LCM procedures and performance monitoring are considered in dedicated test cases and are excluded in tests verifying generalization. RAN4 may define multiple tests with different conditions. In each of the test, TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify its generalizability. (environment differs in each test but not changing dynamically during the test) - Specified UE configuration includes functionality and/or model ID if defined

Consider the following aspects regarding the different conditions for testing generalization for CSI AI/ML use:

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Various antenna spacings (e.g., 0.5 lambda, 0.8 lambda, etc)

- Various antenna virtualization (TxRU mapping)

- Various carrier frequencies and bands (e.g., 2GHz, 4.0GHz)

- Various outdoor/indoor UE distributions for UMa/Umi

- Various UE speeds.

Consider the following aspects regarding the scalability aspect for generalization testing for CSI AI/ML use:

- Various bandwidths (e.g., 20MHz, 50MHz) and/or frequency granularities, (e.g., size of subband), different layers

- Various sizes of CSI feedback payloads

Proposal 1: Consider the following aspects regarding the different conditions for testing generalization for CSI AI/ML use:

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Various antenna spacings (e.g., 0.5 lambda, 0.8 lambda, etc)

- Various antenna virtualization (TxRU mapping)

- Various carrier frequencies and bands (e.g., 2GHz, 4.0GHz)

- Various outdoor/indoor UE distributions for UMa/Umi

- Various UE speeds.

Consider the following aspects regarding the scalability aspect for generalization testing for CSI AI/ML use:

- Various bandwidths (e.g., 20MHz, 50MHz) and/or frequency granularities, (e.g., size of subband), different layers

- Various sizes of CSI feedback payloads

## 2.4 Monitoring Procedures/KPI

During the RAN1#117 meeting, the following agreements on model monitoring are agreed to be studied further

For UE side monitoring, UE can calculate the intermediate KPI between the ground-truth CSI and the recovery CSI (i.e., output of the CSI reconstruction model) indicated by NW. eT2-like high-resolution codebook for reporting format and SGCS for the type of intermediate KPI are likely used to deliver the recovery CSIs. This will create significant signaling overhead.

Instead, we could rely on the input CSI data distribution to detect any anomaly or data drift. Channel State Information (CSI) reflects the state of the wireless channel, which is influenced by the environment (e.g., movement of objects, user mobility) and the system configuration (e.g., antenna array changes, beamforming adjustments). Any significant change in the environment or configuration/scenario will inherently alter the statistical or semantic properties of the CSI samples. We aim at reducing the frequency of monitoring procedures unless there is an indication of an event change that would indicate that the AI/ML model has drifted or the input data distribution to the AI/ML model has drifted

Observation 1: Input CSI data distribution can be used to detect a monitoring event. By detecting monitoring events without ground truth CSI excessive overhead signaling can be avoided

By continuously monitoring how these properties evolve over time, sudden deviations or trends can be detected, signaling a monitoring event. Performing monitoring operations is expensive due to the necessity of signaling the ground truth and/or the assumption of the availability of the ground truth

Proposal 2: Investigate CSI similarity of input data distributions using Angular-Delay Channel Entropy ADCE as a statistical measure. This approach aims to reduce the reliance on more expensive monitoring procedures, such as those based on KPIs and ground truth availability, by initiating these procedures only when significant changes in data distribution are detected. The ADCE can be defined as ADCE=ijPijlog2Pij, where Pij=Hij2||H||F2, and Hij=FdHFaH is a virtual channel representation in angular/delay domain which separates multipath components into resolvable delay taps and spatial angles. ADCE captures the sparsity or randomness of these components.

By monitoring this metric (its computation is inexpensive) and by confirming similarity there is no need to employ the more expensive monitoring procedures (monitoring based on KPIs and availability of the ground truth). Only when the computation of ADCE indicates an event change the KPI based monitoring procedures will be initiated

Another advantage of ADCE is that it can be used as a Measure of Smoothness and Correlation: In the spatial-frequency domain, smoothness or correlation in CSI samples indicates how coherent or predictable the wireless channel is across space and frequency. A low ADCE value suggests a more predictable (smooth/correlated) channel, meaning power is concentrated in a smaller number of dominant components. A high ADCE value indicates a less predictable (less smooth, less correlated) channel, meaning power is distributed more evenly across components. Lower ADCE implies higher sparsity in the delay-beam domain, which simplifies the representation of CSI samples and enhances the efficiency of AI/ML-based compression methods.

Dynamic configuration can obtain an optimal tradeoff between signaling overhead (payload size), performance under various channels and noise conditions. By doing so the UE/NW could adaptively adjust the compression process, optimizing the balance between feedback overhead and accuracy in representing the channel state

Proposal 3: RAN4 to investigate defining metrics for Monitoring CSI Input Distribution to Trigger Monitoring Procedures and their Application to Dynamic CSI Compression

To facilitate network-side (NW) monitoring with minimal feedback overhead, we propose an approach to insert a small number of redundancy elements into the input CSI vector of the encoder at the UE. This allows the NW to use the reconstructed redundancy elements to assess the quality of the CSI after decompression. This method enables the NW to monitor the quality of each CSI feedback in real-time. These redundancy elements, which are additional elements of CSI samples added to the original CSI vector, serve as a reference for the NW to assess the quality of the CSI once it is decompressed. By embedding only, a small number of redundancy elements into the feedback, this approach introduces very little additional overhead, making it suitable for reducing the signaling overhead for monitoring procedures

Redundancy Elements in Input CSI where a subset of known values is embedded within the CSI vector before encoding at UE These elements are designed to be distinguishable and remain consistent across encoding and decoding processes. Fixed Reference Patterns: Predefined sequences (e.g., CSI patterns of small matrices) known to both the UE and NW. Derived Elements: Generated from portions of the CSI vector using known mathematical functions, such as low-dimensional projections.

Proposal 4: For Monitoring CSI AI/ML Models investigate embedding a subset of known values, referred to as redundancy elements, into the CSI vector at the UE before encoding. These elements act as reference points that can be tracked throughout the encoding and decoding processes. The approach is designed to facilitate the monitoring of AI/ML models by providing a measurable and consistent benchmark for evaluating model performance.

For CSI model monitoring identifying the root cause of degradation is important. We propose dividing the process into two stages.

The initial stage focuses on determining whether the AI/ML model is the source of performance degradation or if external factors are at play. This involves assessing the effectiveness of the CSI generation and reconstruction processes, particularly through analyzing the SGCS the encoder/decoder pair which represents nominal CSI feedback. If the first stage identifies the AI/ML model as a potential cause, the second stage proceeds to isolate the exact issue within the model, enabling a more targeted investigation.

Proposal 5: We propose a two-stage approach for CSI model monitoring, first distinguishing whether performance degradation stems from the AI/ML model or external factors and then isolating the specific issue within the model if identified as the cause.

We are also facing with the challenge of how to define requirements for Model Monitoring?  How to evaluate if the monitoring procedure is “Accurate”?

Current approach: Accuracy of the KPI Model monitoring metric: In R18 study, SGCS accuracy for model monitoring is computed by averaging the per-sample difference between the genie SGCS and calculated SGCS based on ground-truth reporting and SGCS estimation, i.e., ESGCSgenie-SGCSmeasured

However, this accuracy metric fails to reflect the performance of decision-making processes critical for monitoring. We would prefer to make use of a different approach: performance of decision-making processes that indicates the accuracy of predicting an event that indicates AI/ML model drift and performance degradation

Monitoring systems are not just about measuring accuracy; they are about identifying and reacting to anomalies, model degradation, or environmental changes.

Therefore, there is a need for Decision-Making Accuracy:

The primary goal of performance monitoring is to detect issues (like data drift, model degradation, hardware faults) and trigger appropriate actions (e.g., model re-training, re-configuration, AI/ML model switch or deactivation).

Thus, what's more important is the accuracy of monitoring decisions rather than just the numerical accuracy of SGCS measurements.

Performance monitoring in AI/ML-driven wireless networks is essential to detect issues such as data drift, model degradation, and hardware faults, and to trigger appropriate actions like model re-training, re-configuration, or deactivation. To achieve this, the system must ensure high event detection accuracy with low false positives and negatives, real-time monitoring, and multi-dimensional metrics. Proactive prediction capabilities, adaptive thresholds, and automated decision-making are critical to address issues before they impact performance. The implementation requires scalable and flexible solutions, including robust data collection, advanced detection algorithms, and predictive models. By meeting these requirements, the system will enhance reliability, maintain optimal performance, and ensure seamless adaptation to changing channel conditions.

Proposal 6: RAN4 to investigate a performance monitoring framework to detect issues such as data drift, model degradation, and AI/ML model aging, triggering actions like re-training, re-configuration, or model switching. The system will ensure high detection accuracy, real-time monitoring, and proactive predictions, enabling reliable adaptation to changing channel conditions while maintaining optimal performance.

Proposal 7: For a performance monitoring framework, event detection for data distribution drift and anomalous behavior is based on the output of a trained model that predicts performance deviations (the event). Consider the following cases as shown below:

(1) Indirect Event Detection: AI/ML model for SGCS EST is trained to predict SGCS from latent space c. Subsequently a detection logic can trigger event detection (2) Direct Event Detection is based on AI/ML model to directly predict an event

Fig 13. Performance monitoring framework for Event detection

There are couple of issues by using SGCS as an intermediate KPI for CSI compression. We list the issues below:

- SGCS Doesn't Capture Information or Capacity Loss Directly

SGCS only measures angular similarity between CSI matrices, ignoring magnitude differences that impact channel quality. Information loss due to quantization, feedback delay, or imperfect CSI compression is not explicitly reflected in SGCS. Capacity loss is a function of channel power and spatial correlation, but SGCS does not account for power scaling effects, which are crucial in MIMO systems.

- SGCS is Weakly Correlated with Throughput

Throughput depends on spectral efficiency, which is determined by the achievable rate formula. SGCS does not directly relate to the determinant or eigenvalues of H, making it an incomplete indicator of spectral efficiency. The impact of CSI errors on MCS selection and beamforming is non-trivial, making it hard to infer actual throughput loss from SGCS degradation.

- SGCS Does Not Capture CSI Feedback Imperfections Well

Compression artifacts: If a UE sends quantized CSI feedback, the distortion in signal space may not align with angular deviations captured by SGCS. Interference and noise effects: SGCS does not distinguish between CSI errors due to channel noise versus quantization/compression, leading to ambiguities in interpreting feedback accuracy.

Observation 2:

- SGCS Doesn't Capture Information or Capacity Loss Directly

SGCS only measures angular similarity between CSI matrices, ignoring magnitude differences that impact channel quality. Information loss due to quantization, feedback delay, or imperfect CSI compression is not explicitly reflected in SGCS. Capacity loss is a function of channel power and spatial correlation, but SGCS does not account for power scaling effects, which are crucial in MIMO systems.

- SGCS is Weakly Correlated with Throughput

Throughput depends on spectral efficiency, which is determined by the achievable rate formula. SGCS does not directly relate to the determinant or eigenvalues of H, making it an incomplete indicator of spectral efficiency. The impact of CSI errors on MCS selection and beamforming is non-trivial, making it hard to infer actual throughput loss from SGCS degradation.

- SGCS Does Not Capture CSI Feedback Imperfections Well

Compression artifacts: If a UE sends quantized CSI feedback, the distortion in signal space may not align with angular deviations captured by SGCS. Interference and noise effects: SGCS does not distinguish between CSI errors due to channel noise versus quantization/compression, leading to ambiguities in interpreting feedback accuracy.

We aim at establishing a KPI metric which addresses the issues discussed with SGCS and it provides a close relation to throughput and spectral efficiency. For this purpose, lets denote a general precoder matrix P. Under ideal conditions (no compression) the ideal CSI would be denoted as V. Under AI/ML model compression and other artifacts the reconstructed CSI is Vc, where c denotes corrupted CSI. Our goal is to derive an approximate formula that describes the capacity loss under two different effective channels: HP|P=V  and : HP|P=Vc .  Assuming we have a system with rank L. Then the system capacity is given by the formula: C=k=1Llog21+SINRk, where SINRk takes into account the interstream interference and it depends on the MIMO receiver.

Proposal 8:  Consider a new KPI for CSI compression that could offer an indication of spectral efficiency loss due to misalignment of precoders. It can be conceptually described as KPI:Closs=C​P=V-C​P=Vc where Vc is the corrupted reconstructed CSI

Proposal 9: Investigate the benefits of defining a new KPI for CSI compression that is linked to capacity loss. For an SVD receiver this metric is  approximated as Closs,SVD≈k=1Llog2ρρIkλk/L+1ρIk+vkHvck2, where ρIk=j=1,j≠kLvkHvck2  is defined as the inter-stream interference to layer k caused by the mismatch between Vc and V.  Also ρ is the SNR and λk is the eigenvalue of the channel H'H

For rank 1: the proposed metric becomes Closs=-log2v1Hvc1, while SGCS=v1Hvc122/vc122.  Under ideal conditions the term ρIk = 0 due to the orthogonality. But under imperfection this term contributes to the denominator and the capacity decreases.

Another advantage of the proposed KPI is that it could be used as an compensation factor to determine the CQI.

In legacy CSI report using PMI codebook, since the PMI construction algorithm is not complicated, it is feasible for UE to construct the PMI for CQI calculation. In this way, the precoder used in CQI calculation is same as the precoding vectors reported to the gNB. However, in AI/ML based CSI compression and feedback, the CSI reconstruction model and CSI generation model are normally run at gNB side and UE side separately, it may or may not be feasible for UE to follow the legacy principle. In RAN1 the following discussions and options are available:

Proposal 10: Investigate the feasibility of using the spectral efficiency KPI to determine the CQI adjustment for option 1b.

# Conclusions

In conclusion the following observations and proposals were discussed in this contribution:

Observation 1: Input CSI data distribution can be used to detect a monitoring event. By detecting monitoring events without ground truth CSI excessive overhead signaling can be avoided

Observation 2:

- SGCS Doesn't Capture Information or Capacity Loss Directly

SGCS only measures angular similarity between CSI matrices, ignoring magnitude differences that impact channel quality. Information loss due to quantization, feedback delay, or imperfect CSI compression is not explicitly reflected in SGCS. Capacity loss is a function of channel power and spatial correlation, but SGCS does not account for power scaling effects, which are crucial in MIMO systems.

- SGCS is Weakly Correlated with Throughput

Throughput depends on spectral efficiency, which is determined by the achievable rate formula. SGCS does not directly relate to the determinant or eigenvalues of H, making it an incomplete indicator of spectral efficiency. The impact of CSI errors on MCS selection and beamforming is non-trivial, making it hard to infer actual throughput loss from SGCS degradation.

- SGCS Does Not Capture CSI Feedback Imperfections Well

Compression artifacts: If a UE sends quantized CSI feedback, the distortion in signal space may not align with angular deviations captured by SGCS. Interference and noise effects: SGCS does not distinguish between CSI errors due to channel noise versus quantization/compression, leading to ambiguities in interpreting feedback accuracy.

Proposal 1: Consider the following aspects regarding the different conditions for testing generalization for CSI AI/ML use:

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Various antenna spacings (e.g., 0.5 lambda, 0.8 lambda, etc)

- Various antenna virtualization (TxRU mapping)

- Various carrier frequencies and bands (e.g., 2GHz, 4.0GHz)

- Various outdoor/indoor UE distributions for UMa/Umi

- Various UE speeds.

Consider the following aspects regarding the scalability aspect for generalization testing for CSI AI/ML use:

- Various bandwidths (e.g., 20MHz, 50MHz) and/or frequency granularities, (e.g., size of subband), different layers

- Various sizes of CSI feedback payloads

Proposal 2: Investigate CSI similarity of input data distributions using Angular-Delay Channel Entropy ADCE as a statistical measure. This approach aims to reduce the reliance on more expensive monitoring procedures, such as those based on KPIs and ground truth availability, by initiating these procedures only when significant changes in data distribution are detected. The ADCE can be defined as ADCE=ijPijlog2Pij, where Pij=Hij2||H||F2, and Hij=FdHFaH is a virtual channel representation in angular/delay domain which separates multipath components into resolvable delay taps and spatial angles. ADCE captures the sparsity or randomness of these components.

Proposal 3: RAN4 to investigate defining metrics for Monitoring CSI Input Distribution to Trigger Monitoring Procedures and their Application to Dynamic CSI Compression

Proposal 4: For Monitoring CSI AI/ML Models investigate embedding a subset of known values, referred to as redundancy elements, into the CSI vector at the UE before encoding. These elements act as reference points that can be tracked throughout the encoding and decoding processes. The approach is designed to facilitate the monitoring of AI/ML models by providing a measurable and consistent benchmark for evaluating model performance.

Proposal 5: We propose a two-stage approach for CSI model monitoring, first distinguishing whether performance degradation stems from the AI/ML model or external factors and then isolating the specific issue within the model if identified as the cause.

Proposal 6: RAN4 to investigate a performance monitoring framework to detect issues such as data drift, model degradation, and AI/ML model aging, triggering actions like re-training, re-configuration, or model switching. The system will ensure high detection accuracy, real-time monitoring, and proactive predictions, enabling reliable adaptation to changing channel conditions while maintaining optimal performance.

Proposal 7: For a performance monitoring framework, event detection for data distribution drift and anomalous behavior is based on the output of a trained model that predicts performance deviations (the event). Consider the following cases as shown below:

(1) Indirect Event Detection: AI/ML model for SGCS EST is trained to predict SGCS from latent space c. Subsequently a detection logic can trigger event detection (2) Direct Event Detection is based on AI/ML model to directly predict an event

Proposal 8:  Consider a new KPI for CSI compression that could offer an indication of spectral efficiency loss due to misalignment of precoders. It can be conceptually described as KPI:Closs=C​P=V-C​P=Vc where Vc is the corrupted reconstructed CSI

Proposal 9: Investigate the benefits of defining a new KPI for CSI compression that is linked to capacity loss. For an SVD receiver this metric is  approximated as Closs,SVD≈k=1Llog2ρρIkλk/L+1ρIk+vkHvck2, where ρIk=j=1,j≠kLvkHvck2  is defined as the inter-stream interference to layer k caused by the mismatch between Vc and V.  Also ρ is the SNR and λk is the eigenvalue of the channel H'H

For rank 1: the proposed metric becomes Closs=-log2v1Hvc1, while SGCS=v1Hvc122/vc122.  Under ideal conditions the term ρIk = 0 due to the orthogonality. But under imperfection this term contributes to the denominator and the capacity decreases.

Proposal 10: Investigate the feasibility of using the spectral efficiency KPI to determine the CQI adjustment for option 1b.

# Reference

[1] RP-242399 - Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

[2] 3GPP TR 38.843, “Technical Specification Group Radio Access Network;Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”