3GPP TSG-RAN WG4 Meeting #115                                 R4-2505946

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.21.7

Source:	Apple

Title:		Demodulation and/or CSI reporting requirements for CSI prediction

Release:       Rel-19

Document for:	Discussion

# Introduction

Rel-19 Work Item (WI) was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (WID in [1]). The application of AI/ML techniques to NR air interface has been studied in FS_NR_AIML_Air.

This work item provides normative support for the general framework of AI/ML concerning air interfaces. It also enables the implementation of recommended use cases outlined in the previous study. Furthermore, several study objectives within this project aim to address outstanding issues identified during the study, with the goal of enhancing understanding in preparation for future normative effort.

The current agreements on how to perform the RAN4 study on general issues for AI/ML, and issues related to interoperability/testing have been captured in the latest TR [2]

In this contribution, we provide our viewpoints on some of the interoperability and testability aspects for AI/ML for NR air interface for CSI compression and prediction.

# Demodulation and CSI Reporting Requirements

In the previous meeting RAN4#112, the following agreements was reached for CSI prediction.

Agreement:

Use relative throughput as requirement metric for CSI prediction

- companies are invited to bring contribution on how the requirement could be defined:

  - what is the baseline that throughput would be compared to

using relative throughput as the test metric for inference, the baseline denominator for relative throughput still requires further clarification.

In the current PMI test case, RAN4 uses random PMI as the baseline to evaluate whether the UE-reported PMI is accurate enough. This approach is analogous to a scenario where no UE reporting occurs.

Issue 2-1: Channel model for CSI prediction tests

Agreement:

- For channel model for CSI prediction tests, TDL channel models will be used.

  - FFS on whether CDL channel model is needed for generalization test.

Issue 2-2: Reference throughput

Agreement:

- reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook

Issue 2-3: Testing conditions

Agreement:

- use static conditions as baseline.

  - FFS on whether to have non-static condition

Issue 2-5: Prediction delay requirements

Agreement:

- Prediction delay includes:

  - Measurement for prediction

    - FFS on whether the legacy measurement delay can be reused

  - Inference delay

  - Other components are FFS

The following issues/agreements were discussed in RAN4#114:

Issue 1-2: Reference model for CSI prediction

Agreement:

- Assume no reference AI model for CSI prediction as baseline

  - If it is difficult to align the results corresponding to test metric, RAN4 will discuss the reference model.

Issue 1-3: Training and test data consistency

Agreement:

- further discuss the following options

  - Option 1: Consistency between training data and test data should be ensured

    - RAN4 to study how to ensure consistency between training data(simulation) and test data (conformance test) and whether anything special should be done during the test

  - Option 2: Consistency will be implicit as test emulated channel will be very similar to the one used in RAN4 simulations

Issue 1-4: Generalization

Agreement:

- Postpone this discussion until the requirement and test definition is clear

According to the agreement, relative throughput should be tested for prediction accuracy. The γ value comparing predicted PMI versus random PMI can be used in the test. To demonstrate the performance gain of AI-based CSI prediction, the γ value from the AI-based prediction would differ from the γ value observed in previous non-AI tests.

Proposal 1: The γ value comparing predicted PMI to random PMI can be used in the test for CSI prediction. To infer the gains of AI/ML-based CSI prediction, the γ values can be compared between AI/ML and non-AI/ML cases.

During RAN4#112bis it was discussed whether the new requirements on AI/ML-based functionality should be worse or better than the requirements on the legacy CSI feedback:

Issue 1-4: AI/ML Performance Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

Given that the requirements for legacy non-AI/ML CSI feedback mechanisms are well-established, it would be advantageous and practical to evaluate the performance of AI/ML-based CSI prediction using the same test parameters that have already been defined.

- Proposal 2: RAN4 should assess the performance of AI/ML-based CSI prediction using the same test configurations and conditions as those defined for non-predicting eTypeII and/or eTypeII Doppler-r18.

It is important to recognize that relying solely on legacy channel models such as TDL for testing may not adequately reflect the true effectiveness of an AI/ML-based solution. Since data-driven models tend to learn from and adapt to complex channel characteristics, the simplified nature of the TDL model is often insufficient to evaluate their generalization capabilities. A model performing well under TDL conditions does not guarantee comparable performance under more realistic scenarios like UMa, CDL, or actual field channels. Therefore, while TDL-based tests can serve as a baseline, it is recommended that CSI-related performance evaluations also include CDL or other representative channel models to ensure a more comprehensive and practical assessment.

Proposal 3: To better evaluate the generalization capability of such models, it is proposed that CSI-related testing also include CDL or other representative channel models that better reflect practical conditions.

Test setup for CSI-RS resource

For Rel-18 eType II doppler codebook test, only 1 time distance for prediction was considered, and K as 4 for both FDD and TDD, the agreed test setup was described as following

Agreement in RAN4#110bis Test setup for FR1 FDD case of TypeII-Doppler-r18 codebook Test setup for FR1 TDD case of TypeII-Doppler-r18 codebook

For RAN1 evaluation for AI CSI, both periodic and Aperiodic CSI-RS configuration are considered.

Agreement in RAN1#116 For the evaluation of the AI/ML based CSI prediction, consider the following CSI-RS configuration - Periodic: 5ms periodicity (baseline), 20 ms periodicity (encouraged) - Aperiodic: Optional, CSI-RS burst with K resources and time interval m slots (based on R18 MIMO eType -II ) - Note: Companies to report observation window (number/distance) and prediction window Agreement in RAN1#116bis For the AI/ML based CSI prediction, adopt the following assumption as a baseline for evaluation purpose - UE speed: 30km/h, 60km/h - Others can be additionally submitted, e.g., 10km/h, 120km/h - Observation Window (number/distance): 5/5ms, 10/5ms - Others can be additionally submitted, e.g., 4/5ms, 15/5ms - Prediction window (number/distance between prediction instance/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms, 4/5ms/5ms - Others can be additionally submitted, e.g., 2/5ms/5ms, 3/5ms/5ms, 1/5ms/10ms - For other assumption, reuse Rel-18 baseline Agreement in RAN1#116bis For the evaluation of the AI/ML based CSI prediction, adopt following as a baseline for evaluation purpose - N4 value: 1, 4 - Others can be additionally submitted, e.g., 2, 8 - paramCombination-Doppler-r18: 6,7 or paramCombination -r16 = 5,6 (for Benchmark 1) - Others can be additionally submitted - Note: The same selected parameter combination shall be applied for benchmarks - CSI report periodicity: 5ms, 20ms (encouraged) - Other can be additionally submitted, e.g., 10ms

RAN4 should further discuss the proper test setup for AI/ML based prediction based on Rel-18 Type II-Doppler codebook.

Proposal 4: RAN4 to further explore and define appropriate test setups for evaluating AI/ML-based CSI prediction utilizing the Rel-18 Type II-Doppler codebook. As an initial step, a focus on periodic CSI-RS configurations under FDD operation can serve as a practical baseline.

The following agreement was reached for the way forward for next steps in simulations for CSI prediction:

Way forward for next step simulation for CSI prediction

  - For step-1,

    - Interested companies are encouraged to provide the 2nd round of CSI prediction simulation results in RAN4#115 meeting.

  - For step-2,

    - Simulation assumptions for step-2 are updated in RAN4#114bis meeting.

    - Interested companies are also encouraged to provide the 1st round of CSI prediction simulation results (SGCS as KPI, SGCS between the eigenvectors of predicted PMI derived from the ideal DL channel estimation and ground-truth eigenvectors), in RAN4#115 meeting.

  - For step-3,

    - CSI prediction simulation (TPs as KPI, with practical DL channel estimation) is postponed to RAN4#116 meeting.

    - Note: Simulation assumptions for CSI prediction (TPs as KPI, with practical DL channel estimation) will be updated further according to the progress of RAN1&RAN4 in RAN4#115. Companies are encouraged to check the updated assumptions and input suggestions/comments in RAN4#115 meeting.

  - Companies are also encouraged to provide the “sample and hold” results for above simulations for steps 1 and 2.

A single AI/ML model is generally insufficient to provide accurate CSI prediction across the full SNR range, particularly at low SNR levels where noise significantly disrupts channel correlation. Training with low-SNR data often leads to poor model generalization and degraded prediction performance. As such, the choice of SNR conditions for training data collection requires careful evaluation. In conventional PMI testing, the SNR corresponding to 90% of the throughput achieved with optimal PMI selection is used to assess performance gain over random PMI. To ensure that AI/ML-based CSI prediction models are robust and generalizable, training data should not be limited to a single SNR value but should span a range of SNR conditions centered around a well-defined reference point. A practical and performance-relevant reference is the SNR level at which 90% of the maximum throughput is achieved using AI/ML-predicted precoding. From this reference SNR, training data should be collected across a defined range (e.g., ±3–5 dB), which captures variations in signal quality that the model may encounter in real deployments. This approach helps the model learn to handle moderate channel degradations and ensures that its performance does not sharply degrade outside of a narrow SNR window.

Proposal 5: To enhance the robustness and generalization of AI/ML-based CSI prediction models, RAN4 should consider a structured approach to training data collection that combines mixed channel conditions and practical channel estimation scenarios. To build models resilient to real-world variability, data should be collected across a range of SNR values (e.g., ±3–5 dB from the reference point) and include a mix of practical channel models (e.g., CDL, UMa) and realistic estimation impairments. This mixed dataset strategy ensures the trained model can adapt to varying deployment conditions while maintaining stable prediction performance.

# Conclusion

In conclusion the following observations and proposals were discussed in this contribution:

Proposal 1: The γ value comparing predicted PMI to random PMI can be used in the test for CSI prediction. To infer the gains of AI/ML-based CSI prediction, the γ values can be compared between AI/ML and non-AI/ML cases.

- Proposal 2: RAN4 should assess the performance of AI/ML-based CSI prediction using the same test configurations and conditions as those defined for non-predicting eTypeII and/or eTypeII Doppler-r18.

Proposal 3: To better evaluate the generalization capability of such models, it is proposed that CSI-related testing also include CDL or other representative channel models that better reflect practical conditions.

Proposal 4: RAN4 to further explore and define appropriate test setups for evaluating AI/ML-based CSI prediction utilizing the Rel-18 Type II-Doppler codebook. As an initial step, a focus on periodic CSI-RS configurations under FDD operation can serve as a practical baseline.

Proposal 5: To enhance the robustness and generalization of AI/ML-based CSI prediction models, RAN4 should consider a structured approach to training data collection that combines mixed channel conditions and practical channel estimation scenarios. To build models resilient to real-world variability, data should be collected across a range of SNR values (e.g., ±3–5 dB from the reference point) and include a mix of practical channel models (e.g., CDL, UMa) and realistic estimation impairments. This mixed dataset strategy ensures the trained model can adapt to varying deployment conditions while maintaining stable prediction performance

# Reference

[1] RP-242399 – “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”

[2] 3GPP TR 38.843, “Technical Specification Group Radio Access Network;Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”