# 3GPP TSG-WG RAN4 Meeting #115	                                                 R4-2505955

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 5004 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 36.104 : New bands for LTE based 5G terrestrial broadcast for early deployments, Rel-19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | EBU, BNE, ORS, Qualcomm Incorporated, SWR |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_new_bands |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of bands in the UHF frequency range to support early deployment of 5G broadcast. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 5.5 : Defined 5G broadcast operating bands - 5.7.3: Updated carrier frequency and EARFCN - 6.6.2.0 : Updated the section with ACLR requirement for the new bands. - 6.6.3.4 : Updated the section with new bands - 6.6.4.5 : Updated the section on transmitter spurious emissions including the new bands |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Bands are not available for early deployment of 5G broadcast. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5, 5.7.3, 6.6.2.0, 6.6.3.4, 6.6.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

## 5.5	Operating bands

E-UTRA is designed to operate in the operating bands defined in Table 5.5-1. Unless stated otherwise, requirements specified for the TDD duplex mode apply for downlink and uplink operations in Frame Structure Type 2 [4].

NB-IoT is designed to operate in the E-UTRA operating bands 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 25, 26, 28, 31, 41 (in certain regions), 42, 43, 48, 54, 65, 66, 70, 71, 72, 73, 74, 85, 87, 88, 103, 106 which are defined in Table 5.5-1.

LTE based 5G terrestrial broadcast is designed to operate in the E-UTRA operating bands 107, 108, 112 and 113.

E-UTRA operating band 1, 2, 3, 5, 7, 8, 20, 25, 26, 28, 34, 38, 39, 41, 67 and 85 which are defined in Table 5.5-1, can be applied for HAPS operation.

Table 5.5-1 E-UTRA frequency bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz |  | FDD |  |
| 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz |  | FDD |  |
| 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  | FDD |  |
| 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz |  | FDD |  |
| 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894MHz |  | FDD |  |
| 6 (NOTE 1) | 830 MHz | – | 840 MHz | 875 MHz | – | 885 MHz |  | FDD |  |
| 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  | FDD |  |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  | FDD |  |
| 9 | 1749.9 MHz | – | 1784.9 MHz | 1844.9 MHz | – | 1879.9 MHz |  | FDD |  |
| 10 | 1710 MHz | – | 1770 MHz | 2110 MHz | – | 2170 MHz |  | FDD |  |
| 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz |  | FDD |  |
| 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz |  | FDD |  |
| 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz |  | FDD |  |
| 14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz |  | FDD |  |
| 15 | Reserved |  |  | Reserved |  |  |  | FDD |  |
| 16 | Reserved |  |  | Reserved |  |  |  | FDD |  |
| 17 | 704 MHz | – | 716 MHz | 734 MHz | – | 746 MHz |  | FDD |  |
| 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz |  | FDD |  |
| 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz |  | FDD |  |
| 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz |  | FDD |  |
| 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  | FDD |  |
| 22 | 3410 MHz | – | 3490 MHz | 3510 MHz | – | 3590 MHz |  | FDD |  |
| 231 | 2000 MHz | – | 2020 MHz | 2180 MHz | – | 2200 MHz |  | FDD |  |
| 249 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz |  | FDD |  |
| 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz |  | FDD |  |
| 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz |  | FDD |  |
| 27 | 807 MHz | – | 824 MHz | 852 MHz | – | 869 MHz |  | FDD |  |
| 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  | FDD |  |
| 29 | N/A |  |  | 717 MHz | – | 728 MHz |  | FDD (NOTE 2) |  |
| 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz |  | FDD |  |
| 31 | 452.5 MHz | – | 457.5 MHz | 462.5 MHz | – | 467.5 MHz |  | FDD |  |
| 32 | N/A |  |  | 1452 MHz | – | 1496 MHz |  | FDD (NOTE 2) |  |
| 33 | 1900 MHz | – | 1920 MHz | 1900 MHz | – | 1920 MHz |  | TDD |  |
| 34 | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz |  | TDD |  |
| 35 | 1850 MHz | – | 1910 MHz | 1850 MHz | – | 1910 MHz |  | TDD |  |
| 36 | 1930 MHz | – | 1990 MHz | 1930 MHz | – | 1990 MHz |  | TDD |  |
| 37 | 1910 MHz | – | 1930 MHz | 1910 MHz | – | 1930 MHz |  | TDD |  |
| 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz |  | TDD |  |
| 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz |  | TDD |  |
| 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz |  | TDD |  |
| 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz |  | TDD |  |
| 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz |  | TDD |  |
| 43 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz |  | TDD |  |
| 44 | 703 MHz | – | 803 MHz | 703 MHz | – | 803 MHz |  | TDD |  |
| 45 | 1447 MHz | – | 1467 MHz | 1447 MHz | – | 1467 MHz |  | TDD |  |
| 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz |  | TDD (NOTE 3, NOTE 4) |  |
| 47 | 5855 MHz | – | 5925 MHz | 5855 MHz | – | 5925 MHz |  | TDD |  |
| 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz |  | TDD |  |
| 49 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz |  | TDD (NOTE 8) |  |
| 50 | 1432 MHz | - | 1517 MHz | 1432 MHz | - | 1517 MHz |  | TDD |  |
| 51 | 1427 MHz | - | 1432 MHz | 1427 MHz | - | 1432 MHz |  | TDD |  |
| 52 | 3300 MHz | - | 3400 MHz | 3300 MHz | - | 3400 MHz |  | TDD |  |
| 53 | 2483.5 MHz | - | 2495 MHz | 2483.5 MHz | - | 2495 MHz |  | TDD |  |
| 54 | 1670 MHz | - | 1675 MHz | 1670 MHz | - | 1675 MHz |  | TDD |  |
| 65 | 1920 MHz | – | 2010 MHz | 2110 MHz | – | 2200 MHz |  | FDD |  |
| 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz |  | FDD (NOTE 5) |  |
| 67 | N/A |  |  | 738 MHz | – | 758 MHz |  | FDD (NOTE 2) |  |
| 68 | 698 MHz | – | 728 MHz | 753 MHz | – | 783 MHz |  | FDD |  |
| 69 | N/A |  |  | 2570 MHz | – | 2620 MHz |  | FDD (NOTE 2) |  |
| 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz |  | FDD6 |  |
| 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz |  | FDD |  |
| 72 | 451 MHz | – | 456 MHz | 461 MHz | – | 466 MHz |  | FDD |  |
| 73 | 450 MHz | – | 455 MHz | 460 MHz | – | 465 MHz |  | FDD |  |
| 74 | 1427 MHz | – | 1470 MHz | 1475 MHz | – | 1518 MHz |  | FDD |  |
| 75 | N/A |  |  | 1432 MHz | – | 1517 MHz |  | FDD (NOTE 2) |  |
| 76 | N/A |  |  | 1427 MHz | – | 1432 MHz |  | FDD (NOTE 2) |  |
| 85 | 698 MHz – 716 MHz |  |  | 728 MHz | – | 746 MHz |  | FDD |  |
| 87 | 410 MHz – 415 MHz |  |  | 420 MHz | – | 425 MHz |  | FDD |  |
| 88 | 412 MHz – 417 MHz |  |  | 422 MHz | – | 427 MHz |  | FDD |  |
| 10310 | 787 MHz – 788 MHz |  |  | 757 MHz | – | 758 MHz |  | FDD |  |
| 106 | 896 MHz – 901 MHz |  |  | 935 MHz | – | 940 MHz |  | FDD |  |
| 107 | N/A |  |  | 612 MHz | – | 652 MHz |  | SDO (NOTE 11) |  |
| 108 | N/A |  |  | 470 MHz | – | 698 MHz |  | SDO (NOTE 11) |  |
| 111 | 1800 MHz – 1810 MHz |  |  | 1820 MHz –1830 MHz |  |  |  | FDD |  |
| 112 | N/A |  |  | 470 MHz | – | 608 MHz |  | SDO (NOTE 11) |  |
| 113 | N/A |  |  | 606 MHz | – | 698 MHz |  | SDO (NOTE 11) |  |
| NOTE 1: Band 6, 23 are not applicable. NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: This band is an unlicensed band restricted to licensed-assisted operation using Frame Structure Type 3. NOTE 4: Band 46 is divided into four sub-bands as in Table 5.5-1A. NOTE 5: The range 2180 – 2200 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured. NOTE 6: The range 2010-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 300 MHz. The range 2005-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 295 MHz. NOTE 7: Void NOTE 8: This band is restricted to licensed-assisted operation using Frame Structure Type 3. NOTE 9: DL operation is restricted to 1526-1536 MHz frequency range. UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48. NOTE 10: This band is restricted to NB-IoT standalone operation. NOTE 11: This band is restricted to LTE based 5G terrestrial broadcast operation. |  |  |  |  |  |  |  |  |

Table 5.5-1A Sub-bands for Band 46

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |
| 46a | 5150 MHz | – | 5250 MHz | 5150 MHz | – | 5250 MHz |
| 46b | 5250 MHz | – | 5350 MHz | 5250 MHz | – | 5350 MHz |
| 46c | 5470 MHz | – | 5725 MHz | 5470 MHz | – | 5725 MHz |
| 46d | 5725 MHz | – | 5925 MHz | 5725 MHz | – | 5925 MHz |

Table 5.5-2: Void

Table 5.5-3: Void

Table 5.5-3A: Void

Table 5.5-3B: Void

Table 5.5-3C: Void

Table 5.5-4: Void

Table 5.5-5: Void

Table 5.5-6: Void

------------------------------ Modified section ------------------------------

### 5.7.3	Carrier frequency and EARFCN

The carrier frequency in the uplink and downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 - 262143. The relation between EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL_low and NOffs-DL are given in table 5.7.3-1 and NDL is the downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL)

The relation between EARFCN and the carrier frequency in MHz for the uplink is given by the following equation where FUL_low and NOffs-UL are given in table 5.7.3-1 and NUL is the uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL)

The carrier frequency of NB-IoT in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 – 262143 and the Offset of NB-IoT Channel Number to EARFCN in the range {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,-0.5,0,1,2,3,4,5,6,7,8,9} for FDD and in the range {-10,-9,-8.5,-8,-7,-6,-5,-4.5,-4,-3,-2,-1,-0.5,0,1,2,3,3.5,4,5,6,7,7.5,8,9} for TDD. The relation between EARFCN, Offset of NB-IoT Channel Number to EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL is the downlink carrier frequency of NB-IoT, FDL_low and NOffs-DL are given in table 5.7.3-1, NDL is the downlink EARFCN, MDL is the Offset of NB-IoT Channel Number to downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1)

The carrier frequency of NB-IoT in the uplink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 –262143, and the Offset of NB-IoT Channel Number to EARFCN in the range {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,0,1,2,3,4,5,6,7,8,9} for FDD and in the range {-11,-10,-9.5,-9,-8.5,-8,-7.5,-7,-6.5,-6,-5.5,-5,  -4.5,-4,-3.5,-3,-2.5,-2,-1.5,-1,-0.5,0,0.5,1,1.5,2,2.5,3,3.5,4,4.5,5,5.5,6,6.5,7,7.5,8,8.5,9,9.5,10, 11} for TDD. The relation between EARFCN, Offset of NB-IoT Channel Number to EARFCN and the carrier frequency in MHz for the uplink is given by the following equation, where FUL is the uplink carrier frequency of NB-IoT, FUL_low and NOffs-UL are given in table 5.7.3-1, NUL is the uplink EARFCN, MUL is the Offset of NB-IoT Channel Number to uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL)

NOTE 1:	For NB-IoT, NDL or NUL is different than the value of EARFCN that corresponds to E-UTRA downlink or uplink carrier frequency for in-band and guard band operation.

NOTE 2:	For FDD MDL = -0.5 is not applicable for in-band and guard band operation. For TDD MDL {-0.5,+3.5,-4.5,+7.5,-8.5} is not applicable for in-band and guard band operation.

NOTE 3:	For the carrier including NPSS/NSSS for in-band and guard band operation, MDL is selected from {-2,-1,0,1}.

NOTE 4:	For the carrier including NPSS/NSSS for stand-alone operation, MDL = -0.5.

Table 5.7.3-1: E-UTRA channel numbers

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low [MHz] | NOffs-DL | Range of NDL | FUL_low [MHz] |  | NOffs-UL | Range of NUL |
| 1 | 2110 | 0 | 0 – 599 | 1920 |  | 18000 | 18000 – 18599 |
| 2 | 1930 | 600 | 6001199 | 1850 |  | 18600 | 18600 – 19199 |
| 3 | 1805 | 1200 | 1200 – 1949 | 1710 |  | 19200 | 19200 – 19949 |
| 4 | 2110 | 1950 | 1950 – 2399 | 1710 |  | 19950 | 19950 – 20399 |
| 5 | 869 | 2400 | 2400 – 2649 | 824 |  | 20400 | 20400 – 20649 |
| 6 | 875 | 2650 | 2650 – 2749 | 830 |  | 20650 | 20650 – 20749 |
| 7 | 2620 | 2750 | 2750 – 3449 | 2500 |  | 20750 | 20750 – 21449 |
| 8 | 925 | 3450 | 3450 – 3799 | 880 |  | 21450 | 21450 – 21799 |
| 9 | 1844.9 | 3800 | 3800 – 4149 | 1749.9 |  | 21800 | 21800 – 22149 |
| 10 | 2110 | 4150 | 4150 – 4749 | 1710 |  | 22150 | 22150 – 22749 |
| 11 | 1475.9 | 4750 | 4750 – 4949 | 1427.9 |  | 22750 | 22750 – 22949 |
| 12 | 729 | 5010 | 5010 – 5179 | 699 |  | 23010 | 23010 – 23179 |
| 13 | 746 | 5180 | 5180 – 5279 | 777 |  | 23180 | 23180 – 23279 |
| 14 | 758 | 5280 | 5280 – 5379 | 788 |  | 23280 | 23280 – 23379 |
| … |  |  |  |  |  |  |  |
| 17 | 734 | 5730 | 5730 – 5849 | 704 |  | 23730 | 23730 – 23849 |
| 18 | 860 | 5850 | 5850 – 5999 | 815 |  | 23850 | 23850 – 23999 |
| 19 | 875 | 6000 | 6000 – 6149 | 830 |  | 24000 | 24000 – 24149 |
| 20 | 791 | 6150 | 6150 - 6449 | 832 |  | 24150 | 24150 - 24449 |
| 21 | 1495.9 | 6450 | 6450 – 6599 | 1447.9 |  | 24450 | 24450 – 24599 |
| 22 | 3510 | 6600 | 6600 - 7399 | 3410 |  | 24600 | 24600 - 25399 |
| 23 | 2180 | 7500 | 7500 – 7699 | 2000 |  | 25500 | 25500 – 25699 |
| 24 | 1525 | 7700 | 7700 – 8039 | 1626.5 |  | 25700 | 25700 – 26039 |
| 25 | 1930 | 8040 | 8040 - 8689 | 1850 |  | 26040 | 26040 - 26689 |
| 26 | 859 | 8690 | 8690 – 9039 | 814 |  | 26690 | 26690 - 27039 |
| 27 | 852 | 9040 | 9040 – 9209 | 807 |  | 27040 | 27040 – 27209 |
| 28 | 758 | 9210 | 9210 – 9659 | 703 |  | 27210 | 27210 – 27659 |
| 29 (NOTE 2) | 717 | 9660 | 9660 – 9769 | N/A |  |  |  |
| 30 | 2350 | 9770 | 9770 – 9869 | 2305 |  | 27660 | 27660 – 27759 |
| 31 | 462.5 | 9870 | 9870 – 9919 | 452.5 |  | 27760 | 27760 – 27809 |
| 32 (NOTE 2) | 1452 | 9920 | 9920 – 10359 | N/A |  |  |  |
| 33 | 1900 | 36000 | 36000 – 36199 | 1900 |  | 36000 | 36000 – 36199 |
| 34 | 2010 | 36200 | 36200 – 36349 | 2010 |  | 36200 | 36200 – 36349 |
| 35 | 1850 | 36350 | 36350 – 36949 | 1850 |  | 36350 | 36350 – 36949 |
| 36 | 1930 | 36950 | 36950 – 37549 | 1930 |  | 36950 | 36950 – 37549 |
| 37 | 1910 | 37550 | 37550 – 37749 | 1910 |  | 37550 | 37550 – 37749 |
| 38 | 2570 | 37750 | 37750 – 38249 | 2570 |  | 37750 | 37750 – 38249 |
| 39 | 1880 | 38250 | 38250 – 38649 | 1880 |  | 38250 | 38250 – 38649 |
| 40 | 2300 | 38650 | 38650 – 39649 | 2300 |  | 38650 | 38650 – 39649 |
| 41 | 2496 | 39650 | 39650 – 41589 | 2496 |  | 39650 | 39650 – 41589 |
| 42 | 3400 | 41590 | 41590 – 43589 | 3400 |  | 41590 | 41590 – 43589 |
| 43 | 3600 | 43590 | 43590 – 45589 | 3600 |  | 43590 | 43590 – 45589 |
| 44 | 703 | 45590 | 45590 – 46589 | 703 |  | 45590 | 45590 – 46589 |
| 45 | 1447 | 46590 | 46590 – 46789 | 1447 |  | 46590 | 46590 – 46789 |
| 46 (NOTE 3) | 5150 | 46790 | 46790 – 54539 | 5150 |  | 46790 | 46790 – 54539 |
| 47 | 5855 | 54540 | 54540 – 55239 | 5855 |  | 54540 | 54540 – 55239 |
| 48 | 3550 | 55240 | 55240 – 56739 | 3550 |  | 55240 | 55240 – 56739 |
| 49 | 3550 | 56740 | 56740 – 58239 | 3550 |  | 56740 | 56740 – 58239 |
| 50 | 1432 | 58240 | 58240 - 59089 | 1432 |  | 58240 | 58240 - 59089 |
| 51 | 1427 | 59090 | 59090 - 59139 | 1427 |  | 59090 | 59090 - 59139 |
| 52 | 3300 | 59140 | 59140 - 60139 | 3300 |  | 59140 | 59140 - 60139 |
| 53 | 2483.5 | 60140 | 60140 - 60254 | 2483.5 |  | 60140 | 60140 - 60254 |
| 54 | 1670 | 60255 | 60255 - 60304 | 1670 |  | 60255 | 60255 - 60304 |
| 65 | 2110 | 65536 | 65536 – 66435 | 1920 |  | 131072 | 131072 – 131971 |
| 66 (NOTE 4) | 2110 | 66436 | 66436 – 67335 | 1710 |  | 131972 | 131972 – 132671 |
| 67 (NOTE 2) | 738 | 67336 | 67336 - 67535 | N/A |  |  |  |
| 68 | 753 | 67536 | 67536 - 67835 | 698 |  | 132672 | 132672 - 132971 |
| 69 (NOTE 2) | 2570 | 67836 | 67836 - 68335 | N/A |  |  |  |
| 70 (NOTE 5) | 1995 | 68336 | 68336 - 68585 | 1695 |  | 132972 | 132972 - 133121 |
| 71 | 617 | 68586 | 68586 - 68935 | 663 |  | 133122 | 133122-133471 |
| 72 | 461 | 68936 | 68936 - 68985 | 451 |  | 133472 | 133472-133521 |
| 73 | 460 | 68986 | 68986 - 69035 | 450 |  | 133522 | 133522-133571 |
| 74 | 1475 | 69036 | 69036 - 69465 | 1427 |  | 133572 | 133572 - 134001 |
| 75 (NOTE 2) | 1432 | 69466 | 69466 - 70315 | N/A |  |  |  |
| 76 (NOTE 2) | 1427 | 70316 | 70316 - 70365 | N/A |  |  |  |
| 85 | 728 | 70366 | 70366 - 70545 | 698 |  | 134002 | 134002 - 134181 |
| 87 | 420 | 70546 | 70546 - 70595 | 410 |  | 134182 | 134182 - 134231 |
| 88 | 422 | 70596 | 70596 - 70645 | 412 |  | 134232 | 134232 - 134281 |
| 103 | 757 | 70646 | 70646 - 70655 | 787 |  | 134282 | 134282 - 134291 |
| 106 (NOTE 6) | 935 | 70656 | 70656 - 70705 | 896 |  | 134292 | 134292 - 134341 |
| 107 (NOTE 7) | 612 | 70706 | 70706 – 71105 | N/A |  |  |  |
| 108 (NOTE 7) | 470 | 71106 | 71106 – 73385 | N/A |  |  |  |
| 111 | 1820 | 73386 | 73386 – 73485 | 1800 | 134342 |  | 134342 – 134441 |
| 112 (NOTE 7) | 470 | 71106 | 71106 - 72485 | N/A |  |  |  |
| 113 (NOTE 7) | 606 | 72466 | 72466 - 73385 | N/A |  |  |  |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7, 15, 25, 50, 75 and 100 channel numbers at the lower operating band edge and the last 6, 14, 24, 49, 74 and 99 channel numbers at the upper operating band edge shall not be used for channel bandwidths of 1.4, 3, 5, 10, 15 and 20 MHz respectively. NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. NOTE 3: The following NDL and NUL are allowed for operation in Band 46 assuming 20MHz channel bandwidth: NDL =NUL = {n-2, n-1, n, n+1, n+2 ¦ n = 46890 (5160 MHz), 47090 (5180 MHz), 47290 (5200 MHz), 47490 (5220 MHz), 47690 (5240 MHz), 47890 (5260 MHz), 48090 (5280 MHz), 48290 (5300 MHz), 48490 (5320 MHz), 48690 (5340 MHz), 50090 (5480 MHz), 50290 (5500 MHz), 50490 (5520 MHz), 50690 (5540 MHz), 50890 (5560 MHz), 51090 (5580 MHz), 51290 (5600 MHz), 51490 (5620 MHz), 51690 (5640 MHz), 51890 (5660 MHz), 52090 (5680 MHz), 52290 (5700 MHz), 52490 (5720 MHz), 52740 (5745 MHz), 52940 (5765 MHz), 53140 (5785 MHz), 53340 (5805 MHz), 53540 (5825 MHz), 53740 (5845 MHz), 53940 (5865 MHz), 54140 (5885 MHz), 54340 (5905 MHz)}. And the following NDL and NUL are allowed for operation in Band 46 assuming 10MHz channel bandwidth: NDL =NUL = {n-2, n-1, n, n+1, n+2 ¦ n = 52590 (5730 MHz), 53590 (5830 MHz)}. 10 MHz channel bandwidth shall only apply in certain regions where the absence of non 3GPP technologies can be guaranteed on a long term basis in this version of specification. NOTE 4: Downlink frequency range 2180 – 2200 MHz is restricted to E-UTRA operation when carrier aggregation is configured. NOTE 5: The range 2010-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 300 MHz. The range 2005-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 295 MHz. NOTE 6: In the present version of the specification, only EARFCN 70686 and 134322 is applicable for 3 MHz channel bandwidth. NOTE 7: This band is restricted to LTE based 5G terrestrial broadcast operation. The channel numbers that designate carrier frequencies so close to the operating band edges that the broadcast carrier with PMCH bandwidth extends beyond the operating band edge shall not be used. |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

### 6.6.2	Adjacent Channel Leakage power Ratio (ACLR)

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

The requirements shall apply outside the Base Station RF Bandwidth or Radio Bandwidth whatever the type of transmitter considered (single carrier or multi-carrier) and for all transmission modes foreseen by the manufacturer's specification.

For a E-UTRA or  E-UTRA with NB-IoT (in-band and/or guard band) BS operating in non-contiguous spectrum, the ACLR also applies for the first adjacent channel inside any sub-block gap with a gap size Wgap ≥ 15MHz or Wgap ≥ 60MHz for Band 46. The ACLR requirement for the second adjacent channel applies inside any sub-block gap with a gap size Wgap ≥ 20 MHz or Wgap ≥ 80MHz for Band 46. The CACLR requirement in clause 6.6.2.2 applies in sub block gaps for the frequency ranges defined in Table 6.6.2.2-1/2/2a.

For a E-UTRA or  E-UTRA with NB-IoT (in-band and/or guard band) BS operating in multiple bands, where multiple bands are mapped onto the same antenna connector, the ACLR also applies for the first adjacent channel inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 15MHz. The ACLR requirement for the second adjacent channel applies inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 20 MHz. The CACLR requirement in clause 6.6.2.2 applies in Inter RF Bandwidth gaps for the frequency ranges defined in Table 6.6.2.2-1/2.

The requirement applies during the transmitter ON period.

#### 6.6.2.0	Additional requirements for LTE based 5G terrestrial broadcast

For LTE based 5G terrestrial broadcast base stations operating in band 107, 108, 112, 113, the ACLR requirement is according to regional regulations, but shall be no less than 45 dB. Examples are captured in Annex A.2.2 in TR 36.792 [30].

------------------------------ Modified section ------------------------------

#### 6.6.3.4	Additional requirements for LTE based 5G terrestrial broadcast

For LTE based 5G terrestrial broadcast base stations operating in band 107, 108, 112, 113, the operating band unwanted emission requirements are according to regional regulations as captured in clause 4.3 to 4.6 and Annex A.2.2 of TR  36.792[30].

------------------------------ Modified section ------------------------------

#### 6.6.4.5	Additional transmitter spurious emissions requirements for LTE based 5G terrestrial broadcast

For LTE based 5G terrestrial broadcast base stations operating in band 107, 108, 112, 113, the transmitter spurious emission requirements are according to regional regulations, as captured in Annex A.2.3 TR 36.792 [30].

------------------------------ End of modified section -------------------------