3GPP TSG-RAN WG4 Meeting #115            	R4-2505972

Malta, MT, 19th-23th, May, 2024

Agenda item:	7.8.6

Source:	Samsung

Title:	View on BS demodulation requirement for 7MHz

Document for:	Discussion

- Introduction

In the RAN#10 meeting, the New WI was endorsed to support NR FR1 7MHz channel bandwidth with the following objectives in WID [1]

- Objective of Core part - Specify the generic core requirements to support 7 MHz NR FR1 channel bandwidth [RAN4] - Introduce the channel bandwidth 7MHz - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements for UL and DL - Specify the band specific core requirements to introduce 7 MHz for NR band n5, n26. [RAN4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed: - MPR (relative bandwidth criteria) - Additional Maximum Power Reduction (A-MPR) - Enable, if feasible, optional UE support of 7 MHz channel bandwidth from Rel-15 onwards in a release independent manner. [RAN2, RAN4] - No impact to 38.307 is expected. - Once RAN4 has introduced the new channel bandwidth, RAN2 will introduce the necessary signalling for this channel bandwidth. [RAN2] - Note: This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN2 will be informed by RAN4 via LS. No additional RAN2 TUs will be requested for this objective. - Once RAN4 has introduced the new channel bandwidth, RAN3 will introduce the necessary signalling for this channel bandwidth [RAN3] - Note: This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN3 will be informed by RAN4 via LS. No additional RAN3 TUs will be requested for this objective. - Objective of Performance part - Specify necessary UE/BS performance requirements for NR operation in dedicated FDD FR1 spectrum allocations for irregular channel bandwidths, corresponding to the core requirements: - Specify necessary RRM performance requirements if needed (RAN4) - Specify, if necessary, UE demodulation performance and CSI reporting requirements for the 7 MHz channel bandwidth (RAN4) - Specify, if necessary, BS demodulation performance requirements (RAN4) - Specify necessary BS conformance tests (RAN4)

According to meeting agenda, this is the 1st meeting to discussion the performance part of NR FR1 7MHz for Rel-19. In this contribution, the view on the test scope of demodulation requirement for NR FR1 7MHz is provided.

# 2	Test Scope

For FR1 NR, RAN4 has introduced BS performance requirements from Rel-15 with different channel bandwidth combination considering the test coverage.

For 7MHz, based on core requirement, the available channel bandwidth is 35PRB for band n5, and band n26. And the following was agreed in RF discussion.

Agreement in RAN4#114 - Scope - The 7 MHz channel bandwidth is only applicable to FDD bands that also support 5 MHz channel bandwidth - In-band NB-IoT can be supported in 7MHz channel BW - System parameters- Release independence - 35RBs will be supported - Support of 7MHz channel BW could be release independent from Rel-15 - FFS on if the maximum initial BWP size shall then be restricted to 5MHz - Support of 7MHz is optional in Rel-19 and in previous release - BS RF - Conducted requirements shall be covered within this WI - Multi-carrier (including MSR) and CA related requirements are considered for new added 7MHz WID, but no new BCS is specified within this WI - 7MHz should not be supported by IAB - 7MHz should not be supported by NCR

Based on RF requirement, the 7MHz channel bandwidth is only applicable to FDD bands that also support 5MHz channel bandwidth. That means that there is no BSs that supporting only 7MHz channel bandwidth.

Observation 1: 7MHz channel bandwidth is optional in Rel-19. 7MHz channel bandwidth is only applicable to FDD bands that also support 5MHz channel bandwidth.

In Rel-18, NR_FR1_less than_5MHz, RAN4 has also lots of discussion whether to define the dedicated requirements for 3MHz for PUSCH, PUCCH and PRACH. It is RAN4 agreement that there are no BS that supporting only less than 5MHz. Meanwhile, based on existing rule for different CBWs, 3MHz requirements will never be tested. Therefore, RAN4 agreed that not to define PUSCH requirements with 3MHz bandwidth for both non-HST and HST condition, as well as UL timing adjustment requirement.

Similar as 3MHz, there is no BS only support 7MHz channel bandwidth. Since RAN4 has introduce PUSCH requirement with 5MHz. Base on the existing test applicability rule, the tests shall be done by using performance requirement for the closet channel bandwidth lower than this widest supported bandwidth, the tested PRB shall then be centered in this widest supported channel bandwidth.

Therefore, we think there is no need to introduce dedicated PUSCH requirement with 7MHz

Proposal 1: No new PUSCH requirement introduced for 7MHz

For PUCCH, during Rel-18 less than 5MHz discussion, based on companies’ evaluation, there is no obvious performance different observed in PUCCH formats 0/1/3/4 compared with existing 5MHz requirement.  As for format2, due to limit number RBs for frequency hopping, the performance is different with existing 5MHz requirement. Therefore, only introduce the new demodulation requirement for PUCCH format 2 with 3MHz CBW. Meanwhile, to reduce the test effort, a test applicability rule for new introduced UCI performance with 3MHz and legacy 5MHz was introduced.

Different with 3MHz, due to the available RBs for frequency hopping is more than 3MHz, we think there should be no different for format 2 compared with 5MHz or 10MHz. Therefore, we propose there is no new PUCCH demodulation requirement for PUCCH.

Proposal 2: No new PUCCH requirement for 7MHz, existing PUCCH requirements with 5MHz can also apply for 7MHz

For PRACH, its requirement is no differentiate with transmission BWs if its configurations can be fitted into the transmission BW, the minimum number of RB as 35 for 7MHz, where the available number of subcarriers is 420. Then, the existing long PRACH formats with LRA=839 and short PRACH formats with LRA=139 can be fitted into the transmission BW of 7MHz. Therefore, it was agreed that no new PRACH requirement for NR with 7MHz. Similar as less than 5MHz, we think there is no new PRACH requirement for 7MHz, existing PRACH requirements with 5MHz can also apply for 7MHz

Proposal 3: No new PRACH requirement for 7MHz, existing PRACH requirements can also apply for 7MHz

# 3	Conclusion

In this contribution, the view on the BS demodulation requirement with 7MHz is provided.

Observation 1: 7MHz channel bandwidth is optional in Rel-19. 7MHz channel bandwidth is only applicable to FDD bands that also support 5MHz channel bandwidth.

Proposal 1: No new PUSCH requirement introduced for 7MHz

Proposal 2: No new PUCCH requirement for 7MHz, existing PUCCH requirements with 5MHz can also apply for 7MHz

Proposal 3: No new PRACH requirement for 7MHz, existing PRACH requirements can also apply for 7MHz

# References

RP-250373, Revised WID on NR FR1 7MHz channel bandwidth, China Telecom, T-Mobile USA