3GPP TSG-RAN WG4 Meeting #115           	R4-2505978

Malta, MT, 19th-23th, May, 2025

Agenda item:	7.31.7

Source:	Samsung

Title:	View on demodulation requirement for IoT NTN

Document for:	Discussion

- Introduction

In the RAN#107 meeting, the revised WI was endorsed to enhance the NTN for IoT with the following objectives， which can be categorized as following

- Objective 1: Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload

- Objective 2: Support of Capacity enhancements for uplink

- Objective 3: Support broadcase of PWS messages of NB-IoT re-using the LTE mechanisms

As stated in the WID,  RAN4 needs to specify the necessary performance requireemnts for enhancements listed above. According to meeting agenda, this is the 1st meeting to discuss the performance part of IoT NTN phase 3. In this contribution, the overall view on the test scope of demodulation requirement for IoT NTN phase 3 is provided. And the initial simulation assumption is provided for specifying requirement.

# 2	Test Scope

2.1	Objective 1: Support of Store&Forward (S&F) satellite operation

The detailed objective 1 as following

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] - Note: strive to minimize UE impact - Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

S&F operation allows for delay-tolerant, non-real-time IoT NTN services to be offered in areas visited by the satellites but with no need to have NTN gateway infrastructure (e.g.mid-sea, remote areas). And long with support for discontinuous coverage, support for S&F operation is key to facilitate cost-effective deployment of IoT NTN services and enable an immediate operational service with (1) sparse LEO constellations and (2) reduced ground segment infrastructure. Based on the objective of WID, there is no RAN4 impact. Therefore, there is no performance impact foreseen from BS side.

Proposal 1: There is no performance impact for the objective 1: “Support of Store&Forward (S&F) satellite operation”

2.2	Objective 2: Support of Capacity enhancement for uplink

The detailed objective 2 as following

- Support of Capacity enhancements for uplink - Study then specifies, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 [RAN1, RAN2, RAN4] - Multi-tone support for 15KHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed - Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2] - Msg3 transmission without msg1/Random Access Reponses (RAR) - Efficient delivery (reduced overhead) of

NB-IoT NTN is already being deployed live at this very moment. In these early and upcoming deployments, it is clearly emerging that IoT-NTN, in particular NB-IoT, will have to support massive capacity in terms of number and types of UE, some of which with worse characteristics than others (e.g., low-cost devices, wearables, etc). Multiplexing of UEs by usage of orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH should therefore be studied and if beneficial being specified. Based on RAN1 design, slot-level OCC is supported for NPUSCH format1 with 15KHz single tone, and symbol-level OCC is supported for NPUSCH format1 with 3.75KHz SCS single tone. The OCC spread process will be added. It is straightforward to introduce NPUSCH demodulation performance to support OCC processing

And based on RAN1 progress, RAN1 did not reach consensus on whether OCC for NPRACH is beneficial or not. RAN1 will not specify support for OCC for NPRACH in Rel-19 IoT NTN. So, there is no PRACH requirement introduced to support OCC processing

Proposal 2: For Uplink Capacity enhancement, introduce NPUSCH demodulation performance to support OCC processing

Proposal 3: For Uplink Capacity enhancement, no NPRACH demodulation performance introduced to support OCC processing

2.3	Objective 3: Support broadcast of PWS messages

The detailed objective 3 as following

- Support broadcast of PWS messages for NB-IoT Re-using the LTE mechanisms [RAN2] - Note: The solutions will be de developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. - Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome - Note: PWS notification in paging, direct indication, and system information in RRC connected modem is not supported

Satellite is by nature a broadcast medium, that can quickly reach very large coverage areas even when terrestrial networks are down. Delivery of same data to a very large UE population in the same area via Unicast is not scalable. Therefore, supporting emergency broadcast messaging is beneficial and should be supported. Based on WID, there is no RAN4 impact. Therefore, there is no performance impact foreseen for both BS side and UE side.

Proposal 4: There is no performance impact for objective 3: “Support broadcast of PWS message”

# 3	Test Setup

In this section, the initial test setup for SAN demodulation requirement related with Uplink Capacity enhancement.

The following is the RAN1 agreements related with OCC processing for NPUSCH, including OCC length, number of tones, and DMRS processing

RAN1#118bis agreement - At least the following schemes are supported for single-tone - For 15Khz SCS OCC for NPUSCH format 1 - OCC length 2, Slot-level, CDM DMRS with legacy pattern - FFS: CDM details, e.g., with or without spreading RAN1#118bis agreement - At least the following schemes are supported for single-tone - For 3.75KHz SCS, OCC for NPUSCH format 1 - OCC length 2, Slot-level, CDM DMRS with legacy pattern - FFS: CDM details, e.g., with or without spreading RAN1#119 agreement - For NPUSCH Format 1 single-tone 15kHz SCS, the slot-level scheme for non-DMRS symbols is that spreading is performed in the unit of one slot. RAN1#120 agreement - DMRS symbols are spread before the OCC is applied - Option 1: according to the formula ru,OCCMn+m=runqm, 0≤n<X/M, m=0, …, M-1 Where M is the OCC length, q is the assigned OCC codeword for the UE, run is the reference signal sequence defined in TS36.211 section 10.1.4.1.1 and X is the total number of slots in the NPUSCH transmission after OCC is applied 120bis agreement - For the support of OCC length 2 for NPUSCH format 1 single-tone with 3.75KHz SCS and 15KHz SCS, the orthogonal sequences are [1, 1; 1 -1]

NPUSCH formats

Based on RAN1 design, slot-level OCC is supported for NPUSCH format1 with 15KHz single tone, and symbol-level OCC is supported for NPUSCH format1 with 3.75KHz SCS single tone. So, both two SCS for NPUSCH1 should be

Proposal 5: Introduce NPUSCH1 requirements for both 15KHz and 3.75KHz SCS with OCC

Regarding the multitone, currently there is no sufficient progress in RAN1. Therefore, we suggest to prioritize the single tone for specifying NPUSCH requirement with OCC firstly, and further discuss whether to introduce the NPUSCH requirement with OCC for multi-tones.

Proposal 6: Prioritize the single tone for specifying NPUSCH requirement with OCC, FFS on multiple tones pending on RAN1 progress.

OCC length

Based on RAN1 design, the maximum OCC length is 2 for both 3.75kHz SCS OCC for NPUSCH format 1 and single tone 15kHz SCS Slot-level OCC for NPUSCH format 1. Therefore, only considering OCC length as 2 to specify the NPUSCH 1 with OCC

Proposal 7:  Only consider OCC length as 2 for specifying NUSCH format 1 requirement with OCC

DMRS pattern for OCC

Regarding the DMRS pattern for OCC, as for 15KHz SCS, it was agreed that DMRS symbols are spread before OCC is applied. according to the formula:

As for 3.75KHz, currently, RAN1 has made assumption as TDM pattern and an LS was sent to RAN4 asking whether there would be any issue (e.g., phase continuity) for supporting such TDM DMRS for IoT NTN. Therefore, pending on conclusion of DMRS for 3.75KHz, RAN4 will further discuss the simulation assumption on DMRS for specifying NPUSCH format 1 requirement

Proposal 8:  FFS on DMRS configuration for NPUSCH 3.75KH SCS, pending on RAN1 progress

Antenna configuration

In Rel-17 IoT NTN, both 1Rx and 2Rx are considered for specifying NPUSCH requirement with test applicability rule introduced. Therefore, both 1Rx and 2Rx will be considered for specifying PUSCH requirement with OCC.

Proposal 9: Consider both 1Rx and 2Rx for specifying PUSCH requirement with OCC

Number of repetitions

In legacy IoT test, the number of repetitions configured for NPUSCH requirement is 1, 16 and 64 for single tone with 15KHz and 3.75KHz SCS.  To reduce the test effort, only one repetition is considered. And in current IoT NTN testing, the maximum number of repetitions with 128 is specified for requirement. Considering the testing time and performance tradeoff, the repetition with 64 can be considered.

Proposal 10: Consider both 1Rx and 2Rx for specifying PUSCH requirement with OCC

MCS for two UEs

For MCS, although there is no limitation of MCS scheduling for two UEs, we prefer to configure the same MCS for two UEs. Currently, BPSK is used for NPUSCH1 requirement with single tone. The same MCS can be used.

Proposal 11: Consider the same MCS as BPSK configured for two UEs for specifying PUSCH requirement with OCC.

Channel Model for two UEs

For channel model, we prefer to configure the same channel model for two UEs. Currently, NTN TDLA100-1 is used for specifying the requirement of NPUSCH. The same channel can be used for NPUSCH requirement with OCC

Proposal 12: Consider the same channel model configured for two UEs for specify PUSCH requirement with OCC as NTN TDLA100-1.

Frequency offset Assumption

One of key factor to enable inter-slots OCC is that UE is required to maintain phase continuity and power consistency for the duration of one OCC group with NPUSCH. Based on RAN1 assumption, we prefer to configure the frequency offset is 0Hz for two UEs. While we are also open to further discuss the frequency offset assumption.

Proposal 13: Consider there is no frequency offset configured for two UEs as baseline assumption, FFS on frequency offset assumption.

Others

For other simulation, the existing test parameters for NPUSCH requirement can be considered as reference.

Proposal 14: Considering the existing test parameters for NPUSCH requirement as reference.

# 4	Conclusion

In this contribution, the view on test scope of demodulation requirement for IoT NTN is provided. And initial simulation assumption for requirement is provided

Proposal 1: There is no performance impact for the objective 1: “Support of Store&Forward (S&F) satellite operation”

Proposal 2: For Uplink Capacity enhancement, introduce NPUSCH demodulation performance to support OCC processing

Proposal 3: For Uplink Capacity enhancement, no NPRACH demodulation performance introduced to support OCC processing

Proposal 4: There is no performance impact for objective 3: “Support broadcast of PWS message”

Proposal 5: Introduce NPUSCH1 requirements for both 15KHz and 3.75KHz SCS with OCC

Proposal 6: Prioritize the single tone for specifying NPUSCH requirement with OCC, FFS on multiple tones pending on RAN1 progress.

Proposal 7:  Only consider OCC length as 2 for specifying NUSCH format 1 requirement with OCC

Proposal 8:  FFS on DMRS configuration for NPUSCH 3.75KH SCS, pending on RAN1 progress

Proposal 9: Consider both 1Rx and 2Rx for specifying PUSCH requirement with OCC

Proposal 10: Consider both 1Rx and 2Rx for specifying PUSCH requirement with OCC

Proposal 11: Consider the same MCS as BPSK configured for two UEs for specifying PUSCH requirement with OCC.

Proposal 12: Consider the same channel model configured for two UEs for specify PUSCH requirement with OCC as NTN TDLA100-1.

Proposal 13: Consider there is no frequency offset configured for two UEs as baseline assumption, FFS on frequency offset assumption.

Proposal 14: Considering the existing test parameters for NPUSCH requirement as reference.

# References

RP-250472, Revised WID on NTN for internet of Things (IoT) phase 3, MTK