3GPP RAN WG4 Meeting #115	R4-2505980

Saint Julian’s, Malta, May 19th – May 23rd, 2025

Title: 	Discussion on RRM core requirements testing framework for BM

Source: 	CATT

Agenda item:	7.21.4

Document for:	Discussion

Introduction

In RAN4#114bis meeting, RAN4 discussed beam management in Rel-19 AI/ML for NR air interface and agreements are captured in [1]. In this contribution, we will continue to discuss the open issues summarized in [2].

Discussion

Metrics/KPIs for beam ID prediction

In RAN4#114bis meeting, this issue was discussed and RAN4 reached the following agreement, which is a big step forward:

Issue 2-1: Metrics/KPIs for beam prediction Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

RAN4 agreed to use the successful rate as a metric/KPI for the correct prediction when only beam ID is reported. But there are still some aspects that need further discussion. The 1st FFS is to determine the value(s) of N, where each of Top-N predicted beams will be verified with the corresponding genie-aided beam. Considering that network needs to handle load-balancing and inter-layer interference, etc., network may not schedule UE with the strongest predicted beam even the AI/ML model has a very good performance, and therefore at least some of the other predicted beams shall be also verified. Hence, N needs to be larger than 1, i.e., 1<N</=K. But if N is equal to K, which means that each predicted beam will be verified and it may be too restricted for UE. As a compromise, N can equal to K2+1, where at least half of the predicted beams will be verified in the tests. This ensures that plural beams predicted by UE-sided AI/ML model have comparable RSRPs to that of the corresponding genie-aided beams with the same rank and provide multiple high-quality candidate beams for scheduling.

Proposal 1: RAN4 to verify at least half of predicted beams, i.e., N= K2+1.

Regarding the values of x, now multiple predicted beams may be verified based on our above discussion, and therefore we think the same x value can apply for each comparison between the same ranked predicted beam and genie-aided beam. The detailed x value can be further discussed based on simulation results.

Proposal 2: The same x value applies for each comparison between the same ranked predicted beam and genie-aided beam. Value of x is further discussed based on the simulation results.

RSRP accuracy

Discussion on RSRP accuracy is divided into absolute RSRP accuracy and relative RSRP accuracy. In last meeting, both aspects were discussed and reached some agreements:

Issue 2-2: Absolute RSRP accuracy Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams Issue 2-3: Relative RSRP accuracy Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

For absolute RSRP accuracy, it was agreed to verify the Top-1 predicted beam at least. For other predicted beam(s) with lower RSRP(s), the differential results will be reported. Hence there will be quantization loss during reporting. Besides, RAN4 also define the relative RSRP accuracy, which is more appropriate to be used to verify the rest of beams. Therefore we do not think the absolute RSRP accuracy requirement needs to apply to other beams.

Proposal 3: RAN4 not to apply absolute RSRP accuracy requirement to other beams, i.e., predicted beams with lower RSRPs.

For relative RSRP accuracy, firstly we think the square bracket in the above agreement can be removed since only the largest reported value will not be quantized and the results of other beams will have performance changes due to quantization procedure. Furthermore, in the test the Top-1 beam can be verified against the absolute RSRP accuracy requirement and the rest of predicted beams can be verified against the relative RSRP accuracy requirement, which can validate the quality of all reported results of predicted beams and meanwhile is able to avoid the impact of performance changes due to quantization procedure.

Proposal 4: RAN4 to remove the square bracket in the definition of relative RSRP accuracy, i.e.,

Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

Secondly, RAN4 need to determine whether reported L1-RSRP can be measured RSRP. In RAN1#120 meeting, RAN1 achieved an agreement on report content as follows [3]:

Agreement-RAN1#120: For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Note: how to capture it in the spec is a separate discussion.

Based on RAN1 agreement, the reported contents are predicted RSRPs which are based on AI/ML output, and during RAN1 discussion, the AI/ML output could be predicted RSRP or measured RSRP. Hence, the reported L1-RSRP can be measured RSRP.

Proposal 5: The reported L1-RSRP in the relative RSRP accuracy definition can be measured RSRP.

Another issue is whether the relative RSRP accuracy requirement applies for BM-Case 2. At first, RAN1 have the following agreement in RAN1#120 [3]:

Agreement-RAN1#120: For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances, - the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP. - The time instance information of the beam with the largest RSRP are additionally indicated in the report.

According to RAN1 design for BM-Case 2 reporting, UE can report results in one report over multiple future time instance. And the largest RSRP value is used as reference RSRP to derive the differential RSRPs of other time instances. This reporting logic is quite similar to that of BM-Case 1. Therefore, in our understanding, the relative RSRP accuracy requirement can apply for BM-Case 2.

Proposal 6: The relative RSRP accuracy requirement can apply for BM-Case 2.

Measurement period for inference

In previous meetings, RAN4 agreed that inference delay includes two parts and use the legacy L1-RSRP measurement delay requirement as baseline for BM-Case 1:

RAN4#113: Issue 3-5: Prediction delay requirements Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS RAN4#114bis: Issue 2-4: Measurement period for inference Agreement: - For measurement delay component of the prediction delay: - Use “legacy” L1-RSRP measurement as the baseline for Case 1 - FFS whether same values for M,N,P should be reused - FFS for case 2

We will discuss prediction delay for BM-Case 1 and BM-Case 2 respectively.

BM-Case 1:

For the measurement part, UE behaviour is exactly the same as legacy UE. Thus, we think the legacy measurement requirements can be reuse for this part. For inference delay, since UE does not need to measure reference signals and the inference is completely a UE internal procedure which can be carried out with other UE procedures in parallel or in some time gaps, e.g., SSB processing and interval between SSB occasions. Thus, the legacy L1-RSRP measurement requirement can be reused.

Observation 1: For BM-Case 1, UE measurement behaviour on SetB beams is the same as L1-RSRP measurement. The prediction for SetA beams in spatial domain is a short UE internal procedure. The legacy L1-RSRP measurement requirement can be reused for BM-Case 1.

BM-Case 2:

In RAN1#120 meeting, RAN1 made the following agreements for BM-Case 2:

Agreement For inference, for BM-Case 2 of UE-side model, - The time gap between two consecutive future time instances is configured by RRC, and the number of future time instance(s) N is configured by RRC. - time gap is [10ms, 20ms, 40ms, 80ms, 160ms] - N = [1, 2, 4, 8] - Reference time of the earliest time instance for the predicted results is based on the most recent occasion of the CSI-RS/SSB resource in Set B for measurement - Where the most recent occasion of the CSI-RS/SSB resource of set B is the latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource of the corresponding inference report.

According to RAN1 agreements, network will configure the time gap between two consecutive future time instances and the number of future time instance(s) N via RRC. Though these parameters are configurable, we do not think they have directly impacts on RRM core requirements. In BM-Case 2, the UE behaviour that takes time is measurement on SetB beams. After UE completes measurements on SetB beams, it can directly predict results for SetA beams over multiple future time instances, which is a short internally procedure and it can also be covered by time interval between reference signals.

Observation 2: For BM-Case 2, UE measurement behaviour on SetB beams is the same as L1-RSRP measurement. The prediction for SetA beams over multiple future time instances is one-shot UE internal procedure. The legacy L1-RSRP measurement requirement can also be reused for BM-Case 2.

Proposal 7: RAN4 to reuse legacy L1-RSRP measurement requirement for both BM-Case 1 and BM-Case 2.

Conclusions

This paper discussed of RRM core requirement issues related to AI/ML for beam management, and following proposals are provided:

Proposal 1: RAN4 to verify at least half of predicted beams, i.e., N= K2+1.

Proposal 2: The same x value applies for each comparison between the same ranked predicted beam and genie-aided beam. Value of x is further discussed based on the simulation results.

Proposal 3: RAN4 not to apply absolute RSRP accuracy requirement to other beams, i.e., predicted beams with lower RSRPs.

Proposal 4: RAN4 to remove the square bracket in the definition of relative RSRP accuracy, i.e.,

Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

Proposal 5: The reported L1-RSRP in the relative RSRP accuracy definition can be measured RSRP.

Proposal 6: The relative RSRP accuracy requirement can apply for BM-Case 2.

Observation 1: For BM-Case 1, UE measurement behaviour on SetB beams is the same as L1-RSRP measurement. The prediction for SetA beams in spatial domain is a short UE internal procedure. The legacy L1-RSRP measurement requirement can be reused for BM-Case 1.

Observation 2: For BM-Case 2, UE measurement behaviour on SetB beams is the same as L1-RSRP measurement. The prediction for SetA beams over multiple future time instances is one-shot UE internal procedure. The legacy L1-RSRP measurement requirement can also be reused for BM-Case 2.

Proposal 7: RAN4 to reuse legacy L1-RSRP measurement requirement for both BM-Case 1 and BM-Case 2.

References

[1] R4-2505105, WF on the requirements for AI/ML air interface normative work, Qualcomm Incorporated, RAN4#114bis.

[2] R4-2504684, Topic summary for [114bis][126] NR_AIML_air_part1, Qualcomm Incorporated, RAN4#114bis.

[3] Chair notes RAN1#120 eom1, RAN1#120.