3GPP RAN WG4 Meeting #115	R4-2505989

Saint Julian’s, Malta, May 19th – May 23rd, 2025

Title: 	Discussion on impacts of SBFD on RRM requirements

Source: 	CATT

Agenda item:	7.25.4.2

Document for:	Discussion

Introduction

In RAN4#114bis meeting, RAN4 continued discussing the RRM requirements of R19 SBFD feature. Some agreements were achieved and captured in the way forward [1]. In this contribution, we will further discuss the impacts of SBFD on RRM requirements.

Discussion

CSI-RS L1 measurement – puncturing in frequency domain

On this issue, some progresses were achieved in last meeting:

Agreements: - Note: the following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD - Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2 - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements - Option 1: Add a lower bound of 8ms on top of existing delay requirements. - Option 2: other - For Case 3 - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs - In the test case, not model the interfering UE.

One open issue is that whether and how to update the delay requirement when UE indicates that CSI processing time is scaled by 2. If UE indicates to network that the time used for CSI processing needs scaling for DUD case in SBFD, network are expected to configure the CSI-RS resources with a large periodicity that can cover the scaled CSI processing time duration. Therefore the existing delay requirements can still apply. In case network configures a dense CSI-RS resource in time domain that UE cannot receive and process each occasion, a longer measurement delay can be expected.

Proposal 1: For DUD case, the existing measurement period for CSI-RS based L1 measurement apply when the periodicity of CSI-RS is large enough to cover scaled processing time. Otherwise, a longer measurement delay can be expected.

CSI-RS L1 measurement – different symbol types

Regarding the impacts of different symbol types on CSI-RS based L1 measurement requirement, some progresses in last meeting are copied below:

Agreements: - For L1-RSRP and L1-SINR - For CSI-RS based L1-RSRP/SINR measurements, UE measures the CSI-RS occasions on the valid symbol type indicated by network - For L1-RSRP and L1-SINR measurement period, further discuss the following options. - Option 1: Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach. - Option 2: other - For RLM/BFD/CBD, further discuss the following options - RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type - Existing measurement period apply - FFS whether to add Note: For the potential power difference between SBFD symbol and non-SBFD symbol, no RAN4 enhancement on power difference handling in Rel-19

There are two issues that need further discussions. For L1-RSRP and L1-SINR measurement period, since network will indicate the type of symbols that UE should measure, it is straightforward to define measurement delay requirement by excluding the invalid types. And technically we agree to follow the R16 NR-U approach. The details can be further determined in CR discussion.

Proposal 2: For L1-RSRP and L1-SINR measurement period, measurement period is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach.

For CSI-RS based RLM/BFD/CBD measurements, RAN1 clarify that the configurations that indicate valid symbol type are not applicable. In our understanding, UE is expected to measure CSI-RS resources on both symbol types, if capable, to shorten the measurement delay. Another option is that the existing measurement period requirement applies assuming the occasions of a single CSI-RS resource are all in the same symbol type, which is also acceptable to us. But one open issue is whether to add a note saying no RAN4 enhancement will be introduced to handle power difference between SBFD symbol and non-SBFD symbol in Rel-19. Considering RAN4 define requirements based on the assumption mentioned above, probably it will make spec clearer if this note is added. Besides, adding this note will not increase any RAN4 work. Therefore we are okay to add this note, but we are also open to further discuss this issue.

Proposal 3: For RLM/BFD/CBD, the existing delay requirement applies based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type.

Proposal 4: Okay to add the note saying no RAN4 enhancement will be introduced to handle power difference between SBFD symbol and non-SBFD symbol in Rel-19.

Requirements for CSI-RS-based L3 measurements

Regarding this issue, RAN4 achieved agreements for DU case and FFS on DUD case:

- Agreements (from Wed AH session) - For DU case: - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols. - FFS on DUD case

In DUD case where one CSI-RS resource is mapping into two discontinuous DL subbands, and a main problem is that the measurement performance will degrade if UE performs neighbour cell measurements in the legacy way. Therefore whether UE is aware of SBFD configuration in neighbour cells is the key issue. We notice that a LS [2] is sent to RAN2 by RAN3 to request RAN2 to define a IE to indicate the SBFD frequency and time configuration of neighbour cells which will be exchanged among gNBs. With this information, RAN4 can assume that UE is able to know the SBFD configuration in neighbour cells, and therefore UE needs to puncture the CSI-RS resources on SBFD symbols in frequency domain when target neighbour cell is a SBFD cell.

Proposal 5: For CSI-RS-based L3 measurements, UE needs to puncture the CSI-RS resources on SBFD symbols in frequency domain when target neighbour cell is a SBFD cell.

Anther open issue for both DU and DUD cases is the priority between CSI-RS measurement on SBFD symbols and dynamic UL transmission. The candidate options are summarized below:

- Recommended WF - Discuss the following options for collision between CSI-RS resource and dynamic UL transmission - Option 1: existing scheduling restriction apply, i.e. measurement is prioritized - Option 2: Serving cell measurement is de-prioritized and neighbour cell measurement is prioritized over dynamic UL transmission.

For the collision between CSI-RS based L3 measurement and dynamic UL transmission, considering that the mobility is more important and the measurements for mobility is always prioritized in current spec, we prefer that measurement is prioritized over UL (same as legacy scheduling restriction) in general. But since the UL transmission is dynamic scheduled in the serving cell in this case, we are okay to deprioritized serving cell measurement, which aligns the rule for the collision between L1 CSI-RS based measurement and dynamic UL transmission.

Proposal 6: For CSI-RS-based L3 measurements, serving cell measurement is de-prioritized and neighbour cell measurement is prioritized over dynamic UL transmission.

Requirements for scheduling restriction

There are two sub-issues in this section: scheduling restriction requirements for UL/DL transition and for inter-frequency SSB measurement without gap. The recommended WF is reproduced below for reference:

- Recommended WF for UL/DL transition - Do not define new scheduling restriction requirements for DL/UL transition. - Recommended WF for inter-frequency SSB measurement outside MG - FFS whether additional scheduling restriction applies for inter-frequency SSB measurement outside MG on SBFD symbols, when the SSB is fully within DL active BWP but not completely contained in DL sub-band. - Option 1: yes, UE is not required to receive DL in DL subbands on SBFD symbols - Option 2: no, UE is required to receive DL in DL subbands on SBFD symbols

Regarding the scheduling restriction requirements for DL/UL transition, we do not think it is necessary to define new requirements. The impacts of DL/UL transition can be covered by the existing L1/L3 scheduling restrictions.

Proposal 7: Do not define new scheduling restriction requirements for DL/UL transition.

For inter-frequency SSB measurement without gap, when the SSB is fully within DL active BWP but not completely contained in DL sub-band, UE is expected to receive SSB and perform measurements. Therefore UE should not be scheduled by network for UL transmission, i.e., UL scheduling restriction is needed. Since requirements for scheduling restriction for inter-frequency SSB measurement without gap have been defined in TS 38.133 clause 9.3.9, RAN4 can use these existing requirements as a starting point and check if any updates are required.

Proposal 8: RAN4 to use the existing scheduling restriction requirements for inter-frequency SSB measurement without gap as a starting point.

Conclusions

This contribution discussed the impacts of SBFD on RRM requirements and some proposals are provided:

Proposal 1: For DUD case, the existing measurement period for CSI-RS based L1 measurement apply when the periodicity of CSI-RS is large enough to cover scaled processing time. Otherwise, a longer measurement delay can be expected.

Proposal 2: For L1-RSRP and L1-SINR measurement period, measurement period is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach.

Proposal 3: For RLM/BFD/CBD, the existing delay requirement applies based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type.

Proposal 4: Okay to add the note saying no RAN4 enhancement will be introduced to handle power difference between SBFD symbol and non-SBFD symbol in Rel-19.

Proposal 5: For CSI-RS-based L3 measurements, UE needs to puncture the CSI-RS resources on SBFD symbols in frequency domain when target neighbour cell is a SBFD cell.

Proposal 6: For CSI-RS-based L3 measurements, serving cell measurement is de-prioritized and neighbour cell measurement is prioritized over dynamic UL transmission.

Proposal 7: Do not define new scheduling restriction requirements for DL/UL transition.

Proposal 8: RAN4 to use the existing scheduling restriction requirements for inter-frequency SSB measurement without gap as a starting point.

References

[1] R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN4#114bis.

[2] R3-250888, SBFD information exchange among gNBs for CLI mitigation, RAN3#127.