3GPP TSG-RAN WG4 Meeting #115				                                                R4-2505992

Saint Julian’s, Malta, May 19th – May 23rd, 2025

Title:	Discussion on FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor

Source:	CATT

Agenda Item:	7.18.3.1

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the enhancement for FR2-1 SSB based measurement delay reduction was discussed. The conclusions were captured in the approved WF [1] and the left open issues can be found in the topic summary [2]. In this paper, we will continue the discussion on FR2-1 SSB based measurement delay reduction by optimizing Rx beam sweeping factor (BSF) and give our views.

2 Discussion

In last meeting, RAN4 concluded the conditions that trigger UE to activate the L3 measurement delay reduction by optimizing Rx BSF while the quit condition and evaluation mechanism have not been discussed. According to summary [2], there are three main options for quit condition:

Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” For scenario 1/2/3/4: - Option 1: UE autonomous quitting. - Option 2: UE indication to the NW is used to deactivate FBS based L3 measurements. - Option 3: Timer-based solution to deactivate Rx BSF reduction.

Firstly, we do not support UE to quit “L3 measurement delay reduction by optimizing Rx BSF” autonomously without any reporting or indication to network, since this may cause misalignment between network and UE.

For Option 2, several solutions were brought up. For example, UE can indicate to network via UAI signaling that it has to deactivate multi-Rx or UE is allowed to quit “L3 measurement delay reduction by optimizing Rx BSF” after it reports a valid result. This option gives some flexibility to UE implementation and helps UE to save power.

Option 3 is quite clear that network controls the time durations, T1 and T2 respectively, during which UE needs to perform L3 measurements with multi-Rx on and during which UE can perform L3 measurements in a legacy way. In this option, the timer values configured by network should be reasonable, because a small value for T1 may decrease the gain of this feature while a large value for T1 may expose too much burden on UE. Similar issues may exist for T2.

Based on above analysis and the discussions in companies’ contributions, we think a new option that combine option 2 and option 3, which also find a balance between network demand and UE power consumption, can be brought up. In the new option, UE is allowed to quit the “L3 measurement delay reduction by optimizing Rx BSF” after it reports a valid result, and a timer starts to run when UE de-activate multi-Rx. And UE needs to evaluate the triggering condition again when this timer expires.

Proposal 1: UE is not allowed to quite “L3 measurement delay reduction by optimizing Rx BSF” autonomously.

Proposal 2: UE quits “L3 measurement delay reduction by optimizing Rx BSF” after it reports a valid report and meanwhile a timer starts to run. When the timer expires, UE re-evaluates the triggering condition.

Proposals for scenario 5/7 - Option 4 (OPPO): - For scenario 5/7, UE deactivate FBS based L3 measurements after the delay for handover or RRC Connection Release with Redirection.

For scenario 5/7, since periodic measurements are not needed, the option listed above is reasonable.

Proposal 3: For scenario 5/7, UE deactivate FBS based L3 measurements after the delay for handover or RRC Connection Release with Redirection.

3 Conclusion

In this paper, we further discussed on FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor, and the following proposals are given:

Proposal 1: UE is not allowed to quite “L3 measurement delay reduction by optimizing Rx BSF” autonomously.

Proposal 2: UE quits “L3 measurement delay reduction by optimizing Rx BSF” after it reports a valid report and meanwhile a timer starts to run. When the timer expires, UE re-evaluates the triggering condition.

Proposal 3: For scenario 5/7, UE deactivate FBS based L3 measurements after the delay for handover or RRC Connection Release with Redirection.

4 References

- R4-2504900, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT, RAN4#114bis.

- R4-2503619, Topic summary for [114bis][209] NR_RRM_Ph5_Part2, CATT, RAN4#114bis.