3GPP TSG-RAN WG4 Meeting #115				                                                R4-2506001

Saint Julian’s, Malta, May 19th – May 23rd, 2025

Title:	Discussion on the impacts of A-IoT on RRM requirements

Source:	CATT

Agenda Item:	7.26.4

Document for:	Discussion

1 Introduction

In RAN4#114bis meeting, RAN4 started to discuss the RRM impacts of ambient IoT. The agreements are captured in the WF [1]. In this contribution, we will further analyse the impacts of A-IoT device 1 deployed in scenario 1 with topology 1 (D1T1-B) on RRM requirements.

2 Discussion

A-IoT random access procedure

In last meeting, RAN4 agreed to capture the correct A-IoT device behaviour during CBRA and CFRA [1]:

Agreement: - RAN4 to include RRM requirements for correct A-IoT device behaviour during CBRA and CFRA. FFS the details

In RAN2#129bis meeting, RAN2 reached some agreements that are relevant to RACH which are reproduced below for reference [2]:

RAN2#129bis: Agreements on CFRA Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message. Agreements on msg1 for CFRA - In case of CBRA, only 16 bits random ID is included in Msg1. FFS can be revisited if message type will be needed for other D2R messages purposes - RN16 is not included in the first D2R message in the CFRA procedure. AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader. FFS if we can assume or need to support multiple device scenario. Agreements on new R2D message for CBRA - A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. - Assumption: The R2D message does not include slot number/count down number. Agreements on msg 2 for CBRA - A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices. - Same Msg2 format is used for initial transmission and retransmission of Msg2. Agreements on NACK for CFRA - For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3. May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion). - For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access. No need for a timer. FFS whether subsequent R2D message is trigger message or paging - For CFRA, NACK feedback and re-access is not supported. FFS how to achieve - FFS on end of procedure

Based on the above progress, we think the RACH solutions are not completed yet. And considering that RAN4 only need to capture the correct A-IoT behaviors, which will not involve controversial discussions, RAN4 can further wait for the completion of RACH design by RAN2 and then directly draft the wording.

Proposal 1: RAN4 to wait for more RAN2 progress on RACH design and then directly draft the wording.

Timing

Another issue that needs discussing is UL transmit timing requirement.

RAN4#114bis: Agreement: - Further discuss whether there is RRM impact on transmit timing requirements of device pending on RAN1 conclusion.

In RAN1#120bis meeting, some agreements related to RACH timing were reached [3]:

Agreement For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access. - FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access. - FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact - FFS: how to define the end of the R2D transmission in 9.4.1 Working assumption For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource: - Derived based on the starting time of the first Msg1 time domain resource plus Toffset2 - FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access.

According to the above RAN1 agreements, device will determine the starting time of Msg1 based on an offset relative to the end of corresponding R2D transmission triggering random access. A Toffset2 is required if device needs to transmit the second Msg1.  It is clear that device will determine its UL transmit timing based on the reception timing of R2D signal, and if device has a large UL transmit timing error, then it may lead to UL decoding failures at reader side. Similar situations exist for UL data transmissions, which will degrade the throughput. Therefore we think it is necessary to define UL transmit timing error for A-IoT device 1.

Proposal 2: RAN4 to define UL transmit timing requirement for A-IoT device 1.

Regarding the methodology to define UL transmit timing requirement, we suggest following the principle in LTE that is reused in NR. In LTE, the Te is defined as 24 Ts for CRS with 1.4 MHz bandwidth and 12 Ts for CRS with bandwidth equal to or larger than 3 MHz. In A-IoT system, sampling frequency of 1.92 MHz was used by RAN1 in simulation assumption in SI phase, i.e., 0.52 us per sample. On top of that, if we reuse the assumption in LTE that the Te is about 1.5 sampling interval, it gives us 0.78 us, i.e., about 24 Ts or 24*64*Tc. Ts and Tc are the basic timing unit in LTE and NR separately.

Based on above analysis, we observe that the estimated UL transmit timing error is aligned with the timing requirement defined for CRS with 1.4 MHz bandwidth in LTE. Considering that the bandwidth of A-IoT system is also very narrow, we think it is reasonable to use this result as a baseline for UL transmit timing requirement for A-IoT device 1.

In addition, from network perspective, the UL transmit timing accuracy will affect UL demodulation performance. RAN4 can double check whether the above proposed value is applicable after demodulation work in done.

Proposal 3: RAN4 to use  [24]*64*Tc for UL transmit timing requirement for A-IoT device 1.

3 Conclusion

In this paper, we discussed the impacts of A-IoT on RRM requirements and the following observations and proposals are given:

Proposal 1: RAN4 to wait for more RAN2 progress on RACH design and then directly draft the wording.

Proposal 2: RAN4 to define UL transmit timing requirement for A-IoT device 1.

Proposal 3: RAN4 to use  [24]*64*Tc for UL transmit timing requirement for A-IoT device 1.

- References

- R4-2504905, WF on RRM requirements for Ambient_IoT_Solutions, CMCC, RAN4#114bis.

- R2_129b_ChairNotes, RAN2#129bis.

- Chair notes RAN1#120bis eom0, RAN1#120bis.